3GPP TSG-SA WG6 Meeting #60	S6-241658

Changsha, China 15th – 19th April 2024	(revision of S6-241456, S6-241292)

Source:	Samsung, Ericsson

Title:	Pseudo-CR on impacts to MC services group communication

Spec:	3GPP TR 23.700-01 V0.2.0

Agenda item:	8.6

Document for:	Approval

Contact:	Basavaraj (Basu) Pattan, basavarajjp@samsung.com

1. Introduction

This pCR proposes a solution analyzing the impacts to MC services group communication due to satellite access support.

2. Reason for Change

To provide solution analyzing the impacts to MC services (particularly group communication) due to satellite access support.

## 4.4 Key issue #4: Satellite access support for MC services ### 4.4.1 Description To ensure reliable MC services with larger coverage area, MC systems can benefit from satellite access, especially when the connectivity provided by the terrestrial networks faces limitations, e.g., bad coverage at remote areas. Hence, MC service UEs can be able to connect to MC system via satellite access to ensure service continuity. There are different satellite systems, based on altitude, roundtrip time and coverage such as; Low Earth Orbit (LEO), Medium Earth Orbit (MEO), and Geostationary Equatorial Orbit (GEO). Among these different systems, LEO offers low latency, and large area capacity density. In this study, it is worth understanding the deployment scenarios to utilize satellite access for MC services, and understanding how the MC KPIs (e.g., latency requirement) can be met when MC service users are connecting to the MC system via satellite access. Location reporting aspects for MC service clients is to be considered and understand whether location reports provided by MC service users over satellite access may or may not be different from MC service users over terrestrial access. ### 4.4.2 Open issues There are several aspects that need to be considered and studied to support satellite access for MC services, including but not limited to: 1. Identify the different deployment scenarios for MC services using satellite access to result in a better MC service user experience.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 V0.2.0.

* * * First Change * * * *

### Annex B (informative):	Information related to Satellite access support for MC Service

As per the current 3GPP specifications 3GPP TS 22.261 and 3GPP TS 23.501, terrestrial communicationis possible via satellite access, where there is a connectivity between the UE and the satellite. This means that 5G services are possible over both Terrestrial Network (TN) and Non-Terrestrial Network (NTN). In NTN, Satellite is placed into Low-Earth Orbit (LEO) typically at an altitude between 300 km to 2000 km, Medium-Earth Orbit (MEO) typically at an altitude between 8000 to 20000 km, or Geostationary satellite Earth Orbit (GEO) at 35786 km altitude. Utilizing satellite access is beneficial to increase coverage and availability of the services. However, there is significant propagation delay associated to connectivity via satellite as compared to connecting via terrestrial network.

The propagation delay via satellite associated with different orbit is as below as discussed in 3GPP TS 22.261:

Table B-1: Satellite propogation delays

|  | UE to serving satellite propagation delay [ms] [NOTE 1] |  | UE to ground max propagation delay [ms] [NOTE 2] |
| --- | --- | --- | --- |
|  | Min | Max |  |
| LEO | 3 | 15 | 30 |
| MEO | 27 | 43 | 90 |
| GEO | 120 | 140 | 280 |
| NOTE 1: The serving satellite provides the satellite radio link to the UE NOTE 2: delay between UE and ground station via satellite link; inter satellite links are not considered |  |  |  |

Satellite systems of relatively low altitude, e.g., LEO, can offer relatively lower latency, which can be the most beneficial choice latency sensitive services such as MC services.

Furthermore, unlike GEO, beams of non-GEO satellites (LEO and MEO) are moving with respect to the ground. Therefore, it is necessary to have a constellation of several non-GEO satellites associated with the MC service under consideration to ensure proper service continuity over the non-terrestrial networks. This results in having the same cell being covered by different beams, i.e., different non-GEO satellites over time.

Following KPIs are requirements for MC service as specified in 3GPP TS 22.179:

Table B-2: Mission Critical KPIs

| KPI 1 | MCPTT Access time | < 300 ms |
| --- | --- | --- |
| KPI 2 | End-to-end MCPTT Access time | < 1000 ms |
| KPI 3 | Mouth-to-ear latency | < 300 ms |
| KPI 4a | Maximum Late call entry time (without application layer encryption) | < 150 ms |
| KPI 4b | Maximum Late call entry time (with application layer encryption) | < 350 ms |

MC services are not restricted only to the ones defined in this clause and such services can also have priority treatment, if defined via operator's policy and/or local regulation. Priority treatment for MC Services require appropriate ARP and 5QI (plus 5G QoS characteristics) setting for QoS Flows according to the operator's policy. As per 3GPP TS 23.501, the following 5QIs are proposed for catering mission critical KPIs:

Table B-3: 5QIs catering Mission Critical KPIs

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 69 | Non-GBR | 5 | 60 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| 8 | Non-GBR | 80 | 300 ms | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive |
| 65 | GBR | 7 | 75 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 67 | GBR | 15 | 100 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 4 | GBR | 50 | 300 ms | 10-6 | N/A | 2000 ms | Non-Conversational Video (Buffered Streaming) |
| 70 | Non-GBR | 55 | 200 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9) |
| 10 | Non-GBR | 90 | 1100ms | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) and any service that can be used over satellite access type with these characteristics |

Considering MCPTT Access time and Mouth-to-ear latency KPI requirements, which are less than 300 ms, the consumption of MC Services has impacts when MC Service users are connected via Satellite due to significant propagation delay.

* * * End of Changes * * * *