3GPP TSG-SA WG6 Meeting #65	S6-250335

Athens, Greece 17th – 21st February 2025	(revision of S6-250016)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0631 | rev | 1 | Current version: | 19.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove MCData recording from Rel-19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Airbus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2025-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Recording of MCData services feature was not completed in Rel-19 and it was postponed to Rel-20. It must be removed from Rel-19 specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing MCData from recording/replay related clauses and/or clarifying that MCData recording is not supported in this version of TS 23.280. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading Rel-19 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 10.1.2.1a, 10.1.4.2.6a, 10.18.1, A.4, A.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR shall undo and/or clarify all MCData related changes agreed in: - CR584 S6-244415, clause 3.1, - CR612 S6-245353, clauses 10.1.2.1a and 10.1.4.2.6a, - CR595 S6-244422, clause 10.18.1, - CR587 S6-244418, clause A.4 and - CR586 S6-245356, clause A.11. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1]. Not all definitions are used in this document.

Accuracy: Reflects the uncertainty of the location at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

ACM: Administrative Configuration Management, which enables the exchange of administrative configuration data between interconnected MC systems.

ACMC: Administrative Configuration Management Client, client entity which initiates administrative configuration exchange request to an interconnected partner MC system and which could make decision on such request received from an ACMC of a partner MC system.

ACMS: Administrative Configuration Management Server, server entity which receives administrative configuration exchange requests from an ACMC belonging to the same primary MC system or via an ACMS of an interconnected partner MC system.

Active MC service user profile: The MC service user profile that is currently used by an MC service client of an MC service user while receiving MC service.

Ad hoc Group Communication: The combining of a multiplicity of MC service users into a group for the duration of a communication. When the communication is released, the group no longer exists. If the communication is associated with an alert, then the group continues to exist until the alert is also cancelled.

Ad hoc Group emergency alert: The combining of a multiplicity of MC service users into a group for sending an emergency alert. When the alert is cancelled, the group no longer exists. If the alert is associated with a communication, then the group continues to exist until the communication is also cancelled.

Altitude: Third dimension for the geographical coordinates at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

Bearing: Direction at the moment of location measurement, e.g. see 3GPP TS 25.305 [30].

Chat group: An MC service group that is pre-defined with MC service group ID and member list in the group management server. Group members must join the pre-established group call to participate.

ECGI: E-UTRAN Cell Global Identifier, which is used to identify cells globally, where the ECGI is constructed from the Mobile Country Code (MCC), Mobile Network Code (MNC) and the E-UTRAN Cell Identifier (ECI).

Interconnection: A means of communication between MC systems whereby MC service users obtaining MC service from one MC system can communicate with MC service users who are obtaining MC service from one or more other MC systems.

Interconnection group: An MC service group that is configured to allow inclusion of MC service group members who are MC service users from partner MC system(s).

LCS network: The 3GPP network that provides location service as defined in 3GPP TS 23.271 [29].

Location: The current physical location of the MC service UE.

MBMS SAI: Multimedia Broadcast Multicast Service Area Identity which is mapped to the MBMS service area.

MC gateway server: A server providing topology hiding for MC service interconnection with a partner MC system, where that partner MC system is in a different trust domain.

MC service: A generic name for any one of the three mission critical services: either MCPTT, or MCVideo, or MCData.

MC service affiliated group member: An MC service user who has indicated an interest in a particular MC service group and has been accepted to participate in MC service group communication for that MC service group.

MC service client: A generic name for the client application function of a specific MC service. MC service client could be replaced by MCPTT client, or MCVideo client, or MCData client depending on the context.

MC service group: A defined set of MC service users with associated communication dispositions (e.g. media restrictions, default priority and commencement directions) configured for the use with one or more MC services.

MC service group affiliation: A mechanism by which an MC service user's MC service(s) communication interest in one or more MC service groups is determined.

MC service group call: A mechanism by which an MC service user can make a one-to-many MC service(s) transmission to other users that are members of MC service group(s).

MC service group de-affiliation: A mechanism by which an MC service user's MC service(s) communication interest in one or more MC service groups is removed.

MC service group home system: The MC system where the MC service group is defined.

MC service group host MC service server: The MC service server within an MC system which provides centralised support for a particular MC service of an MC service group defined in a MC service group home system.

MC service group member: An MC service user, whose MC service ID is listed in a particular MC service group.

MC service ID: A generic name for the user ID of a mission critical user within a specific MC service. MC service ID could be replaced by MCPTT ID, MCVideo ID, or MCData ID depending on the context.

MC service server: A generic name for the server application function of a specific MC service. MC service server could be replaced by MCPTT server, MCVideo server, or MCData server depending on the context.

MC service user: An authorized user, who can use an MC service UE to participate in one or more MC services.

MC service user profile: The set of information associated to an MC service user that allows that user to employ one or more MC services in a given role and from a given MC service UE.

MC service UE: A UE that can be used to participate in one or more MC services and recording admin and replay services.

MC service UE label: A generic name for identification of a specific MC service UE.

MC system: The collection of applications, services, and enabling capabilities required to provide a single mission critical service or multiple mission critical services to one or more mission critical organizations.

MC user: A user, identified by an MC ID, who, after authorization, obtains mission critical service(s).

Migration: A means for an MC Service user to obtain MC service directly from a partner MC system.

Partner MC system: Allied MC system that provides MC services to an MC service user based on the MC service user profiles that are defined in the primary MC system of that MC service user.

Preconfigured MC service group: an MC service group used only for regrouping that has been configured in advance of a group or user regrouping operation to serve as the source of regroup group configuration.

Pre-arranged group: An MC service group that is pre-defined with MC service group ID and member list in the group management server. Affiliated group members are invited when the group communication is setup.

Pre-selected MC service user profile: The MC service user profile that is to be selected as the active MC service user profile through configuration, and applicable for an authenticated MC service user upon MC service authorization.

Primary MC system: MC system where the MC service user profiles of an MC service user are defined.

Recording admin UE: A UE that can be used to configure targets for recording. Recording admin UE can be a mobile device or a (computer) workstation.

Recording admin and replay service user profile: A set of parameters defining the authorizations for a recording administrator and/or replay user.

Recording administrator: A user, identified by MCRec ID, who, after authorization, is able to set and modify target users and target groups for recording.

Recording admin client: A client application that a recording administrator can use to set target users and target groups for recording.

Recording server: A server that is able to log the metadata and record the media of MCPTT and MCVideo group communications and private communications. This server is also able to retrieve the logged/recorded data when requested by an authorized replay user.

Replay user: A user, identified by an MCRec ID, who, after authorization, is able to fetch and replay recorded MCPTT and MCVideo metadata and media of MC service group communications and MC service private communications under the user’s authority.

Replay UE: A user equipment that can be used to fetch and replay recorded MCPTT and MCVideo metadata and media of MC service group communications and MC service private communications under the UE user’s authority. The Replay UE can be a mobile device or a (computer) workstation.

Replay client: A client application that an authorized replay user can use to fetch and replay recorded metadata and media from a recording server.

Requested Priority: A value for use in a MC service group or MC private communication that, if accepted, is used by the MCX service server to temporarily replace the priority level that is predefined in the MC service group or MC service user profile. This value is used in combination with other factors to determine the application priority for the requested communication.

Selected MC service user profile: The MC service user profile that is to be selected as the active MC service user profile for an MC service upon request by an MC service user.

Serving MC service server: The MC service server which is providing MC service to an MC service client.

NOTE 1:	There is one serving MC service server for each MC service, which can be the primary MC service server of the MC service user of the MC service client, or can be a partner MC service server to which the MC service user has migrated.

Serving MC system: The MC system which is providing MC service to an MC user.

NOTE 2:	The MC system can be the primary MC system of the MC service user, or can be a partner MC system to which the MC service user has migrated.

Speed: Movement at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

Time of measurement: Date and time expressed with a certain precision to reflect the moment of the location measurement.

For the purposes of the present document, the following terms given in 3GPP TS 22.280 [3] apply

Mission Critical

Mission Critical Applications

Mission Critical Organization

Mission Critical Service

Functional alias

For the purposes of the present document, the following terms given in 3GPP TS 22.179 [2] apply

Multi-talker control

Group-broadcast group

For the purposes of the present document, the following terms related to a MC gateway UE function apply

MC gateway UE:  A UE with functionality that enables access to the MC service for non-3GPP devices.

MC client: Aggregates a set of clients (i.e. Group management client, Configuration management client, Identity management client, Key management client, Location management client and MC service client).

MC server: Aggregates a set of servers (i.e. Group management server, Configuration management server, Identity management server, Key management server, Location management server and MC service server) which serves the MC client accordingly.

Non-3GPP device: A device that enables connectivity towards an MC gateway UE using an access method not specified by 3GPP. A subset of these devices can host an MC client specified by 3GPP.

# * * * * Second change * * * *

#### 10.1.2.1a	Store target group configuration request

Table 10.1.2.1a-1 describes the information flow store target group configuration request from the group management client to the group management server.

Table 10.1.2.1a-1: Store target group configuration request

| Information element | Status | Description |
| --- | --- | --- |
| MCRec ID | M | The MCRec ID of the requestor. |
| MC service group ID (NOTE 1) | M | MC service group ID of the target group. |
| MC service group configuration data (see NOTE 2) | M | The contents of the MC service group configuration data to be updated. |
| NOTE 1: In this version of this specification, only MCPTT group ID and MCVideo group ID is supported. NOTE 2: A Recording admin user (MCRec ID) has the authority to update only two parameters in the group configuration data - “Group is a target for recording” and “Recording server address”. |  |  |

# * * * * 3rd change * * * *

##### 10.1.4.2.6a	Update target MC service user profile data request

Table 10.1.4.2.6a-1 describes the information flow update target MC service user profile data request from the configuration management client to the configuration management server.

Table 10.1.4.2.6a-1: Update target MC service user profile data request

| Information element | Status | Description |
| --- | --- | --- |
| MCRec ID | M | The MCRec ID of the requestor. |
| MC service ID (NOTE 1) | M | The MC service ID of the target MC service user whose profile is updated. |
| Updated MC service user profile data (see NOTE 2) | M | The contents of the user profile data to be updated. |
| NOTE 1: In this version of this specification, only MCPTT ID and MCVideo ID is supported. NOTE 2: A recording admin user (MCRec ID) has the authority to update only two parameters in target MC service user’s profile – the “User is a target for recording” and “Recording server address”. |  |  |

# * * * * 4th change * * * *

### 10.18.1	Recording procedures

MC recording procedures involves capturing the contents of MC services that can be call/session based MC services (e.g. MCPTT/MCVideo Call etc.) or non-call/session based MC Services/events (e.g. MCPTT Emergency Alert etc.).

# * * * * 5th change * * * *

# A.4	Group configuration data

The group configuration data is stored in the group management server. The group management server is used to configure the group configuration data to the MC service UE (CSC-2), recording admin UE (CSC-2), replay UE (CSC-2), MC service server (CSC-3) and the recording server (REC-5).

A single group can support one or multiple MC service configurations. Hence, the MC service group configuration data is divided into common group configuration data and MC service specific group configuration data.

The group configuration data can be configured offline using the CSC-12 reference point.

The common group configuration data are specified in table A.4-1. Tables A.4-1 and A.4-2 contain the group configuration required to support the use of on-network MC service. Tables A.4-1 and A.4-3 contain the group configuration required to support the use of off-network MC service.

The MC service related group configuration data specific to each MC service is specified as follows:

-	MCPTT related group configuration data is specified in 3GPP TS 23.379 [16];

-	MCVideo related group configuration data is specified in 3GPP TS 23.281 [12]; and

-	MCData related group configuration data is specified in 3GPP TS 23.282 [13];

Table A.4-1: Common group configuration data (on and off network)

| Reference | Parameter description | MC service UE | Rec. admin UE & Replay UE(see NOTE 9) | MC service Server | Group management server | Recording server |
| --- | --- | --- | --- | --- | --- | --- |
| [R-5.17-004], [R-6.4.3-001], [R-6.4.3-002], [R-6.9-004] and [R-5.1.3-002] of 3GPP TS 22.280 [3] | MC service group ID | Y | Y (NOTE 12) | Y | Y | Y (NOTE 12) |
| [R-5.17-004], [R-6.4.3-001], [R-6.4.3-002] and [R-6.9-004] of 3GPP TS 22.280 [3] | Group Alias (Contact URIs) | Y |  | Y | Y |  |
|  | List of group members (see NOTE 1) (see NOTE 8) |  |  |  |  |  |
| [R-5.1.3-001], [R-5.1.5-001], [R-6.4.5-005] and [R-6.4.5-006] of 3GPP TS 22.280 [3] | > MCPTT |  |  |  |  |  |
|  | >> MCPTT ID (see NOTE 2) | N | Y | Y | Y |  |
| 3GPP TS 33.180 [25] | >> KMSUri for security domain of the MCPTT ID (see NOTE 3) | N |  | Y | Y |  |
| [R-6.2.2-001] and [R-7.6-007] of 3GPP TS 22.280 [3], [R-6.2.3.7.4.1-002] and [R-6.2.3.7.4.1-006] of 3GPP TS 22.179 [2] | >> User priority for the group | N |  | Y | Y |  |
| [R-5.1.3-001] of 3GPP TS 22.280 [3] | >> Participant type for the group (group membership information). The particpant type values are defined and configured by the Mission Critical Organisation (e.g.first responder, second responder, dispatcher, dispatch supervisor, MC service administrator). | N | Y | Y | Y |  |
| [R-5.1.3-001], [R-5.1.5-001], [R-6.4.5-005] and [R-6.4.5-006] of 3GPP TS 22.280 [3] | > MCVideo |  |  |  |  |  |
|  | >> MCVideo ID (see NOTE 2) | N | Y | Y | Y |  |
| 3GPP TS 33.180 [25] | >> KMSUri for security domain of the MCVideo ID (see NOTE 3) | N |  | Y | Y |  |
| [R-6.2.2-001] and [R-7.6-007] of 3GPP TS 22.280 [3] | >> User priority for the group | N |  | Y | Y |  |
| [R-5.1.3-001] of 3GPP TS 22.280 [3] | >> Participant type for the group (group membership information). The particpant type values are defined and configured by the Mission Critical Organisation (e.g.first responder, second responder, dispatcher, dispatch supervisor, MC service administrator). | N | Y | Y | Y |  |
| [R-5.1.3-001], [R-5.1.5-001], [R-6.4.5-005] and [R-6.4.5-006] of 3GPP TS 22.280 [3] | > MCData |  |  |  |  |  |
|  | >> MCData ID (see NOTE 2) | N |  | Y | Y |  |
| 3GPP TS 33.180 [25] | >> KMSUri for security domain of the MCData ID (see NOTE 3) | N |  | Y | Y |  |
| [R-6.2.2-001] and [R-7.6-007] of 3GPP TS 22.280 [3] | >> User priority for the group | N |  | Y | Y |  |
| [R-5.1.3-001] of 3GPP TS 22.280 [3] | >> Participant type for the group (group membership information). The particpant type values are defined and configured by the Mission Critical Organisation (e.g.first responder, second responder, dispatcher, dispatch supervisor, MC service administrator). | N |  | Y | Y |  |
| [R-5.16.2-001] and [R-5.16.2-002] of 3GPP TS 22.280 [3] | Group’s owner (Mission Critical Organisation) | Y | Y | Y | Y | Y |
| Subclause 8.3.2, 10.2.3 | MC service specific configuration (see NOTE 4) |  |  |  |  |  |
|  | > MCPTT configuration (see NOTE 5) |  |  |  |  |  |
|  | > MCVideo configuration (see NOTE 6) |  |  |  |  |  |
|  | > MCData configuration (see NOTE 7) |  |  |  |  |  |
|  | List of subordinate groups |  |  |  |  |  |
| [R-5.2.2-002] of 3GPP TS 22.280 [3] | > MC service group ID | N | Y | Y | Y |  |
|  | Group broadcast information |  |  |  |  |  |
| [R-5.2.2-001] of 3GPP TS 22.280 [3] | Level within group hierarchy | N |  | Y | Y |  |
| [R-5.2.3-001] of 3GPP TS 22.280 [3] | Level within user hierarchy | N |  | Y | Y |  |
| Subclause 10.6.2.9 of 3GPP TS 23.379 [16] | Group is a preconfigured regroup group | Y | Y | Y | Y |  |
| [R-5.2.1-002] of 3GPP TS 22.280 [3] | Group is a broadcast group | Y | Y | Y | Y |  |
| [R-6.15.4-001…010] of 3GPP TS 22.280 [3] | Group is a target for recording | Y (see NOTE 11) | Y | Y | Y | Y |
| [R-6.15.4-001…010] of 3GPP TS 22.280 [3] | Recording server address (URI) (see NOTE 10) | Y (see NOTE 11) | Y | Y | Y |  |
| NOTE 1: The list of group members is provided to an authorized user only upon request and not by default. NOTE 2: MC service Ids for all configured MC services shall be provided for the configured group member. NOTE 3: If this parameter is absent, the KMSUri shall be that identified in the initial MC service UE configuration data (on-network) configured in table A.6-1. NOTE 4: An MC service specific configuration shall be provided for all of the MC services for which the group is enabled. At least one MC service shall be enabled for a group. NOTE 5: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.379 [16]. NOTE 6: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.281 [12]. NOTE 7: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.282 [13]. NOTE 8: The list of group members comprises all group members in primary and in partner MC systems of the MC service group. NOTE 9: This is the recording admin UE and/or replay UE of a user who is authorized to set this MC service group as a target for recording and/or authorized to replay this MC service group’s recordings. NOTE 10: If this parameter is absent, the Recording server URI shall be that identified in the initial MC service UE configuration data (on-network, table A.6-1) or in the initial recording admin and replay UE configuration data (on-network, table A.10-1) NOTE 11: These parameters, when provided to an MC service UE, are relevant only for recording administrators and replay users. NOTE 12: In this version of this specification only MCPTT and MCVideo groups shall be configured to the recording admin UE, replay UE and the recording server. Recording of MCData services is not supported. |  |  |  |  |  |  |

Table A.4-2: Common group configuration data (on-network)

| Reference | Parameter description | MC service UE | MC service Server | Group management server |
| --- | --- | --- | --- | --- |
| [R-6.1-001] of 3GPP TS 22.280 [3] | Maximum number of group members (Nc6) | N | Y | Y |
| [R-6.1-003] of 3GPP TS 22.280 [3] | Enabled/disabled group (basic status) | Y | Y | Y |
| [R-6.6.1-006] of 3GPP TS 22.280 [3] | Temporary group indication | Y | Y | Y |
| [R-6.4.5-005] and [R-6.15.4-001…010] of 3GPP TS 22.280 [3] | List of authorised users who can request the list of members of an MC service group | Y | Y | Y |
|  | > MCPTT IDs |  |  | Y |
|  | > MCVideo IDs |  |  | Y |
|  | > MCData IDs |  |  | Y |
|  | > MCRec IDs |  |  | Y |
|  | MC service specific configuration |  |  |  |
|  | > MCPTT configuration (see NOTE 1) |  |  |  |
|  | > MCVideo configuration (see NOTE 2) |  |  |  |
|  | > MCData configuration (see NOTE 3) |  |  |  |
| [R-6.4.9-006] of 3GPP TS 22.280 [3] | Geographic area within which affiliation to the group is permitted (see NOTE 4) | Y | Y | Y |
| [R-6.4.9-006] of 3GPP TS 22.280 [3] | Geographic area outside which de-affiliation from the group is mandatory (see NOTE 4) | Y | Y | Y |
| Subclause 10.2.5 and [R-6.15.4-001…010] of 3GPP TS 22.280 [3] | List of authorised users who can retrieve the group information |  |  |  |
|  | > MCPTT IDs |  |  | Y |
|  | > MCVideo IDs |  |  | Y |
|  | > MCData IDs |  |  | Y |
|  | > MCRec IDs |  |  | Y |
| Subclause 5.2.3 | Associated relay service code (as specified in 3GPP TS 23.303 [14]) | Y | N | Y |
| Subclause 5.2.9 of 3GPP TS 23.280 [16] | List of partner MC systems from which interconnection is permitted for the group |  |  |  |
| Subclause 5.2.9 of 3GPP TS 23.280 [16] | > Identity of partner MCPTT system |  | Y | Y |
| Subclause 5.2.9 of 3GPP TS 23.280 [16] | > Identity of partner MCVideo system |  | Y | Y |
| Subclause 5.2.9 of 3GPP TS 23.280 [16] | > Identity of partner MCData system |  | Y | Y |
| [R-6.4.4-003] of 3GPP TS 22.280 [16] | List of functional aliases which prevents de-affiliating |  |  |  |
|  | > Functional alias | N | Y | Y |
| [R-6.4.4-004] of 3GPP TS 22.280 [16] | List of functional aliases which prevents de-affiliating, if only one user has bound the functional alias to the group |  |  |  |
|  | > Functional alias | N | Y | Y |
| NOTE 1: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.379 [16]. NOTE 2: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.281 [12]. NOTE 3: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.282 [13]. NOTE 4: The geographic area may consist of a union of regions that are non-contiguous or contain holes. |  |  |  |  |

Table A.4-3: Common group configuration data (off-network)

| Reference | Parameter description | MC service UE | MC service server | Group management server |
| --- | --- | --- | --- | --- |
| Subclause 8.1.3.2 | ProSe layer-2 group ID (as specified in 3GPP TS 23.303 [14]) | Y | N | Y |
| Subclause 8.1.3.2 | ProSe group IP multicast address (as specified in 3GPP TS 23.303 [14]) | Y | N | Y |
| Subclause 8.1.3.2 | Indication of whether the UE shall use Ipv4 or Ipv6 for the MC service group (as specified in 3GPP TS 23.303 [14]) | Y | N | Y |
|  | MC service specific configuration |  |  |  |
|  | > MCPTT configuration (see NOTE 1) |  |  |  |
|  | > MCVideo configuration (see NOTE 2) |  |  |  |
|  | > MCData configuration (see NOTE 3) |  |  |  |
| NOTE 1: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.379 [16]. NOTE 2: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.281 [12]. NOTE 3: This parameter is a parent parameter whose child parameters are defined in 3GPP TS 23.282 [13]. |  |  |  |  |

# * * * * 6th change * * * *

# A.11	Recording admin and/or replay service user profile configuration data

The recording admin and/or replay service user profile configuration data is stored in the configuration management server. The recording server and the group management server obtain the recording admin and/or replay service user profile configuration data from the configuration management server.

Table A.11-1 contains the recording admin and/or replay service user profile configuration required to support the use of on-network recording and replay service. Data in table A.11-1 can be configured offline using the CSC-11 reference point.

Table A.11-1: Recording admin and/or replay service user profile configuration data (on-network)

| Reference | Parameter description | Rec. admin UE & Replay UE | Recording server | Conf. mgmt server | Group mgmt server |
| --- | --- | --- | --- | --- | --- |
| [R-6.15.4-001…010] of 3GPP TS 22.280 [3] and subclause 5.2.4a | Recording admin and replay user identity (MCRec ID) | Y | Y | Y | Y |
| Subclause 5.2.4a | Recording admin and/or replay user profile name | Y |  | Y |  |
| Subclause 5.2.4a | User profile status (enabled/disable) | Y | Y | Y | Y |
| [R-6.15.4-001] of 3GPP TS 22.280 [17] and subclause 5.2.4a | User's Mission Critical Organization | Y | Y | Y | Y |
| [R-6.15.4-001] - [R-6.15.4-010] of 3GPP TS 22.280 [17] and subclause 5.2.4a | Authorized to set and modify target MC service users for recording in his/her organization (yes/no) | Y |  | Y |  |
|  | > All MC service users (see NOTE 1) |  |  |  |  |
|  | >> MCPTT | Y |  | Y |  |
|  | >> MCVideo | Y |  | Y |  |
|  | > All MC service users with exceptions (see NOTE 1) |  |  |  |  |
|  | >> List of exceptions (see NOTE 5) |  |  |  |  |
|  | >>> MCPTT ID | Y |  | Y |  |
|  | >>> MCVideo ID | Y |  | Y |  |
|  | > Only the listed MC service users (see NOTE 1) |  |  |  |  |
|  | >> List of users (see NOTE 5) |  |  |  |  |
|  | >>> MCPTT ID | Y |  | Y |  |
|  | >>> MCVideo ID | Y |  | Y |  |
| [R-6.15.4-001] - [R-6.15.4-010] of 3GPP TS 22.280 [17] and subclause 5.2.4a | Authorized to set and modify target groups for recording in his/her organization (yes/no) | Y |  | Y | Y |
|  | > All MC service groups (see NOTE 2) |  |  |  |  |
|  | >> MCPTT | Y |  | Y | Y |
|  | >> MCVideo | Y |  | Y | Y |
|  | > All MC service groups with exceptions (see NOTE 2) |  |  |  |  |
|  | >> List of exceptions (see NOTE 5) |  |  |  |  |
|  | >>> MC service group ID | Y |  | Y | Y |
|  | >>> Group management server URI | Y |  | Y | Y |
|  | > Only the listed MC service groups (see NOTE 2) |  |  |  |  |
|  | >> List of groups (see NOTE 5) |  |  |  |  |
|  | >>> MC service group ID | Y |  | Y | Y |
|  | >>> Group management server URI | Y |  | Y | Y |
| [R-6.15.4-001] - [R-6.15.4-010] of 3GPP TS 22.280 [17] and subclause 5.2.4a | Authorized to replay recordings of users in his/her organization (yes/no) | Y | Y | Y |  |
|  | > All MC service users (see NOTE 3) |  |  |  |  |
|  | >> MCPTT | Y | Y | Y |  |
|  | >> MCVideo | Y | Y | Y |  |
|  | > All MC service users with exceptions (see NOTE 3) |  |  |  |  |
|  | >> List of exceptions (see NOTE 5) |  |  |  |  |
|  | >>> MCPTT ID | Y | Y | Y |  |
|  | >>> MCVideo ID | Y | Y | Y |  |
|  | > Only the listed MC service users (see NOTE 3) |  |  |  |  |
|  | >> List of users (see NOTE 5) |  |  |  |  |
|  | >>> MCPTT ID | Y | Y | Y |  |
|  | >>> MCVideo ID | Y | Y | Y |  |
| [R-6.15.4-001] - [R-6.15.4-010] of 3GPP TS 22.280 [17] and subclause 5.2.4a | Authorized to replay recordings of MC service groups in his/her organization (yes/no) | Y | Y | Y |  |
|  | > All groups (see NOTE 4) |  |  |  |  |
|  | >> MCPTT | Y | Y | Y |  |
|  | >> MCVideo | Y | Y | Y |  |
|  | > All MC service groups with exceptions (see NOTE 4) |  |  |  |  |
|  | >> List of exceptions (see NOTE 5) |  |  |  |  |
|  | >>> MC service group ID | Y | Y | Y |  |
|  | >>> Group management server URI | Y | Y | Y |  |
|  | > Only the listed groups (see NOTE 4) |  |  |  |  |
|  | >> List of groups (see NOTE 5) |  |  |  |  |
|  | >>> MC service group ID | Y | Y | Y |  |
|  | >>> Group management server URI | Y | Y | Y |  |
| NOTE 1: If authorized to set and modify target MC service users for recording, one and only one of these parameter sets shall be set. Recording of MCData users is not supported in this version of this specification. NOTE 2: If authorized to set and modify target groups for recording, one and only one of these parameter sets shall be set. Recording of MCData groups is not supported in this version of this specification. NOTE 3: If authorized to replay recordings of users, one and only one of these parameter sets shall be set. Recording/replay of MCData users is not supported in this version of this specification. NOTE 4: If authorized to replay recordings of groups, one and only one of these parameter sets shall be set. Recording/replay of MCData groups is not supported in this version of this specification. NOTE 5: The list may contain zero or more of each of its subitems, but if included, the list shall not be empty. |  |  |  |  |  |

# * * * * End of changes * * * *