3GPP TSG-SA WG6 Meeting #65	S6-250369

Athens, Greece 17th – 21st February 2025	(revision of S6-250174)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0365 | rev | 1 | Current version: | 18.10.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resvole the EN about the security and privacy aspects for location management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution main resolves the following ENs about the security and privacy aspects for location management: Editor's note: The security and privacy aspects related to VAL server obtaining UE's location information are to be addressed by SA3. Editor's Note: It's FFS the security aspects for LM-Uu and LM-S in relation to the VAL service ID that need to be coordinated with SA3. In general, the security issues are to be addressed by SA3 and SA6 needs to coordicate with SA3 for final realization. It’s also same for the location management in application enabled layer. What’s more, no matter the security issues about the VAL server obtaining UE’s location information or LM-Uu and LM-S in relation to the VAL service ID, they all belong to the location management security related issues. So it will be better to summarize all of security and privary issues related to the location management in a word as following: NOTE: The security and privacy aspects related to location management need to be coordinated with SA3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace the EN with the NOTE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s Notes in frozen specifications impact the quality of the TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1,9.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3	Procedures and information flows for Location management (on-network)

9.3.1	General

Location information of VAL service user shall be provided by the location management client to the location management server. The location information reporting triggers are based on the location reporting configuration. Different type of location information can be provided.

NOTE:	The security and privacy aspects related to location management (e.g. VAL server obtains UE’s location information  or LM-Uu and LM-S in relation to the VAL service ID) are out of scope of this specifiction.

* * * Next Change * * * *

9.3.2.3	Location information request

Table 9.3.2.3-1 describes the information flow from the VAL server to the location management server and/or from the location management server to the location management client for requesting an immediate location information report.

Table 9.3.2.3-1: Location information request

| Information element | Status | Description |
| --- | --- | --- |
| Identity list | M | List of VAL users or VAL UEs whose location information is requested |
| VAL service ID | O | Identity of the VAL service for which the location information is requested. |
| Requested location information | O | Identifies what location information is requested |
| Requested location access type (NOTE 4) | O (NOTE 1) | Identifies the location access type for which the location information is requested, e.g. as described in TS 23.273[50] and TS 29.572[51]. |
| Requested positioning method | O (NOTE 1) | Identifies the positioning method for which the location information is requested, e.g. as described in TS 23.273[50] and TS 29.572[51]. If the requested VAL user/UE is not the receiving VAL user/UE hosting the location management client, the UE positioning method includes PC5 SEAL LM, BT, WiFi. |
| Location QoS | O (NOTE 3) | Definition of the location Quality of Service for which the location information is requested (see NOTE 2). |
| Velocity requested indication | O (NOTE 5) | It indicates whether the velocity of the requested VAL users/UEs is needed. |
| List of association IDs | O (NOTE 6) | Identifies list of associations with other UEs may share the location information when they are at the same location. |
| NOTE 1: This e element is only applicable for request sent from the location management server to the location management client. NOTE 2: The definition of location QoS has been defined in clause 4.1b of TS 23.273 [50] and the clause 6.1.6.2.13 of TS 29.572 [51]. NOTE 3: The element is only applicable for the information flow from the VAL server to the location management server. NOTE 4: The non-3GPP access as defined in TS 23.273[50] and TS 29.572[51] is out of scope of the present specification. NOTE 5: The definition of the velocity refers to 3GPP TS 23.271 [60]. NOTE 6: The association IDs are known to the UE based on offline methods and are out of scope of this specification. The requestor can be part of multiple associations and one association can have two or more UEs. |  |  |

* * * End of Changes * * * *