3GPP TSG-SA WG6 Meeting #65	S6-250382

Athens, Greece 17th – 21st February 2025	(revision of S6-250145)

Source:	China Mobile, Huawei, HiSilicon

Title:	pCR on adding Application ID

Spec:	3GPP TS 23.392 v1.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR is  proposed to update of 8.2.1 DC application and profile configuration. S6-250075 is merged to this document.

2. Reason for Change

This pCR is  proposed to update of 8.2.1 DC application and profile configuration.

4. Proposal

This pCR is  proposed to update of 8.2.1 DC application and profile configuration.

* * * First Change * * * *

7	Identities

7.1	General

The following clauses list identities that are used in this technical specification.

7.2	Application ID (APPID)

The APPID is a globally unique value that identifies a Data Channel Application. It is assigned by the MMTel Enabler Server. The value of this IE is also used to set the "req-app-id" parameter in "a=3gpp-req-app" media-level SDP attribute as specified in 3GPP TS 26.114 [4] when the application requests application data channel(s).

* * * Next Change * * * *

#### 8.2.1.2	Procedure

Figure 8.2.1.2-1 illustrates the DC application and profile configuration procedure.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have established a secured connection and configured with the necessary credentials to enable authenticating one another.

3.	Both DC application and profile have been uploaded to Controlling Application Server.

Figure 8.2.1.2-1: DC application and profile configration

1.	The Controlling Application Server sends a DC Application and Profile configuration request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.2.1.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, If the requester is authorized then the MMTel Enabler Server initiates authentication procedures with the Requester Identity. If the registration is successful, the MMTel Enabler Server allocates an APPID and creates the DC Application Profile for each DC Application included in the DC Application and Profile configuration request, and maintain the mapping of APPID and DC Application Profile.

3.	The MMTel Enabler Server configures the DC Application and Profile on DCAR.

4.	The MMTel Enabler Server sends a DC Application and Profile configuration response to the Controlling Application Server. The response message includes information elements as specified in clause 8.2.1.3.2.

Editor's note:	 The detailed interactions between the MMTel Enabler Server and DCAR are FFS.

* * * Next Change * * * *

##### 8.2.1.3.2	DC Application and profile configuration response

Table 8.2.1.3.2-1 describes information elements for the DC Application and profile configuration response from the MMTel Enabler Server.

Table 8.2.1.3.2-1: Information elements for DC Application and Profile configuration response

| Information element | Status | Description |
| --- | --- | --- |
| DC Application and profile configuration response list | M | A list of DC Application and profile configuration response. This list contains a response for each DC Application. The detailed information is listed in Table 8.2.1.3.2-2. |

Table 8.2.1.3.2-2: Information elements in DC Application and profile configuration response list

| Information element | Status | Description |
| --- | --- | --- |
| Application Identifier | M | Identifier of the Data Channel Application assigned by the Application Provider. |
| Successful response | O | Indicates the success of the configuration of this DC Application and Profile |
| > APPID | M | Unique identifier of the Data Channel Application |
| Failure response | O | Indicates the failure of the configuration of this DC Application and Profile |
| > Reason | O | Indicates the reason for failure. |

* * * Next Change * * * *

8.2.2.2	Procedure

Figure 8.2.2.2-1 illustrates the DC application and profile update procedure.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have established a secured connection and configured with the necessary credentials to enable authenticating one another.

Figure 8.2.2.2-1: DC application and profile update

1.	The Controlling Application Server sends a DC Application and Profile update request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.2.2.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, If the requester is authorized then the MMTel Enabler Server initiates authentication procedures with the Requester Identity. If the validation is successful, the MMTel Enabler Server updates the DC Application or DC Application Profile for each DC Application based on information provided in the request.

3.	The MMTel Enabler Server updates the DC Application and Profile to DCAR.

4.	The MMTel Enabler Server sends a DC Application and Profile update response to the Controlling Application Server. The response message includes information elements as specified in clause 8.2.2.3.2.

Editor's note:	 The detailed interactions between the MMTel Enabler Server and DCAR are FFS.

8.2.2.3	Information flows

8.2.2.3.1	DC Application and profile update request

Table 8.2.2.3.1-1 describes information elements for the DC Application and profile update request from the Controlling Application Server to the MMTel Enabler server.

Table 8.2.2.3.1-1: Information elements in DC Application and Profile update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| Number of DC Application included | M | Indicates total number of DC Application included in this request |
| DC Application list | M | A list of DC application and configuration parameters included in this request. Each element in this list contains a Data Channel Application package and corresponding configuration parameters. The detailed information elements of DC application and profile are listed in Table 8.2.1.3.1-2 except that the Application Identifier is replaced by APPID. |

Table 8.2.2.3.1-2: Information elements in DC application list

| Information element | Status | Description |
| --- | --- | --- |
| APPID | M | Unique identifier of the Data Channel Application |
| Application Name | O | Name of the Data Channel Application |
| Service Type | O | Service type of the Data Channel Application, used to help the user understand the type of services provided by the application |
| Application Icon Url | O | Icon url of the Data Channel Application |
| Application Version | O | The newest version of the application. |
| Application Validity | O | The latest validity of the application |
| Application Loading Phase | O | Indicates when this Data Channel Application is allowed to be used. The values of this IE include: Precall only:the Data Channel Application is allowed to be used before the MMTel call session is established, i.e. after the 18x response is sent/received and before the 200 OK of the initial SIP INVITE request is sent/received. Incall:the Data Channel Application is allowed to be used after the MMTel call session is established, i.e. after the 200 OK of the initial SIP INVITE request is sent/received. Precall+Incall: the Data Channel Application is allowed to be used during the entire Precall and incall. |
| Autoload | O | Indicates whether this Data Channel Application is needed to be load to the UE automatically. |
| Autolaunch | O | Indicates whether this Data Channel Application is needed to be downloaded to the UE and run automatically. |
| If Work Without Peer DC | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Supported Scenario | O | Indicates supported media type required for this Data Channel Application. The values of this IE include: Voice call only: this Data Channel Application can be used if and only if the corresponding call is a voice call. Video call only: this Data Channel Application can be used if and only if the corresponding call is a video call. Voice and video call: this Data Channel Application can be used in both voice call and video call. |
| Condition | O | Indicates whether this Data Channel Application can be used if under this condition. The values of this IE include but not limited: Service area: the application is allowed to be used in a certain area. Time period: the application is allowed to be used for a certain period of time. |
| 3gpp Qos Hint | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| PersonalDataCollection | O | Indicates whether this Data Channel Application will collect the user personal data (along with related consent opt-in/opt-out procedures). |
| PersonalDataCollectionInfoURL | O | URL to retrieve the description of the purpose of mandatory and optional personal data to be collected, their processing their protection etc. |
| Application Package (see NOTE 2) | O | The Code package of the Data Channel Application. |
| NOTE 1: At least one of these optional IEs is present in the update request. A special case. When the application is updated, the request must carry the application package and application version. NOTE 2: The Application Package also support incremental updates. |  |  |

8.2.2.3.2	DC Application and profile update response

Table 8.2.2.3.2-1 describes information elements for the DC Application and profile update response from the MMTel Enabler Server.

Table 8.2.2.3.2-1: Information elements for DC Application and Profile update response

| Information element | Status | Description |
| --- | --- | --- |
| DC Application and profile update response list | M | A list of Application and profile update response. This list contains a response for each DC Application and Profile. The detailed information is listed in Table 8.2.2.3.2-2. |

Table 8.2.2.3.2-2: Information elements in DC Application and profile update response list

| Information element | Status | Description |
| --- | --- | --- |
| APPID | M | Unique identifier of the Data Channel Application |
| Successful response | O | Indicates the DC Application and Profile update resquest was successful. |
| Failure response | O | Indicates that the DC Application and Profile update resquest failed |
| > Cause | O | Indicates the cause of DC Application and Profile update resquest failure. |

* * * Next Change * * * *

8.2.3.2	Procedure

Figure 8.2.3.2-1 illustrates the DC application deletion procedure.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have established a secured connection and configured with the necessary credentials to enable authenticating one another.

Figure 8.2.3.2-1: DC application deletion

1.	The Controlling Application Server sends a DC Application delete request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.2.3.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, If the requester is authorized then the MMTel Enabler Server initiates authentication procedures with the Requester Identity. If the registration is successful, the MMTel Enabler Server requests the DCAR to delete the DC Application and Profile (DC applications and Profile need to be deleted at the same time) and delete the mapping of APPID and DC Application Profile.

3.	The MMTel Enabler Server deletes the DC Application and Profile on DCAR.

4.	The MMTel Enabler Server sends a DC Application delete response to the Controlling Application Server. The response message includes information elements as specified in clause 8.2.3.3.2.

Editor's note:	 The detailed interactions between the MMTel Enabler Server and DCAR are FFS.

8.2.3.3	Information flows

8.2.3.3.1	DC Application delete request

Table 8.2.3.3.1-1 describes information elements for the DC Application delete request from the Controlling Application Server to the MMTel Enabler server.

Table 8.2.3.3.1-1: Information elements in DC Application delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| Number of DC Application included | M | Indicates total number of DC Application included in this request |
| List of APPID | M | List of identifiers of the DC application to be deleted. |

8.2.3.3.2	DC Application delete response

Table 8.2.3.3.2-1 describes information elements for the DC Application delete response from the MMTel Enabler Server.

Table 8.2.3.3.2-1: Information elements for DC Application delete response

| Information element | Status | Description |
| --- | --- | --- |
| DC Application delete response list | M | A list of Application deletion response. The detailed information is listed in Table 8.2.3.3.2-2. |

Table 8.2.3.3.2-2: Information elements in DC Application delete response list

| Information element | Status | Description |
| --- | --- | --- |
| APPID | M | Unique identifier of the Data Channel Application |
| Successful response | O | Indicates the deletion of the DC Application was successful |
| Failure response | O | Indicates the deletion of the DC Application was failed |
| > Cause | O | Indicates the cause of failure |

* * * Next Change * * * *

8.2.4.2	Procedure

Figure 8.2.4.2-1 illustrates the procedure of getting DC application profile information.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have established a secured connection and configured with the necessary credentials to enable authenticating one another.

Figure 8.2.4.2-1: Getting DC application profile information

1.	The Controlling Application Server sends a DC Application and Profile information retrieval request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.2.4.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, If the requester is authorized then the MMTel Enabler Server initiates authentication procedures with the Requester Identity. If the registration is successful, the MMTel Enabler Server gets the DC application Profile information based on the APPID included in the Getting DC Application Profile information request.

3.	The MMTel Enabler Server sends a Getting DC Application Profile information response to the Controlling Application Server. The response message includes information elements as specified in clause 8.2.4.3.2.

8.2.4.3	Information flows

8.2.4.3.1	Getting DC Application profile information request

Table 8.2.4.3.1-1 describes information elements for the Getting DC Application profile information request from the Controlling Application Server to the MMTel Enabler server.

Table 8.2.4.3.1-1: Information elements in Getting DC Application Profile information request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| Number of DC Application included | M | Indicates total number of DC Application included in this request |
| APPID list | M | A list of APPID for the request. |

8.2.4.3.2	Getting DC Application profile information response

Table 8.2.4.3.2-1 describes information elements for the Getting DC Application profile information response from the MMTel Enabler Server.

Table 8.2.4.3.2-1: Information elements in Getting DC Application Profile information response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the Getting DC Application profile information request was successful. |
| > List of DC Application profile information | O | List of DC Application profile information as defined in Table 8.2.1.3.1-2 except that the Application Identifier is replaced by APPID. |
| Failure response | O | Indicates that the Getting DC Application profile information request failed. |
| > Cause | O | Indicates the cause of Getting DC Application profile information request failure. |

* * * Next Change * * * *

8.3.2.3.4	Get Application Profile List response

Table 8.3.2.3.4-1 describes information elements for the Get Application Profile List response from the MMTel Enabler server to the MMTel Enabler client.

Table 8.3.2.3.4-1: Information elements in Get Application Profile List response

| Information element | Status | Description |
| --- | --- | --- |
| Data Channel Application profile list | M | A list of Data Channel Application profiles available for this user. Each element in this list contains a DC application profile of this Data Channel Application. The detailed information elements of DC application profile are listed in Table 8.3.2.1-5. |

Table 8.3.2.3.4-2: Information elements in DC application profile

| Information element | Status | Description |
| --- | --- | --- |
| APPID | M | Unique identifier of the Data Channel Application |
| Application Name | O | Name of the Data Channel Application |
| Service Type | O | Service type of the Data Channel Application, used to help the user understand the type of services provided by the application |
| Application Icon | O | Icon of the Data Channel Application |
| Application Version | O | The newest version of the application. |
| Application Validity | O | The latest validity of the application. |
| Application Loading Phase | O | Indicates when this Data Channel Application is allowed to be used. The values of this IE include: Precall only:the Data Channel Application is allowed to be used before the MMTel call session is established, i.e. after the 18x response is sent/received and before the 200 OK of the initial SIP INVITE request is sent/received. Incall:the Data Channel Application is allowed to be used after the MMTel call session is established, i.e. after the 200 OK of the initial SIP INVITE request is sent/received. |
| Autoload (see NOTE 1) | O | Indicates whether this Data Channel Application is needed to be load to the UE automatically. |
| Autolaunch (see NOTE 1) | O | Indicates whether this Data Channel Application is needed to be downloaded to the UE and run automatically. |
| If Work Without Peer DC | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Supported Scenario | O | Indicates supported media type required for this Data Channel Application. The values of this IE include: Voice call only: this Data Channel Application can be used if and only if the corresponding call is a voice call. Video call only: this Data Channel Application can be used if and only if the corresponding call is a video call. Voice and video call: this Data Channel Application can be used in both voice call and video call. |
| Condition | O | Indicates whether this Data Channel Application can be used if under this condition. The values of this IE include but not limited: Service area: the application is allowed to be used in a certain area. Time period: the application is allowed to be used for a certain period of time. |
| 3gpp Qos Hint | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| PersonalDataCollection (see NOTE 2) | O | Indicates whether this Data Channel Application will collect the user personal data. |
| PersonalDataCollectionInfoURL (see NOTE 2) | O | URL to retrieve the description of the purpose of mandatory and optional personal data to be collected (along with related consent opt-in/opt-out procedures), their processing their protection etc. |
| NOTE 1: These IEs can be applied only if the specific service using this data channel application is agreed to be used by the user based on the subscription of this specific service, or explicitly agreed to be applied by the user on the UE. NOTE 2: These IEs are mandatory when regulatory requirements request that the user has to be informed if the personal data are collected by the application and how they are treated. |  |  |