3GPP TSG-SA WG6 Meeting #65	S6-250377

Athens, Greece 17th – 21st February 2025	(revision of S6-250276)

Source:	Ericsson

Title:	Pseudo-CR on MMTel_App_DCH_alignment_with_SA2_A2P_flow

Spec:	3GPP TS 23.392 v. 1.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	Cristina Badulescu, Ericsson; cristina.badulescu@ericsson.com

1. Introduction

A2P: Alignment with the SA2 flows and agreed solution.  SA2 has removed the use of OMA API in their flows in 23.228 Rel-19, this CR updates the TS23.392 to align with the SA2 solution.  
Current solution using “OMA API” entity to interact with the MMTel AS does not work, as the OMA API session ID is the REST session ID and not the IMS session ID which is not passed over in the REST API. So the assumption that the IMS session ID is avaialble to the MMTel enabler server is faulted.

The architecture figure in Figure 6.2.1-1: MMTel Enabler architecture shows the MMTel enabler server interacting via N33, DC4, MDC2, MDC3 with EPC/5GC/IMS but sections 8.4.1 and 8.4.2 show a different interaction not based on N33, nor DC4 but rather interacting with an entity “OMA API”. Unclear which entity in the network is designated by the “OMA API” box (which AEF maps to in the figure 6.2.1-1 architecture)2.

Reason for Change

Update the flows to replace OMA API with exposed NEF capabilities o manage an IMS session for the A2P case. Removal of Editor’s Notes.3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v. 1.0.0

* * * First Change * * * *

### 8.4.x	Establishing an Application Call with Data Channel capability (A2P)

### 8.4.x.1 Procedure

Figure 8.4.x-1 illustrates the operation to establish an Application Call with Data Channel capability between the Application and the MMTel Enabler Server in order to add an A2P Data Channel to an existing IMS session of the user on UE1 in alignment with the A2P flow specified in [3].

Pre-conditions:

1.	The Application can use DCH (supports the media set information for DCH) and is authorized to use the service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in SA2 [3].

3.	The MMTel Enabler Server is authorized to use the NEF exposed IMS session management capabilities as specified in [8].

4. The MMTel Enabler Server is subscribed to the AEF (NEF) for IMS session events information e.g., media changes such as requested for the specific DCH Application, for a list of users.

NOTE: how the IMS session ID is provided by the NEF exposure of IMS events [8] is out of scope of this specification.

5. The DCH Application Server is subscribed to the MMTel enabler server for events related to the DCH application.

6. An IMS MMTel session is ongoing for User (UE1) with another user who requests the DCH Application to provide some DCH media to the user on UE1.

Figure 8.4.x-1: Establishing a Application Call with Data Channel capability

- The user sends a request, via the 5GC and IMS, to add DCH media for a specific DCH Application. The NEF notifies the MMTel Enabler Server of the new IMS session event for the for the media changes related to the DCH Application. The request includes parameters such as the session ID, MDC2 endpoint, A2P type.

- The MMTel enabler server requests an IMS session update to the NEF, providing the Session ID and MDC2 endpoint address. The NEF propagates the request to the IMS core which sets the media operation. The NEF sends the response to the MMTel enabler to the IMS session Update request.

- The MMTel enabler server sends the response to the DCH Application Server including the Session ID for the Add media request.

- Once the IMS system completed the media preparation for the IMS session, the NEF sends an IMS session management notification request to the MMTel enabler server with MDC2 resource information for the Session ID. The DCH Application Server responds (acknowledges) the MMTel enabler server notification.

- The MMTel enabler server notifies the DCH Application Server of the MDC2 resource information for the Session ID.

- The MMTel enabler server acknowledges the NEF notification.

Post-condition: The UE1 upon getting the updated IMS session information for the DCH media proceeds to download the DCH Application.

* * * Next Change * * * *

### 8.4.x.2	Information flows

##### 8.4.x.2.1	Application Add Data Chanel media to UE1 session (A2P) request

Table 8.4.x.2.1-1: Application Add Data Chanel media to UE1 session (A2P) request

| Information element | Status | Description |
| --- | --- | --- |
| Data Channel Application ID | M | The identifier of the DCH application |
| Session ID | M | The session identifier of the session on which MMTel enabler server must add the new media. |
| Media resource information | O | The media information to be passed via the Data Chanel. It includes information described in SA2 [8] clause 5.2.6.40. |
| Application Profile requested | O | The DC application profile expected to be used in this call. |
| Notification address | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. |

##### 8.4.x.2.2	Application Add Data Chanel media to UE1 session (A2P) response

Table 8.4.x.2.2-1: Application Add Data Chanel media (A2P) response

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel enabler server must add the new media. |
| Successful response | O | Indicates the Notificaiton of Application of DCH media info (A2P) request was successful |
| Failure response | O | Indicates the Notificaiton of Application of DCH media info (A2P) request has failed |
| > Cause | O | Indicates the cause of request failure |

##### 8.4.1.y.3	Notify Application of DCH media info (A2P) request

Table 8.4.1.y.3-1: Notify Application of DCH media info (A2P) request

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel enabler server must add the new media. |
| Notification address | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. |
| Media resource information | O | The media information to be passed via the Data Chanel. It includes information described in SA2 [8] clause 5.2.6.40. |
| Application Profile requested | O | The DC application profile expected to be used in this call. |

##### 8.4.1.y.4	Notify Application of DCH media info (A2P) response

Table 8.4.1.y.4-1: Notify Application of DCH media info (A2P) response

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel enabler server must add the new media. |
| Successful response | O | Indicates the Notificaiton of Application of DCH media info (A2P) request was successful |
| Failure response | O | Indicates the Notificaiton of Application of DCH media info (A2P) request has failed |
| > Cause | O | Indicates the cause of request failure |

* * * End of Changes * * * *