3GPP TSG-SA WG6 Meeting #65	S6-250584

Athens, Greece, 17th – 21st February 2025	(revision of S6-250207)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0249 | rev | 3 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE accessing resources not owned by that UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Solution#24 of TR 23.700-22 v19.0.0 develops a solution that enables a UE-deployed API invoker can access to resources not owned by that UE, based on the control in the CCF of the authorization relationship between UEs, (CAPIF controls that UE2 is authorized by UE1 to access UE1 resources). The proposal contains two aspects to consider: (1) how UE2 identity is handled in the Obtain Service API authorization procedure and (2) how to solve the use case when UE1 and UE2 belong to different CAPIF provider domains. For Rel-19 it is proposed to describe two variants of the solution, based on: - the control of the authorization relationship between UEs is only in the application (i.e., the application controls that UE2 is authorized by UE1 to access UE1 resources). This option does not require that UE2 identity is handled in the procedure; and/or - the control of the authorization relationship is in the CAPIF provider domain. This option requires UE2 identity is handled in the procedure. In addition, the solution considers that: - both UE1 and UE2 belong to the same CAPIF domain; and - the control in the CCF for the authorization of the API Invoker of UE2 to access to resources of UE1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The solution that enables a UE can access to resources that belong to other UEs has as precondition that: - All the UEs belong to the same CAPIF provider domain. - The CCF authenticates and authorizes the API Invoker of UE2 to access UE1 resources. The API invoker of UE2 includes, in the obtain service API authorization request the RO Identity of UE1. If the authorization relationship between UE1 and UE2 is in the CAPIF provider domain, the API invoker also includes in the request the UE2 identity. The CCF checks whether the UE1 authorized the access by the application to the network resource, and if UE2 was included in the request, also if UE2 is authorized. The authorization information returned by the CCF to the API invoker contains the identity of the resource owner and the authorized scope. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The problem of enabling CAPIF can authorize a UE-deployed API invoker to access to resources not owned by that UE is not solved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x(NEW), 8.x.1(NEW), 8.x.2(NEW), 8.x.3 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 8.x	UE-deployed API invoker accessing other UEs’ resources

### 8.x.1	General

This clause specifies the procedures for API invoker on a UE to be able to access the services exposing resources of other UEs.

The following scenarios are supported:

1.	Obtaining resource owner authorization information by the API invoker when the authorization relationship between UE2 (the API invoker is in UE2) and UE1 (UE1 is the resource owner of the accessed network resources) is not in CAPIF provider domain (it is e.g. in the application). The difference with the procedure described in clause 8.31 is that the API invoker is deployed in a UE different to the resource owner UE.

2.	Obtaining resource owner authorization information by the API invoker when the authorization relationship between UE2 (the API invoker is in UE2) and UE1 (UE1 is the resource owner of the accessed network resources) is in the CAPIF provider domain. The difference with the procedure described in clause 8.31 is that the API invoker is deployed in a UE different to the resource owner UE and the API invoker needs to provide in the request, in addition to the identity of the resource owner, the identity of the UE accessing to resource owner’s resources.

NOTE:	In this release of the specification, the UEs belong to the same CAPIF provider domain.

* * * Next Change * * * *

### 8.x.2	Information flows

NOTE:	The security aspects of this procedure are specified in 3GPP TS 33.122 [12].

* * * Next Change * * * *

### 8.x.3		Procedure

Figure 8.x.3-1 presents the procedure for enabling a UE-hosted API invoker accessing network-hosted resources owned by other UEs for the scenarios introduced in clause 8.x.1.

Pre-condition:

1.	The network hosts individual resource(s) for a UE which can be accessed via AEF provided the necessary security conditions are met.

2.	UE 1 and UE 2 belong to the same CAPIF provider domain. The API invoker in UE2 knows the RO identifier for UE1.

3.	The CAPIF domain stores resource owner authorization information for UE1 for accessing to network resource(s). When the authorization relationship between UE1 and UE2 is in CAPIF provider domain, the resource authorization information of UE1 contains whether UE2 is allowed to access to UE1 network resources.

Figure 8.x.3-1: UE-deployed API invoker accessing other UEs’ resources

1.	The resource owner (UE1) provides resource owner authorization information for accessing network hosted resource(s). When the authorization relationship between UE2 and UE1 is in the CAPIF provider domain, the resource owner authorization information provided by UE1 includes the authorization grants for UE2.

2.	The API invoker in UE2 sends an Obtain service API authorization request to the CCF for obtaining permission to access the service API for other UE’s (UE1) resources hosted in the network by including the API invoker identity information and any information required for authentication of the API invoker. If the control of the authorization relationship between UE1 and UE2 is only not in the CAPIF provider domain (it is e.g. in the application), the request shall include the RO identity for UE1, whose resources are trying to be accessed, the application that is requesting the access, and the information about the purpose and scope to be performed on the targeted resources. If the control of the authorization relationship between UE1 and UE2 is in the CAPIF provider domain, then the request shall include the identity of UE2.

3.	The CCF validates and authorizes the API invoker in UE2 as specified in 3GPP TS 33.122 [3]. If the API invoker validation is successful, the CCF checks the Resource Owner authorization information of UE1. The CCF obtains whether UE1 authorized to the application the access to the network resource(s) for the specified purpose and scope. If the UE2 identity was received in step 1, the CCF shall additionally validate whether UE2 is authorized by UE1. If the RO authorization information is not available, the CCF may interact with UE1 via ROF to obtain it.

4.	Based on the UE1 resource owner authorization obtained in step 2, the CCF sends an Obtain service API authorization response to the API invoker. The response indicates that UE1, as resource owner, authorizes the service API access for an indicated scope.

5.	The API invoker sends service API invocation request to the API exposing function with the RO authorization information received in step 2.

6.	The API invoker receives the service API invocation response resulting from the service API invocation once the API exposing function has checked whether the API invoker is authorized to invoke that service API based on the RO authorization information.

* * * END of Change * * * *