3GPP TSG-SA WG6 Meeting #66	S6-251035

Gothenburg, Sweden, 7th – 11th April 2025	(revision of S6-xxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0020 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial anchor group operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-App |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is group object in WID Metaverse_Ph2-App: 3. Enhancements to Spatial anchors service for Group handling of spatial anchors |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add feature for spatial anchor group operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No realize group object. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

8.3.x	Spatial anchor group

8.3.x.1	General

Spatial anchor group enhancements to spatial anchors service for group handling of spatial anchors.

8.3.x.2	Spatial anchor group creation

The spatial anchor group creation operation by SAn client or VAL server to create a spatial anchor group, which follows the procedure of group creation in clause 10.3.3 of 3GPP TS 23.434 [4].

The spatial anchor group creation request follows the information flow group creation request in table 10.3.2.1-1 of 3GPP TS 23.434 [4], the Identity list is List of Spatial anchor ID.

The spatial anchor group creation response follows the information flow group creation response in table 10.3.2.2-1 of 3GPP TS 23.434 [4], the VAL group ID is Spatial anchor group ID.

The spatial anchor group creation notification follows the information flow group creation notification in table 10.3.2.3-1 of 3GPP TS 23.434 [4], the VAL group ID is Spatial anchor group ID, the Identity list is List of Spatial anchor ID.

8.3.x.3	Spatial anchor group information query

The spatial anchor group information query operation by SAn client or VAL server to request the membership list on a spatial anchor group, which follows the procedure of group information query in clause 10.3.4.2 of 3GPP TS 23.434 [4].

The spatial anchor group information query request follows the information flow group information query request in table 10.3.2.4-1 of 3GPP TS 23.434 [4], the VAL group ID is Spatial anchor group ID.

The spatial anchor group information query response follows the information flow group information query response in table 10.3.2.5-1 of 3GPP TS 23.434 [4], the VAL group ID is Spatial anchor group ID.

8.3.x.4	Spatial anchor group membership

The spatial anchor group membership operations by SAn client or VAL server to subscribe/notify and update the spatial anchor group membership, which follows the procedures of group membership in clause 10.3.5 of 3GPP TS 23.434 [4].

The spatial anchor group information subscribe request follows the information flow group information subscribe request in table 10.3.2.14-1 of 3GPP TS 23.434 [4], the VAL group ID is Spatial anchor group ID.

The spatial anchor group information subscribe response follows the information flow group information subscribe response in table 10.3.2.15-1 of 3GPP TS 23.434 [4], the VAL group ID is Spatial anchor group ID.

The spatial anchor group membership notification follows the information flow group membership notification in table 10.3.2.8-1 of 3GPP TS 23.434 [4], the VAL group ID is Spatial anchor group ID, the Identity is Spatial anchor ID.

The spatial anchor group membership update request follows the information flow group membership update request in table 10.3.2.6-1 of 3GPP TS 23.434 [4], the VAL group ID is Spatial anchor group ID, the Identity is Spatial anchor ID.

The spatial anchor group membership update response follows the information flow group membership update response in table 10.3.2.7-1 of 3GPP TS 23.434 [4], the VAL group ID is Spatial anchor group ID.

8.3.x.5	Spatial anchor group deletion

The spatial anchor group deletion operation by SAn client or VAL server to delete a spatial anchor group, which follows the procedure of group deletion in clause 10.3.13 of 3GPP TS 23.434 [4].

The spatial anchor group deletion request follows the information flow group deletion request in table 10.3.2.9-1 of 3GPP TS 23.434 [4], the VAL group ID is Spatial anchor group ID.

The spatial anchor group deletion response follows the information flow group deletion response in table 10.3.2.10-1 of 3GPP TS 23.434 [4], the VAL group ID is Spatial anchor group ID.

The spatial anchor group deletion notification follows the information flow group deletion notification in table 10.3.2.11-1 of 3GPP TS 23.434 [4], the VAL group ID is Spatial anchor group ID.

# * * * * End of changes * * * *