3GPP TSG-SA WG6 Meeting #66	S6-251045

Gothenburg, Sweden 7th – 11th April 2025

Source:	Motorola Solutions

Title:	Discussion paper on supporting non-session-based MC service recording.

Agenda Item:	9.2

Contact:	Harish N (harish.negalaguli@motorolasolutions.com)

Adinarayana K Setty (adinarayana.setty@motorolasolutions.com)

# 1	Introduction.

The 3GPP specifications currently outline procedures for supporting MC services such as MCPTT, MCVIDEO, and MCDATA services. The Release 19 version has introduced procedures for supporting recording of session-based services. This discussion paper aims to contribute to the ongoing dialogue and presents a proposal for supporting recording for non-session-based services.

# 2	Proposed architecture for supporting non session based recording.

## 2.1	General

The 3GPP standards TS 23.280 [1], TS 23.379 [2], TS 23.281[3], and TS 23.282[4] outline Architecture and flows for various mission-critical services, including Mission Critical Push-to-Talk (MCPTT), Mission Critical Data (MCDATA), Mission Critical Video (MCVIDEO), and Mission Critical Location (MCLocation) services. Non-session based recording in this context means capturing and storing communication and multimedia data independently of a specific call or session.

Below mentioned procedure lists recording support for non session based procedures followed while supporting MC services (some of the non session based procedures are MCDATA, MC Location, Emergency alerts e.t.c).

## 2.2	Solution to support MCDATA service recording.

The 3GPP specifications TS 23.282 [4] and TS 24.282 [5] outline the architecture, flows, and procedures for supporting MCDATA communication. The following procedure proposes a recording flow for the MCDATA service.

Upon completing the delivery of the MCDATA FDS/SDS message (with or without acknowledgment) to the target MC users, the MCDATA server will initiate the transfer of MCDATA conversation information to the recording server. This transfer will include Metadata, MCDATA Messaging content, file attachments (if any) etc.

On receipt of MCDATA specific recorded information, recording server shall push this information to Recording mass storage server (for later retrieval).

Figure 2.2-1: MCDATA recording flow.

## 2.3	Solution to support non-session-based recording.

The 3GPP standards TS 23.280 [1], TS 23.379 [2], TS 23.281[3], and TS 23.282[4] outline the requirements and frameworks for various mission-critical services, including Mission Critical Push-to-Talk (MCPTT), Mission Critical Data (MCDATA), Mission Critical Video (MCVIDEO), and Mission Critical Location (MCLocation) services.

Proposal is for supporting non-session-based events generated while supporting MCPTT, MCVIDEO, MCDATA, MC Location service such as Location Information, Emergency alerts etc.

Figure 2.2-1: Non session-based recording flow.

# 3 	Proposed non-session-based data transfer methods.

## 3.1	Using Secured FTP (SFTP).

Secured FTP (SFTP) can be utilized to transfer non-session-based recorded information from the recording client to the server. Recording information involving metadata, actual media files can be stored in files and transferred either immediately or periodically from the recording client to the recording server.

### 3.1.1 Proposed Usage of SFTP:

Preconfigured SFTP User: The SFTP user designated for file transfer will be preconfigured with the necessary permissions to send and receive files. A password less SFTP transfer is the preferred method for setup.

Designated Transfer Path: The paths for transferring metadata and media files could be predefined.

Segregation of Non-Session Based Recording Information: Non-session-based recording information collected over time will be organized based on criteria such as MCX user, group, or corporate entity, and then compressed into one or more ZIP files.

Transfer of Information using SFTP : Collected non session based recording content shall be transferred periodically from recording client to recording server.

## 3.2	Using RESTful API’s (using HTTPS)

RESTful APIs can be designed to transfer non-session-based recorded information from the recording client to the recording server. These APIs can also facilitate the transfer of large files in chunks. Proposed Usage of REST API’s:

### 3.2.1 	Proposed Usage of REST API's:

Authentication:
A well-established authentication mechanism will be employed for invoking the non-session-based recording data transfer APIs.

- A digest-based authentication method may be utilized.

Context Creation:
The recording client will establish a REST API session context with the recording server to facilitate the transfer of non-session-based recording content.

Transfer of Recording Content:
 Within the established REST API session, the client will transfer recording information, which may include multiple metadata files, media content files etc.

Ending the Context:
Upon completing the required recording data transfer, the REST API session context created for the transfer must be closed.

# 4	Conclusion

It is advisable to use secured FTP (SFTP) for transferring non-session-based information from recording client to recording server.  Since sftp is secured and better proposed method for transferring larger files from recording client to server.

# 5	References

[1]		3GPP TS 23.280 : Common functional architecture to support mission critical services

[2]	3GPP TS 23.379 : Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT)

[3]	3GPP TS 23.281 : Functional architecture and information flows to support Mission Critical Video (MCVideo).

[4]	3GPP TS 23.282 : Functional architecture and information flows to support Mission Critical Data (MCData)

[5]		3GPP TS 24.282 : Mission Critical Data (MCData) signalling control