3GPP TSG-SA WG6 Meeting #66	S6-251068

Göteborg, Sweden, 07th – 11st April 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	Correction to SBA of 3GPP System Supporting SEAL Services

Spec:	3GPP TR 23.700-35 v0.3.0

Agenda item:	9.3

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

Application client can invoke SEAL server services via SEAL client(s), but not support to consume SEAL server services directly. This paper proposes to correct the errors in clasues 8.2.1 and 8.2.2.

2. Reason for Change

In different specifications, different terms are used for representing the same thing i.e., VAL client. For example, VAL client in TS 23.434 is used to represent entity that provides the client side functionalities corresponding to the vertical applications, application client in TS 23.558 is used to represent the application resident in the UE performing the client function in application. This paper introduces corrections to clause 8.2.1 by removing direct link between application client and SEAL server.

3. Conclusions

This pCR proposes to remove the direct link between application client and SEAL server.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-35 v0.3.0.

* * * First Change * * * *

## 8.2	Solutions for SEAL services adoption gaps

### 8.2.1	Adoption Solution#1: Service based Architecture of 3GPP system supporting SEAL services

#### 8.2.1.1	Solution description

This solution is to solve GAP# 5-1-2-1.

Figure 8.2.1.1-1 Service based Architecture of 3GPP system supporting SEAL services

In the figure 8.2.1.1-1, SEAL services layer represents the SEAL server(s) supporting SEAL services defined in the SEAL specification 3GPP TS 23.434 [3].

SEAL services which are part of the 3GPP system can be deployed either by the MNO or third party.

SEAL server(s) can invoke service APIs exposed by 3GPP core network functions directly over the Core network SBA when deployed by MNO’s trusted domain. SEAL server(s) can invoke service APIs exposed by 3GPP core network functions via NEF as a non-trusted AF when deployed by the third party.

SEAL server(s) can invoke service APIs exposed by the OAM system over the OAM SBA when deployed by MNO. SEAL server(s) can invoke services APIs exposed by 3GPP core network functions via EGMF as non-trusted AF, when deployed by the third party.

SEAL services API can be exposed by MNOs or third party, to the Application server (VAL server), or to the Application client(s) (VAL client(s)) via SEAL client(s).

NOTE 1:	In some serverless application software implementations, application server (VAL server) may not be implemented and deployed. In such deployments, Application client(s) (VAL client(s)) can support all the application requirements by invoking SEAL service APIs via SEAL client(s).

NOTE 2:	3GPP TS 23.222 [9] supports where the API invokers can either be on the network side (Application server) or on the UE side (Application clients (VAL clients) via SEAL clients), to use services API.

Editor's Note: It is FFS whether and how OAM interacts with SEAL in a service-based manner.

Editor's Note: It is FFS whether and how to add the interface between UE and Radio network.

* * * Next Change * * * *

### 8.2.2	Adoption Solution#2: Layered representation of 3GPP network exposure system

#### 8.2.2.1	Solution description

This solution is to solve Generic gap#3.

AT command

Uu

SEAL-UU

Network Services

(e.g., AMF, NEF, NWDAF)

O&M Services

(e.g., FCAPS, slicing)

SEAL services servers

(e.g., NRM/SEAL DD for connection over 5G/4G, NSCE for 5G Slice, Group, location)

Southbound Interface (SBI)

(Network APIs)

Northbound Interface (NBI)

(Service APIs)

APIs publish

APIs discovery

VAL server

(V2X)

VAL server

(UAS)

VAL server

(IoT)

VAL Client

(V2X, UAS, IoT, …)

SEAL Clients

Application Enablement layer services over 5G/EPS

SEAL-C APIs

UE/Device

5G Network services

Application Layer

UE Modem

APIs

APIs

APIs

APIs

APIs

APIs

CCF

AEF (Authorization checking， message routing)

APIs

APIs

Network connection layer

Service abstraction layer/value added service layer

Exposure framework

Application layer

APIs publish

Figure 8.2.2.1-1: Layered representation of SEAL service layer in 3GPP network exposure system

# *** End of Changes ***