3GPP TSG-SA WG6 Meeting #66	S6-251097

Gothenburg, Sweden 7th – 11st April 2025	(revision of S6-25xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0377 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction in Configure VAL service area identifier procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GFLS |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The figure and related text is not matched in Configure VAL service area identifier procedure. So it’s needed to correct the step 3 to make them alignment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the step 3 in Configure VAL service area identifier procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Consumers will be confused about the Configure VAL service area identifier procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.13.2, 9.4.1, 9.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.13.2	Configure VAL service area identifier procedure

Figure 9.3.13.2-1 illustrates the high level procedure of the VAL service area identifier configuration.

Figure 9.3.13.2-1: Configure VAL service area identifier procedure

1.	The VAL server sends the VAL service area configuration request to the location management server to configure a new VAL service area identifier. In the request message, the VAL server includes the information as specified in table 9.3.2.21-1.

2.	The location management server shall check if the VAL server is authorized to initiate VAL service area identifier configuration request.

3.	If the VAL server is authorized to configure the VAL service area identifiers, the location management server  checks the consistency of the provided identifiers (e.g., all VAL service area identifiers shall be unique) and configures the provided VAL service area identifiers.

4.	The location management server sends the VAL service area configuration response with the information as specified in table 9.3.2.22-1.

* * * Next Change * * * *

9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting Notify_Trigger_Location_Reporting Notify_Adaptive_Configuration | VAL server | Subscribe/Notify |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Info | VAL server |  |
|  | Update_Location_Info_Subscription | VAL server |  |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Monitoring | VAL server |  |
|  | Update_Location_Monitoring_Subscription | VAL server |  |
|  | Notify_Location_Monitoring_Events | VAL server |  |
| SS_LocationAreaMonitoring | Subscribe_Location_Area_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Area_Monitoring_Events |  |  |
|  | Update_Location_Area_Monitoring_Subscribe |  |  |
|  | Unsubscribe_Location_Area_Monitoring |  |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | VAL server | Request/Response |
|  | Obtain_VAL_Service_Area | VAL server |  |
|  | Update_VAL_Service_Area | VAL server |  |
|  | Delete_VAL_Service_Area | VAL server |  |
|  | Subscribe_VAL_Service_Area_Change_Event | SEAL server | Subscribe/Notify |
|  | Notify_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Unsubscribe_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Update_Subscription_VAL_Service_Area_Change_Event | SEAL server |  |
| SS_LocationHistoryInfoEvent | Create_Location_Tracing_Configuration | VAL server | Request/Response |
|  | Update_Location_Tracing_Configuration | VAL server |  |
|  | Delete_Location_Tracing_Configuration | VAL server |  |
|  | Obtain_history_Location_Info | VAL server |  |
| SS_SLPositioningManagement | Subscribe_SLPositioningManagement | VAL server | Subscribe/Notify |
|  | Notify_SLPositioningManagement | VAL server |  |
| SS_SRPositioningInformation | SR_Positioning_Information | VAL server | Request/Response |
| SS_ConfirmLocation | Confirm_Location | LMC | Request/Response |
|  | ConfirmLocationServiceSubscription | VAL server | Request/Response |
|  | Notify_ConfirmLocationServiceUsage (NOTE) | VAL server | Subscribe/Notify |
| NOTE: The subscribe operation is part of ConfirmLocationServiceSubscription |  |  |  |

* * * Next Change * * * *

9.5.4	On-demand location reporting procedure

Figure 9.5.4-1 illustrates the procedure for on-demand location report from the location management client-1 residing in UE-1 to the location management client-2 residing in UE-2.

Pre-condition:

-	The UE-1 and UE-2 are within PC5 communication range of each other, and aware of Layer-2 ID of each other.

-	The VAL service user in UE-1 is authorized to request location report from the UE-2.

-	The VAL service user in UE-1 requests immediate location reporting to the UE-2.

Figure 9.5.4-1: Location reporting trigger cancel

1.	Based on configurations such as periodical location information timer the location management client-1 initiates the immediately request location information from the location management client-2. The location management client-1 sends an off-network location request to the location management client-2. The message includes information elements as specified in Table 9.5.2.7-1.

2.	VAL user or VAL UE is notified and asked about the permission to share its location. VAL user can accept or deny the request

3.	The location management client-2 immediately responds to the location management client-1. If permission is received from the VAL user, the location management client-2 includes a report containing location information identified by the location management client-1 and available to the location management client-2. The message includes information elements as specified in Table 9.5.2.8-1.

4.	Upon receiving the off network location reporting trigger configuration response message, the location management client-1 sends off-network location management ack message. The message includes information elements as specified in clause 9.5.2.3

* * * End of Changes * * * *