3GPP TSG-SA WG6 Meeting #66	S6-251098

Gothenburg, Sweden 7th – 11st April 2025	(revision of S6-25xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0378 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the reference and clause numbering of location management procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The numbers of some clauses or reference are missing or not corrected.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add some reference number or correct the clause numbering. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specific will not have a good quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.7, 9.3.21.3, 9.3.21.5.2，9.3.22, 9.3.23.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.7	Location information subscription procedure

Figure 9.3.7-1 illustrates the high level procedure of location information subscription request. The same procedure can be applied for location management client and other entities that would like to subscribe to VAL user or VAL UE location information. This procedure is also used for initiating tracking a UE's location. The procedure considers target UE location information provided by its surrounding UEs, the procedure can improve location accuracy and can be used also for target UE temporarily out of 3GPP radio coverage. Location management client 1 is used as example in the figure as target UE that the VAL server is interested to know its location information.

Figure 9.3.7-1: Location information subscription request procedure

1.	The VAL server sends a location information subscription request to the location management server to subscribe location information of one or more VAL users or VAL UEs. The request may include an indication for supplementary location information, the location QoS which contains the location accuracy, response Time and QoS class as defined in clause 4.1b of TS 23.273 [50], the velocity information of target UEs, and an indication whether the statistic of target UE location is needed per e.g., temporal/spatial granularity. If the indication whether the statistic of target UE location is needed, the corresponding time/location information should also be included. In addition, the VAL server may request velocity information of the VAL UEs/Users in step 1.

2.	The location management server shall check if the VAL server is authorized to initiate the location information subscription request and may retrieve the available and proper location access type and positioning methods (e.g. as described in TS 23.273 [50] and TS 29.572 [51]) for the target VAL UE based on the received location QoS. Further, the location management server may initiate location reporting configuration optionally including the retrieved location access type and positioning methods with the location management client of the UE for immediate reporting.

3.	The location management server may optionally subscribe for UE location information from 3GPP core network for the UE. If the indication for supplementary location information is included in step 1, then UE location information is obtained from the 3GPP core network and/or the 3rd party location management server.

If the velocity information of target UE is requested in step 1, the location management server asks the location management client or 3GPP core network to report the target UE location data with the requested velocity information.

NOTE 1:	How the location management server obtains the UE location from the 3rd party location management server is out of scope of this specification.

4.	The location management server determines the UE location information of the UE as received in steps 2 and 3 and checks if it meets the location QoS requirements (if any) or not.

If the indication whether the statistic of target UE location is included in step 1, the location management server performs the following actions based on different granularity.

If the indication whether the statistic of target UE location is needed per temporal granularity is included in step 1, the location management server calculates the UE location data received in step 3 per the temporal granularity with the requested time condition.

If the indication whether the statistic of target UE location is needed per spatial granularity is included in step 1, the location management server calculates the UE location data received in step 3 per the spatial granularity with the requested location information.

5.	The location management server may optionally request the UE location from its surrounding UEs (as described in step 5 to 9). The location management server decides how to retrieve surrounding UEs which may be based on location management client 1 registered positioning method in location service registration. If the location management server decides to use UE-assisted surrounding UE retrieval, step 6 is executed; otherwise, NW-assisted surrounding UE retrieval is used as described in step 7.

6.	The location management server requests the location management client 1 with an optional surrounding UE retrieval method (e.g. ProSe, BT, WiFi) to provide its surrounding UEs and the location management client 1 responds the location management server with VAL UE/User ID of discovered UEs as described in clause 9.3.22;

NOTE 2:	For ProSe capable UE, the User Info ID defined in 3GPP TS 23.586 [57] can represent VAL user ID. For Bluetooth capable UE, the Bluetooth MAC address can represent VAL UE ID. For WiFi Direct capable UE, the WiFi MAC address can represent VAL UE ID.

7a.	The location management server derives an appropriate location area (where UE to UE communication is possible) internally using the location information of UE 1 (determined in step 4) as reference location. If the UE 1 location is last known UE location, the location management server uses UE mobility analytics service from 3GPP CN to derive UE location estimation for UE 1.

7b.	The location management server uses 3GPP CN service (NEF service of Number of UEs present in a geographical area) to obtain all UEs within the derived location area in step 6.

8.	The location management server selects a set of UEs (location management client 2..n) from UEs obtained in step 6 or step 7 based on registered location service information (e.g. positioning method) from location management client 2..n. Then location management server sends location request to location management client 2..n using On-demand location reporting procedure (as defined in clause 9.3.4) to obtain UE 1 location including an optional UE positioning method (e.g. PC5 SEAL LM, BT, WiFi) to be used to retrieve location.

NOTE 3:	The location management server can, based on local policy, select an UE without LM registration and trigger location request to the UE. If so, the request can be ignored by the UE (e.g., if SEAL LM-UU service is not supported) or the UE can reject the request (if the required positioning method is not supported).

9.	The location management server akes the location information of UE 1 reported by location management client 2..n into consideration to calibrate UE 1 location determined in step 4.

NOTE 4:	It is assumed that different location information collected over different positioning method may have differences in accuracy. How location management server processes calibration is implementation specific.

10.	The location management server replies with a location information subscription response indicating the subscription status and if immediate reporting was requested, the location information of the VAL UE(s).

NOTE 5:	The VAL server can use obtained UE location and velocity in application specific ways (e.g. traffic monitoring in V2X).

* * * Next Change * * * *

9.3.21.3	Location tracing configuration cancel procedure

Figure 9.3.21.3-1 illustrates the high level procedure of location tracing configuration cancel request which the VAL server triggers to cancel the location tracing configuration for a single UE or multiple UEs.

Figure 9.3.21.3-1: Location tracing configuration cancel procedure

1.	If the VAL server decides to stop the location tracing services for the UE(s), it sends a location tracing configuration cancel request to the location management server with the UE identities.

2.	The location management server stops getting the UE's location and deletes the stored UE location data and the location tracing configuration for the service.

3.	The location management server sends location tracing configuration cancel response to the VAL server.

* * * Next Change * * * *

9.3.21.4.2	Off-network history location result report

Figure 9.3.21.4.2-1 illustrates the high level procedure of reporting the cached history location result to the location management server.

Pre-condition:

-	The location management client is configured with the off-networking location positioning and report policy.

Figure 9.3.21.4.2-1: Off-network history location result report

1.	The Off-network location positioning trigger is met, e.g., the reference VAL UE which hosts the location management client detects it is in off-network and the target VAL UE identified by the target UE identity is also in off-network.

2.	The location management client checks whether it is within the valid period as indicated by the time duration if configured. If yes, the location management client initiates the UE-only Operation SL/Ranging as described in 3GPP TS 23.586 [57], obtains its own location or the target VAL UE location, and caches the location result with time stamp. This step may be performed periodically as configured.

3.	When the connectivity is back and it is within the valid period as indicated by the time duration, the location management client sends the Off-network history location result to the location management server.

* * * Next Change * * * *

9.3.22	Surrounding UEs retrieval procedure

Figure 9.3.22-1 illustrates the procedure of surrounding UE(s) retrieval. The location management server can use this procedure to know how many UE(s) are close to the requested UE hosting the location management client, and their identities.

Figure 9.3.22-1: Surrounding UE(s) retrieval procedure

1.	The location management server decides to send Surrounding UE retrieval request to the location management client. The request may include surrounding UE retrieval method.

2.	The location management client obtains UE(s) close to itself. The location management client considers the surrounding UE retrieval method (if received) during the obtaining procedure.

3.	The location management client sends its surrounding UE(s) to the location management server in Surrouding UE retrieval response.

* * * Next Change * * * *

9.3.23.2	Procedure of LM Server identifying if the UEs sharing the same location

Figure 9.3.23.2-1 describes the procedure that how location management server identifies the UEs that sharing the same location.

Pre-condition:

-	The VAL UE1 and VAL UE2 are associated and may belong to the same user or different users.

-	The location management client 1(in VAL UE1) and location management client 2(in VAL UE2) have reported the location capabilities (e.g. associated ID) for VAL UE1 and VAL UE2 to location management server respectively as defined in clause 9.3.15.

NOTE:	The associated ID for VAL UE1 and VAL UE2 is same.

Figure 9.3.23.2-1: Procedure of LM Server identifying the UEs that sharing the same location

1.	The location management server identifies the registered location management client1 and location management server client 2 are associated and may share the same location with each other based on the received location service registration information (e.g. same associated ID as defined in clause 9.3.15). Then it further verifies if they are sharing the same location at present.

2.	The location management server initiates the verify location sharing request to the location management server client 1 with the UE ID of location management client 2 to verify if they are sharing the same location.

NOTE:	The step 2 may occur after the location management server receives the location request to the location management client 2 from the VAL server.

3.	The location management client 1 checks if the location management client 2 is close enough and shares the same location via the following ways.

-	The location management client 1 may trigger the location reporting to the location management client 2 via some positioning capabilities (e.g., Prose) and receive off-network location report from the location management client 2 and determines that the VAL UE2 is within allowed proximity range of the VAL UE1 as specified in step 1 of clause 9.3.23.3.

-	The location management client 1 may ask the VAL user to check if VAL UE1 and VAL UE2 are in the same location. If the answer is yes, the VAL user may inform the location management client 1 that the VAL UE1 and VAL UE2 are in the same location area.

4.	The location management server client 1 sends the verify location sharing response to the location management server with the confirmation of sharing same location with location management client 2, and the validity timer is also included to indicate the valid duration for the confirmation.

Before the validity timer expired, if the location management client 1 discovers the location management client 2 is far away from itself and they are not able to share the same location, the location management client 1 will inform the location management server they are no long sharing the same location as specified in step 2(disable) of clause 9.3.23.3.

If the response is negative (e.g. the location management client 1 confirms the location management client 2 is not close with location management client 1 or the location management client 1 can't confirm whether both of UEs are closing enough), the location management server will not reuse the stored UE1's location for UE2.

* * * End of Changes * * * *