3GPP TSG-SA WG6 Meeting #66	S6-251099

Gothenburg, Sweden 7th – 11st April 2025	(revision of S6-25xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0379 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the information flow of location reuse procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The description for UE-1 and UE-2 in information flow 9.3.2.59 is not clear and can’t be understood what they mean. What’s more, the IE description for the identity should be more general instead of UE-1/UE-2. So it’s needed to add the reference and more texts to keep the information flow texts aligning with the related procedures more clearly. For the information flow of Verify location sharing request, the “Identity of VAL UE who sharing the location” should be mandatory based on the related procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Keep similar information flow and procedures alignment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The information flow for “location reuse request” will not align with the related procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.57, 9.3.2.59, 9.3.23.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.2.57	Verify location sharing request

Table 9.3.2.57-1 describes the information flow from the location management server to the location management client to verify if it is sharing the same location with other UEs.

Table 9.3.2.57-1: Verify location sharing request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requested VAL users or VAL UEs. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| Identity of VAL UE who sharing the location | M | Indicates the list of identities of VAL UEs who share the same location with the requested VAL UEs. |

* * * Next Change * * * *

9.3.2.59	Location reuse request

Table 9.3.2.59-1 describes the information flow from the location management client to the location management server for enabling or disabling location reuse.

Table 9.3.2.59-1: Location reuse request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requestor VAL UEs or VAL users (e.g.UE-1 as specified in clause 9.3.23.3). |
| VAL service ID | O | Identity of the VAL service for which the location service is registered. |
| Identity of VAL UE who reusing the location | M | Identity of the VAL UEs (e.g. UE-2 as specified in clause 9.3.23.3) who will reuse the location of requestor VAL UEs (e.g.UE-1 as specified in clause 9.3.23.3). |
| Location reuse | M | Indicates whether to enable or disable reuse of location |
| Current location report | O | Location report of the requestor VAL UEs or VAL users(as specified in Table 9.3.2.2-1) |

* * * Next Change * * * *

9.3.23.3	Location reuse request

Figure 9.3.23.3-1 illustrates the procedure of client-triggered enable location reuse request.

Pre-condition:

1.	All UEs in the association are registered to share the location.

2.	UE has configured other UEs in the association to received offline location reports as specified in clause 9.5.3.

3.	The VAL UE1 and VAL UE2 are associated and may belong to the same user or different users.

4.	The LM Client 1(in VAL UE1) and LM Client 2(in VAL UE2) have reported the location capabilities (e.g. associated ID) for VAL UE1 and VAL UE2 to LM server respectively as defined in clause 9.3.15.

Figure 9.3.23.3-1: Location reuse procedure

1.	The SEAL LMC-1 (in UE-1) receives off-network location report from the SEAL LMC-2 (in UE-2). Based on location report, if UE-2 is within certain range of UE-1, the SEAL LMC-1 determines that the UE-2 is within allowed proximity range of the UE-1 (i.e. UE-1 and UE-2 are close enough) and so UE-1's location can be used by SEAL LMS instead of UE-2's location.

Based on location report, if UE-2 is out of certain range of UE-1, the SEAL LMC-1 determines that the UE-2 is outside allowed proximity range of the UE-1 (i.e. UE-1 and UE-2 are not close enough) and so UE-1's location can not be used by SEAL LMS instead of UE-2's location.

2.	The SEAL LMC-1 (in UE-1) sends location reuse request message to the SEAL LMS to enable reuse of location of UE-1 for UE-2. If both UE1 and UE2 are close enough (i.e. within allowed proximity range), the request includes indication to enable reuse of location of UE-1 for UE-2. If both UE1 and UE2 are not close enough (i.e. outside allowed proximity range), the request includes indication to disable reuse of location of UE-1 for UE-2. The request message also includes VAL user’s identity, identity of VAL UE-1, identity of UE-2 and latest location report of UE-1 as specificed in clause 9.3.2.59.

3.	The SEAL LMS authorizes the request. If authorized, the SEAL LMS identifies that if SEAL LMC-1 and SEAL LMC-2 are associated (e.g. same associated ID) or not based on the received registration information of SEAL LMC-1 and SEAL LMC-2, and if reusing of location is allowed or not.

4.	If both UE-1 and UE-2 are assocaited and reusing of location is allowed (as determined in step-3), then the SEAL LMS stores the location of UE-1. If the request includes indication to enable reuse of location of UE-1, the SEAL LMS enables reuse of location of UE-1 for UE-2. If the request includes indication to disable reuse of location of UE-1, the SEAL LMS disables reuse of location of UE-1 for UE-2. The SEAL LMS sends response back to the SEAL LMC-1.

When location reuse is enabled, if location request for UE-2 is received from VAL server, the SEAL LMS reuses and provides the location of UE-1 (instead of location of UE-2) to the VAL server. When location reuse is disable, if location request for UE-2 is received from VAL server, the SEAL LMS provides the location of UE-2 only (instead of reusing location of UE-1) to the VAL server.

* * * End of Changes * * * *