3GPP TSG-SA WG6 Meeting #66	S6-251100

Gothenburg, Sweden 7th – 11st April 2025	(revision of S6-25xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0380 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the procedure of verify UE locations and APIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - There are some references missing in procedure of confirm location verification. And some response also lacks of details. - The SEAL APIs for location management only list the APIs between SEAL LM server and VAL server and not include the ones between LM server and LM client. So the APIs between LMS and LMC should be removed. - LMS can’t communicate with LMF directly but via NEF. So the related location details between NEF and LMF is not necessary to add. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the procedure of 9.3.26.3 and related APIs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It’s not clear how to realize the “Confirm location verification” and the specific will not have a good quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.26.3, 9.4.1, 9.4.12.1, 9.4.12.2, 9.4.12.3, 9.4.12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.26.3	Confirm location verification

The location management client (LMC)/application in the UE gets its location from non-3GPP positioning (e.g. GNSS). The application in the UE wants to check if this is really the correct location.

Pre-conditions:

1.	The LMC is authorized to discover the related LMS;

2.	LMS is authorized to use Nnef Event Exposure API, based on SLA with NEF/5GS of MNO;

3.	It is assumed there is user consent given in the UE for exposing location information from UE to LMS.

Figure 9.3.26.3-1: Procedure of confirm location verification

1.	The UE sends Confirm Location request from the LMC towards LMS. It contains the location measured from the UE (non-3GPP location information, e.g. GNSS). It contains latitude and longitude, accuracy, timestamp, carrier name, cell id (known to the UE from the network signalling), application identifier.

2.	 The LMS shall check if there is subscription (based on VAL subscription in clause 9.3.26.2) for confirm location service with the specific application identifier received in step 1. If subscription is present, step 3 is performed. If subscription is not present, the LMS replies back to LMC with confirm location status unknown (specified in step 4).

3.	LMS requests UE location from 5GC via NEF (LMS send determine Location operation via NEF (N33 interface)). It also requests accuracy to be a parameter present in the response. LMS can also perform a location information request to GMLC directly or via NEF (as defined in clause 6.1.2 of 3GPP TS 23.273 [5]), acting as AF. LMS can also perform a location information request to 3rd party location servers via LM-3P interface as defined in clause 9.2.2. LMS can also use the T8 interface to obtain UE location. Or the LMS can have already the UE location based on the methods described in clause 9.3.4 and 9.3.5. The received response contains the UE location from the network. LMS can use event exposure using NEF according to table 4.15.3.1-1 of 3GPP TS 23.502 [11] to obtain user location information from AMF. Finally, the LMS compares the location provided from the network with the location provided from the LMC. And responses to the LMC with confirmed location status (e.g. unknown, verified, mismatch in the same country, mismatch in other country, roaming country mismatch, error unauthorized, etc.) as specificed in clause 9.3.2.76.

4.	LMS sends confirm location report to LMC with confirmed location status.

5.	The LMS sends notification towards the VAL server with the UE identifier and application using the service.

* * * Next Change * * * *

9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting Notify_Trigger_Location_Reporting Notify_Adaptive_Configuration | VAL server | Subscribe/Notify |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Info | VAL server |  |
|  | Update_Location_Info_Subscription | VAL server |  |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Monitoring | VAL server |  |
|  | Update_Location_Monitoring_Subscription | VAL server |  |
|  | Notify_Location_Monitoring_Events | VAL server |  |
|  | Subscribe_Location_Area_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Area_Monitoring_Events |  |  |
|  | Update_Location_Area_Monitoring_Subscribe |  |  |
|  | Unsubscribe_Location_Area_Monitoring |  |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | VAL server | Request/Response |
|  | Obtain_VAL_Service_Area | VAL server |  |
|  | Update_VAL_Service_Area | VAL server |  |
|  | Delete_VAL_Service_Area | VAL server |  |
|  | Subscribe_VAL_Service_Area_Change_Event | SEAL server | Subscribe/Notify |
|  | Notify_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Unsubscribe_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Update_Subscription_VAL_Service_Area_Change_Event | SEAL server |  |
| SS_LocationHistoryInfoEvent | Create_Location_Tracing_Configuration | VAL server | Request/Response |
|  | Update_Location_Tracing_Configuration | VAL server |  |
|  | Delete_Location_Tracing_Configuration | VAL server |  |
|  | Obtain_history_Location_Info | VAL server |  |
| SS_SLPositioningManagement | Subscribe_SLPositioningManagement | VAL server | Subscribe/Notify |
|  | Notify_SLPositioningManagement | VAL server |  |
| SS_SRPositioningInformation | SR_Positioning_Information | VAL server | Request/Response |
| SS_ConfirmLocation | ConfirmLocationServiceSubscription | VAL server | Subscribe/Notify |
|  | Notify_ConfirmLocationService | VAL server |  |

* * * Next Change * * * *

9.4.12.1	General

API description: This API enables the VAL server to subscribe for Confirm Location Service.

9.4.12.2	Void

9.4.12.3	Confirm Location Service Subscription

API operation name: ConfirmLocationServiceSubscription

Description: Subscription for the Confirm Location Service.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.73

Outputs: See subclause 9.3.2.74

See subclause 9.3.26.2 for the details of usage of this API operation.

* * * Next Change * * * *

9.4.12.4	Notify Confirm Location Service

API operation name: Notify_ConfirmLocationService

Description: Notification about the usage of the Confirm Location Service.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.77

Outputs: None

See subclause 9.3.26.3 for the details of usage of this API operation.

* * * End of Changes * * * *