3GPP TSG-SA WG6 Meeting #66	S6-251117

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-250xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0459 | rev | - | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for combining multiple ad hoc group call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph6-MC |  |  |  |  |  | Date: |  |  | 2025-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The clause introduces a new procedure to combine multiple ongoing ad hoc group calls upon the request of an authorised MCPTT user. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new procedure is included to combine multiple ad hoc group calls. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 requirements are not met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.3.1.7 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 10.19.3.1.7	Combining multiple ad hoc group calls

This clause describes the procedure to combine multiple ad hoc group calls. Multiple ad hoc group calls may be combined upon the request of an authorised MC service user.

NOTE:	The upper limit of the number of ad hoc group calls to be combined is up to implementation.

The ad hoc group MCPTT calls to be combined may be initiated differently, i.e., the participants are either selected by the MCPTT server based on criteria or provided by the initiated MCPTT user. The resulting, i.e., the combined ad hoc group MCPTT call is based on the combined criteria and/or list of participant lists (provided by the initiated MCPTT user) of each individual ad hoc group MCPTT call.

Figure 10.19.3.1.7-1 describes the procedure for combining multiple ad hoc group calls. The procedure is applicable to more than two ongoing ad hoc group calls.

The procedure is applicable to ad hoc group calls irrespective how they are set up, i.e., whether the initiated (authorized) MC service user selects the list of participants as in clause 10.19.3.1.1, or the MC service server selects the list of participants based on criteria as in clause 10.19.3.1.3.

Pre-conditions:

- The MCPTT user 1 (authorised user) is participating in an ongoing ad hoc group call #1 with MCPTT user 2, …, MCPTT user n.

- The MCPTT user 1 is participating in an ongoing ad hoc group call #2 with MCPTT user n+1, …, MCPTT user m.

Figure 10.19.3.1.7-1: Combining multiple ad hoc group call procedure

1.	The MCPTT user at MCPTT client 1 wishes to combine multiple going ad hoc group MCPTT calls. MCPTT client 1 sends an ad hoc group call combine request towards the MCPTT server. The request includes among others the identities of the ad hoc group calls to be combined.

2.	The MCPTT server checks whether the MCPTT client 1 is authorised to combine multiple ongoing ad hoc group calls. If the MCPTT client 1 is authorised, the MCPTT server proceeds with the next step.

3.	The MCPTT server combines the different lists of participants into one and/or the different set of criteria into one to create the combined ad hoc group call. Furthermore, it assigns a new ad hoc group associated with the combined list of participants and/or the combined criteria. The aspects and procedures related to combine the criteria is left for implementation. The MCPTT server continuously checks whether other MCPTT clients meet the combined criteria, or whether the existing participating MCPTT clients no longer meet the combined criteria.

4.	The MCPTT server sends the MCPTT client 1 an ad hoc group call combine response including the identity of the ad hoc group to be associated with the combined ad hoc groups.

5.	The MCPTT client 1 sends an ad hoc group call request towards the MCPTT server. The request includes the identity of the ad hoc group associated with the combined ad hoc groups.

6.	The MCPTT server checks whether MCPTT client 1 is authorised to send ad hoc group call request. If the authorisation is successful, the MCPTT server proceeds with the next step.

7.	The MCPTT server sends ad hoc group request return to the MCPTT client 1

8a.-8b.	The MCPTT server sends ad hoc group call request to the MCPTT clients based on determining the MCPTT users to participate in the combined ad hoc group call, which is done in step 3.

9a.-9b.	The target MCPTT clients notify the corresponding MCPTT users about the ad hoc group call.

10a.10b.	The MCPTT clients send the MCPTT server an ad hoc group call response.

11.	The MCPTT server sends ad hoc group call response to MCPTT client 1.

12.	The MCPTT server may send ad hoc group call notify to MCPTT client 1.

13.	Media plane and follow control associated with the combined ad hoc group call is established.

14.	MCPTT client 1 initiates ad hoc group call release procedure according to clause 10.19.13.1.2a.

# * * * * End of changes * * * *