3GPP TSG-SA WG6 Meeting #66	S6-251121

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-250xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0148 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The support to enable 5G ProSe multi-hop UE-to-UE relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MultiHop-MC |  |  |  |  |  | Date: |  |  | 2025-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR introduces the support of 5G ProSe multi-hop UE-to-UE relay in alignment with 3GPP TS 23.304. The CR is aligned with the concepts defined in TS 23.304, 5G ProSe single hop is not explicitly mentioned in the relevant architecture. Please refer to clause 4.2.8 with the two figures below: figure 4.2.8-1 and figure 4.2.8A-1: Figure 4.2.8-1: Reference architecture for 5G ProSe UE-to-UE Relay Figure 4.2.8A-1: Reference architecture for Layer-3 5G ProSe Multi-hop UE-to-UE Relay 5G ProSe single hop is equivalent to the concept of having a 5G ProSe UE2UE between the two end UEs, which is what is already specified in Rel-18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Adding a new clause describing the main parameters to enable 5G ProSe Layer-3 multi-hop UE-to-UE relay. 2. Updating the general clause to include the possibility of utilizing several 5G ProSe UE2UE relay UEs between the two end UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe Layer-3 multi-hop UE-to-UE relay is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10.1, 4.10.2 and 4.10.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 4.10		Use of 5G ProSe UE-to-UE relay

### 4.10.1	General

The MC service shall support the capabilities for 5G ProSe UE-to-UE  relay. For this matter, 5G ProSe Layer-2 and 5G ProSe Layer-3 UE-to-UE relaying techniques can be utilized, as described in 3GPP TS 23.304 [17].

A 5G ProSe UE-to-UE relay supporting MC service UE provides means of connectivity and relaying of MC traffic from a MC service UE to another MC service UE(s) via a MC UE Relay. For this matter, the 5G ProSe UE-to-UE Relay Discovery service allows the MC service remote UE to discover a potential UE-to-UE relay supporting MC service in its proximity as described in 3GPP TS 23.304 [17]. Upon its discovery, the 5G ProSe Direct UE-to-UE Relay Communication functionality is utilized to achieve communication to provide the MC service between MC UEs, relaying MC traffic via the UE-to-UE relay UE over the NR PC5 reference point.

5G ProSe UE-to-UE relay service is either using one 5G ProSe UE-to-UE relay MC service UE (as described in clause 4.10.2), or using multiple 5G ProSe UE-to-UE relay MC service UEs (as described in clause 4.10.x).

### 4.10.2	5G ProSe UE-to-UE relay service requirements

In order to enable 5G ProSe UE-to-UE relaying capabilities – whether based on Layer-3 or Layer-2 UE-to-UE relaying techniques, the MC system provides the appropriate parameters and configurations to the MC service UE(s).

As defined in 3GPP TS 23.304 [17], among these parameters are: Relay Service Code(s) (RSCs) which can be associated to a certain MC service group, User Info ID, ProSe Layer-2 Group ID and ProSe Group IP multicast address. Moreover, the MC service group ID is resolved to the ProSe Layer-2 Group ID and ProSe Group IP multicast address, which are utilized within the 5G ProSe Relay Discovery and 5G ProSe Direct Communication procedures, as described in 3GPP TS 23.304 [17]. Furthermore, the RSCs are utilized to restrict the necessary UE-to-UE relay service and related procedures within members of a certain MC service group.

# * * * * Second change * * * *

### 4.10.x	5G ProSe multi-hop UE-to-UE relay service requirements

In order to enable 5G ProSe Layer-3 multi-hop UE-to-UE relaying capabilities, the MC system provides the appropriate parameters and configurations to the MC service UE(s). As defined in 3GPP TS 23.304 [17], among these parameters are: RSC(s) which can be associated to a certain MC service group, User Info ID, ProSe Layer-2 Group ID and ProSe Group IP multicast address, RSC(s), UE-to-UE relay layer indicator (indicates that the associated RSC is supporting 5G ProSe Layer-3 UE-to-UE relay service), multi-hop indicator per RSC, and maximum number of hops per RSC.

NOTE 1:	The RSC used for multi-hop service is different from the RSC used in clause 4.10.2.

# * * * * End of changes * * * *