3GPP TSG-SA6 Meeting #66	S6-251124

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.283 | CR | 0094 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction in 10.17.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Hytera Communications Corp. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The 'Next Change' is intended to assist with document revisions and should not appear in the main text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete “Next Change”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong description remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.17.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

10.17.2	Information flows for ad hoc group call

10.17.2.1	General

The following clauses define information flows for ad hoc group calls on the IWF-1 interface. Ad hoc group call related information flows on reference points other than IWF-1 are defined in 3GPP TS 23.379 [7].

10.17.2.2	IWF ad hoc group call request (MCPTT server – IWF and IWF - MCPTT server)

Table 10.17.2.2-1 describes the information flow IWF ad hoc group call request from the MCPTT server to the IWF and from the IWF to the MCPTT server.

Table 10.17.2.2-1 IWF Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID to be associated with the ad hoc group communication |
| Preconfigured MCPTT group ID | O | Indicates pre-configured group configuration to be used for the ad hoc group call |
| MCPTT ID | O | MCPTT ID of the participant being invited for the ad hoc group call |
| SDP offer | M | Offered Media parameters |
| Implicit floor request (see NOTE 1) | O | When originating client requests the floor, this element shall be included |
| Broadcast indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MCPTT imminent peril ad hoc group call |
| Emergency Indicator (see NOTE 2) | O | Indicates that the ad hoc group call request is an MCPTT emergency ad hoc group call |
| Location information | O | Location of the calling party. |
| Criteria for determining the participants (see NOTE 3) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCPTT server for determining the participants e.g., it can be a location based criteria to invite participants in a particular area |
| Additional application specific data (see NOTE 4) | O | Some LMR systems use additional information at the application layer. |
| Requested priority | O | Application priority level requested for this group call |
| NOTE 1: This element is included only when the originating client requests the floor. NOTE 2: If used, only one of these information elements is present. NOTE 3: This element can be used by the IWF together with the LMR system to determine the users to be invited in the LMR system. NOTE 4: This element can be present if the LMR system uses it (like GSM-R). GSM-R uses for example UUI as defined in ETSI TS 103 389 [10] and ETSI TS 102 610 [11]. |  |  |

10.17.2.3	IWF ad hoc group call response (IWF - MCPTT server and MCPTT server - IWF)

Table 10.17.2.3-1 describes the information flow IWF ad hoc group call request from the IWF to the MCPTT server and from the MCPTT server to the IWF.

Table 10.17.2.3-1 IWF Ad hoc group call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID associated with the ad hoc group call |
| SDP answer | O | Media parameters selected and present if the Result is success. |
| Result | M | Result of the group call request (success or failure) |
| Additional application specific data (see NOTE) | O | Some LMR systems use additional information at the application layer. |
| NOTE: This element can be present if the LMR system uses it (like GSM-R). GSM-R uses for example UUI as defined in ETSI TS 103 389 [10] and ETSI TS 102 610 [11]. |  |  |

10.17.2.4	IWF ad hoc group call release request (MCPTT server – IWF and IWF - MCPTT server)

Table 10.17.2.4-1 describes the information flow IWF ad hoc group call release request from the MCPTT server to the IWF from the IWF to the MCPTT server.

Table 10.17.2.4-1 IWF ad hoc group call release request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the user authorized to release the ad hoc group call |
| Functional alias | O | The functional alias of the authorized user requesting to release the ad hoc group call |
| MCPTT ad hoc group ID | M | The MCPTT group ID of the ad hoc group call on which call is released |
| Additional application specific data (see NOTE) | O | Some LMR systems use additional information at the application layer. |
| NOTE: This element can be present if the LMR system uses it (like GSM-R). GSM-R uses for example UUI as defined in ETSI TS 103 389 [10] and ETSI TS 102 610 [11]. |  |  |

10.17.2.5	IWF ad hoc group call release response (IWF - MCPTT server)

Table 10.17.2.5-1 describes the information flow IWF ad hoc group call release response from the IWF to the MCPTT server.

Table 10.17.2.5-1 IWF ad hoc group call release response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the ad hoc group call participant |
| Functional alias | O | The functional alias of the ad hoc group call participant |
| MCPTT ad hoc group ID | M | The MCPTT group ID of the ad hoc group call that is released |
| Additional application specific data (see NOTE) | O | Some LMR systems use additional information at the application layer. |
| NOTE: This element can be present if the LMR system uses it (like GSM-R). GSM-R uses for example UUI as defined in ETSI TS 103 389 [10] and ETSI TS 102 610 [11]. |  |  |

10.17.3	Procedures

10.17.3.1	Ad hoc group call initiated by an MCPTT user with MCPTT server determining the participant list

In this procedure, an MCPTT user is initiating an ad hoc group call based on criteria involving both MCPTT users and LMR users.

NOTE 1:	How the LMR users are determined and invited to the MCPTT ad hoc group call is outside the scope of the present document.

Pre-conditions:

1.	The MCPTT user at MCPTT client 1 is authorized to initiate ad hoc group call.

2.	The MCPTT user at MCPTT client 1 wants to invite MCPTT and LMR users who are satisfying certain criteria for the ad hoc group call.

3.	The mapping relationship of group and user identities between the MCPTT system and the LMR system has been configured at the IWF.

NOTE 2:	For all the signalling messages passing through the IWF between the MCPTT system and the LMR system, the IWF performs the identity conversion and protocol translation.

Figure 10.17.3.1-1: Ad hoc group call setup initiated by an MCPTT user in the MCPTT system

The procedure and information flows as defined in 3GPP TS 23.379 [7] clause 10.19.3.1.1 is applied with the following differences:

1-7.	Same as clause 10.19.3.1.3 in 3GPP TS 23.379 [7].

8.	The MCPTT server sends the IWF ad hoc group call request to the IWF.

NOTE 3:	The MCPTT server can send this message any time after step 4, in particular it does not have to wait for the Ad hoc group call responses in steps 7a and 7b.

9.	The IWF returns with an IWF ad hoc group call response to the MCPTT server.

NOTE 4:	IWF can handle the IWF ad hoc group call as a normal group call towards the LMR user in the LMR system.

10.	Same as step 10 in clause 10.19.3.1.3 in 3GPP TS 23.379 [7].

11.	The MCPTT server may notify the initiating MCPTT user of all MCPTT users who acknowledged the ad hoc group call request and joined the ad hoc group call. This notification may be sent to the initiating MCPTT user by the MCPTT server more than once during the call when MCPTT users join or leave the MCPTT ad hoc group call. The authorized users (not shown in figure), who are configured to receive the participants information of ad hoc group call, are notified to receive the MCPTT IDs of the MCPTT users who acknowledged the ad hoc group call request and joined the ad hoc group call, when the MCPTT users joins late or leave the MCPTT ad hoc group call.

NOTE 5:	For LMR users, notification of acknowledging the ad hoc group call is not applicable.

12.	MCPTT client 1, MCPTT client 2, and the LMR users establish media plane and floor control resources.

# * * * * End of changes * * * *