3GPP TSG-SA WG6 Meeting #66	S6-251125

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251xxx)

Source:	Samsung

Title:	Pseudo-CR on SEAL clients deployment options

Spec:	3GPP TR 23.700-35 V0.3.0

Agenda item:	9.3

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

This pCR provides proposal for addressing the gap :

Gap#5-6-2-1: Study how to cover SEAL client in the deployment mode.

2. Reason for Change

TS 23.434 covers the deployment options of SEAL service but the aspects of the SEAL clients are missing. This pCR proposes to capture the SEAL client deployment options on the UE side which can be captured as part of informative annex in TS 23.434.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 V0.3.0.

* * * First Change * * * *

### 8.2.X	Adoption Solution # X: SEAL Client(s) deployment options in UE ecosystem

#### 8.2.X.1	Solution description

This solution addresses the gap specified in Gap#5-6-2-1 which is to “Study how to cover SEAL client in the deployment mode”.

##### 8.2.X.1.1	General description

3GPP TS 23.434 [3] captures the deployment options for SEAL server(s) but the deployment aspects of SEAL Client(s) which plays a major role in the SEAL ecosystem is missing. This solution captures the different deployment options for SEAL client(s) in the UE ecosystem. Various organizations can play the role of SEAL client providers. SEAL clients and SEAL servers can be provided by the same SEAL provider organization or different organization. SEAL clients can be deployed at different layers of the UE and it depends on who is the SEAL client provider and what is their role in the device ecosystem. Depending upon the device or UE ecosystem the deployment option may vary for different UEs. Some UEs may support all the deployment options described here and some UEs may support subset of these options.

NOTE :	In some cases, the OEM and OS provider can be part of the same organization.

##### 8.2.X.1.2	Option 1: Deployment of SEAL Client(s) in the Operating System(OS) Provider domain of the UE

Figure 8.x.1.2-1 illustrates the deployment of the SEAL client(s) in the OS provider domain of the UE. In this case, SEAL client(s) are deployed at the services framework layer which is provided by the OS vendor as part of the operating system environment. Services framework layer provides the tools and components for building applications at the application layer. SEAL client(s) deployed at this layer will have more priveleges to access the functionalities available at the underlying layers like network, hardware etc.

Figure 8.x.1.2-1: SEAL client(s) deployed in the OS provider domain of the UE

##### 8.2.X.1.3	Option 2: Deployment of SEAL Client(s) in OEM domain

Figure 8.x.1.3-1 illustrates the deployment of the SEAL client(s) in the OEM domain. In this case, it is assumed that the UE vendor and OS vendor are from different organization. UE vendor has the option to customize the service framework provided as part of the OS. UE vendor can choose to implement and deploy the SEAL client(s) as part of the customized framework.

Figure 8.2.x.1.3-1: SEAL client(s) deployed in UE vendor domain

##### 8.2.X.1.4	Option 3: Deployment of SEAL Client(s) in the application layer domain

Figure 8.x.1.4-1 deployment of the SEAL client(s) in the application layer of the UE. SEAL client(s) at this layer can be deployed by the 3rd party application providers as downloadable applications or by the OEM as pre-installed applications.

Figure 8.2.x.1.3-1: SEAL client(s) deployed in UE vendor domain

#### 8.2.X.2	Evaluation

* * * End Change * * * *