3GPP TSG-SA WG6 Meeting #66	S6-251153

Gothenburg, Sweden, 7th – 11th April 2025	(revision of S6-25xxxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on new solution to KI on new NRM API

Spec:	3GPP TR 23.700-35 v0.2.0

Agenda item:	9.3

Document for:	Approval

Contact:	gecuili@huawei.com

- Introduction

This pCR is proposed to a new solution to the gaps identified in clause 4.1.

2. Reason for Change

As described in the corresponding discussion paper in “Service API principle” and “New API service design guideline”.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR  23.700-35 v0.2.0.

* * * First Change * * * *

4	SEAL services technical gap analysis

4.1	Existing SEAL service#1: NRM service

4.1.1	Analysis

XR is an emerging mobile media service expected to contribute more and more traffics to 5G network. 3GPP specifies several new capabilities to support this feature, including the enhancement of PCF/N33 APIs towards the AF/AS. There are some gaps from the easy-to-use perspective.

(1)	Unnecessary parameters for XR, which is against minimum parameters principle

Currently the PCF/N33 APIs are common to all the verticals and its input parameters for the PCF/N33 APIs are increasing more and more and become excessive complicated. From the XR service point of view, some parameters introduced by URLLC is not required. Those non-XR related parameters, may cause confusion and mis-use risks for XR application developers.

(2) API parameters is not application developers understandable. XR specific input parameters covers several in the PCF/N33 API e.g., AFSessionWithQoS are present for various purposes (or features as defined in 3GPP TS 29.522).

The following table show the 5G added features to AsSessionWithQoS.  All the yellow highlighted features can be used by eXtended Reality (XR) and interactive media services, but not been described. It is difficult for XR application developers to link them with certain XR related service.

| AsSessionWithQoS | - The following 5G-only features defined in clause 5.14.4 of 3GPP TS 29.122 [4] may be supported only by the NEF: "EthAsSessionQoS_5G", "QoSMonitoring_5G", "PacketDelayFailureReport", "MacAddressRange_5G", "AlternativeQoS_5G", "TSC_5G", "DisableUENotification_5G", "ExposureToEAS", "AltQosWithIndParams_5G", "EnEthAsSessionQoS_5G", "enNB_5G", "AltQoSProfilesSupportReport", "ExtQoS_5G", "EnTSCAC", "L4S", "MultiMedia", "PowerSaving", "EnQoSMon", "PDUSetHandling", "RTLatency", "ToSTC_5G", "QoSTiming_5G", "ListUE_5G" and "GMEC". and "QoSMonCapRepo". - The "LOSS_OF_BEARER", "RECOVERY_OF_BEARER" and "RELEASE_OF_BEARER" events do not apply for 5G. |
| --- | --- |

NRM are defined to ease the usage of 5G network resource services, but there are some issues to well support the XR service:

-	The existing input parameters of the SS_NetworkResourceAdaptation API are not suffcient to support the latest network XR capabilities to be used by the XR service, e.g., lack of the support of PDU set handling.

-	The issue of unnecessary parameters for XR also exists, i.e., SS_NetworkResourceAdaptation API cover all the verticals XR, TSN, TSC and so on, which make it is not friendly for the XR service to identify which parameters to be used.

The above issues are caused due to the lack of application requirements - oriented and friendly API design principle including:

-	The parameters and granularity of service shall be defined in application developer’s understandable way.

-	Minimum parameters for an API to support certain service.

Similarly, for other applications e.g., multi-modal communications, cyber-physical control applications, V2X, rail communications, AV production service, medical or gaming applications may have the same issue to use the existing NEF API and NRM API to request the correct network resource and handling for their traffic.

So how to design NRM APIs complied with above principle needs to be studied.

4.1.2	gaps

-Gap 4-1-2-1: How to enhance NRM services API for various services/applications to ease the usage of 5G services by the application developer.

* * * Next Change * * * *

8.a	Solution #x: NRM APIs for network transport resource and handlings

8.a.1	Solution description

This solution proposes to address the gaps identified in clause 4.1 regarding the support various application requirements at NRM.

It is proposed to introduce a new set of NRM APIs to support the various applications per their requirements to the network resource and handling based on the following principles:

- -	The new NRM APIs are designed per application layer requirements as described in clause 8.4.A single service API serves a single service;

- The input parameters of each service API are minimum and necessary to achieve the desired benefit corresponding to the application requirement(s);

- The name of the service, service APIs, input parameters are familiar to the XR application community.

The list of new NRM API are as follows:

-	Network transport with high data rate and low latency API. This API enables the VAL server to request the network transport resource and handlings high data rate and low latency. This may be used by AR/VR applications.

-	Network transport with low latency API. This API enables the VAL server to request network transport resource and handlings with low latency. Optionally, the loose requirements on bit rate is applied. This may be used by multi-modal communications.

-	Network transport with low latency and high reliability API. This API enables the VAL server to request network transport resource and handlings with low latency and high reliability. This may be used by cyber-physical control applications, V2X, rail communications.

-	Network transport with low latency & bandwidth & high reliability. This API enables the VAL server to request network transport resource and handlings with low latency, high bandwidth and high reliability. This may be used by AV production service, medical or gaming applications.

NOTE:	More service APIs can be added is FFS.

8.a.1.1	Procedures

The procedure and information flows to be added is FFS.

8.a.1.3	NRM service and APIs

The following new NRM service and API listed in table 8.a.1.3 are introduced:

Table 8.a.1.3-1: List of new NRM APIs for network resource management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| Network transport with high data rate and low latency API | _Create | AS | Request/response |
|  | _Update | AS | Request/response |
|  | _Delete | AS | Request/response |
| Network transport with low latency API | _Create | AS | Request/response |
|  | _Update | AS | Request/response |
|  | _Delete | AS | Request/response |
| Network transport with low latency and high reliability API | _Create | AS | Request/response |
|  | _Update | AS | Request/response |
|  | _Delete | AS | Request/response |
| Network transport with low latency & bandwidth & high reliability | _Create | AS | Request/response |
|  | _Update | AS | Request/response |
|  | _Delete | AS | Request/response |

8.a.2	Impacts on architecture, services, entities

This solution has no architecture impacts.

This solution creates new NRM service and APIs at NRM.

8.a.3	Solution evaluation