3GPP TSG-SA WG6 Meeting #66	S6-251156

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0029 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial anchor group management operations - Group creation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-APP |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Sometimes several spatial anchors at a specific location illustrates services which are of similar characteristics and in these cases they can be better managed as groups. Managing them as groups offers more flexibility to the spatial anchor creators and users. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the procedure for creation of spatial anchor groups. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Functionality of managing spatial anchors as groups will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections : 8.3.x, 8.3.x.1, 8.3.x.2, 8.3.x.2.1, 8.3.x.3, 8.3.x.3.1, 8.3.x.3.2 |  |  |  |  |  |  |  |  |
| , |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.3.x	Spatial anchor group management

#### 8.3.x.1	General

Support of Spatial anchor group management by the SAn server enables a SAn client or VAL server to group the list of spatial anchors of similar nature and manage them effeciently.

#### 8.3.x.2	Procedures

##### 8.3.x.2.1	Spatial anchor group creation

Figure 8.3.x.2.1-1 illustrates the spatial anchor group creation procedure through which the requestor SAn client or the VAL server can request the SAn server to create a spatial anchor group associating or linking a list of spatial anchors with the group. SAn server creates and stores the spatial anchor group and assigns a unique identifier (Spatial anchor group ID) to enable the requestor to perform spatial anchor group related operations

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

Figure 8.3.x.2.1-1: Spatial anchor group creation

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor group create request to the SAn server. The request includes a requestor identifier, security credentials, list of spatial anchors identified by the spatial anchor IDs to be associated or linked as part of the spatial anchor group, human readable description about the spatial anchor group and service area where the spatial anchor group is accessible as described in clause 8.3.x.3.1.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the creation of spatial anchor group. If the requestor is authorized, the SAn server creates the spatial anchor group and assigns a unique identifier to it and stores the spatial anchor group related information provided in the step 1. The spatial anchor group related information contains spatial anchor group ID, list of spatial anchor IDs associated with the group, service area etc.

3.	The SAn server sends a spatial anchor group create response as described in clause 8.3.x.3.2 to the requestor. If the SAn server created spatial anchor group, the response includes an indication of success and spatial anchor group identifier of newly created spatial anchor group and list of spatial anchor IDs associated with the created group. Otherwise, the response includes an indication of failure and can include a reason for failure.

* * * Next Change * * * *

#### 8.3.x.3	Information flows

##### 8.3.x.3.1	Spatial anchor group create request

Table 8.3.x.3.1-1 describes information elements for the spatial anchor group create request from the SAn client or VAL server to the SAn server.

Table 8.3.x.3.1-1: Spatial anchor group create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| Group description | O | Human readable description about the spatial anchor group (e.g. List of restaurants) |
| Service area | O | Service area where the spatial anchor group is visible or accessible. |
| VAL service information | M | Information about the application service associated with the spatial anchor group. |
| Access control rules | O | Access control rules defining which entities are permitted to discover the spatial anchor group. |
| Service ID | M | The identifier of the application service. |
| List of spatial anchors IDs | M | List of spatial anchors IDs to be associated with the group. |

##### 8.3.x.3.2	Spatial anchor group create response

Table 8.3.x.3.2-1 describes information elements for the spatial anchor group create response sent by the SAn server.

Table 8.3.x.3.2-1: Spatial anchor group create response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the spatial anchor group create request was successful. |
| >Spatial anchor group ID | M | The unique identifier of the created spatial anchor group. |
| >List of Spatial anchor IDs | M | List of spatial anchor IDs associated with the created spatial anchor group. |
| Failure response (NOTE) | O | Indicates that the spatial anchor group create request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: Only one of these IE shall be present. |  |  |

* * * End Change * * * *