3GPP TSG-SA WG6 Meeting #66	S6-251168

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251xxx)

# Source:	China Mobile

# Title:	New SID on Study on Application enablement aspects to support Integrated Sensing and Communication

# Document for:	Approval

# Agenda Item:	11

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Application enablement aspects to support Integrated Sensing and Communication

Acronym:	FS_ISAC_APP

Unique identifier:

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  | X |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a study

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| Sensing | SA1 | 1000026 | Stage 1 of Integrated Sensing and Communication |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 950003 | Study on Integrated Sensing and Communication | Stage 1 Study Item on Integrated Sensing and Communication |

Dependency on non-3GPP (draft) specification:

3	Justification

5G wireless sensing is a technology enabler to acquire information about characteristics of the environment and/or objects within the environment. The related service requirements and KPIs are specified in 3GPP TS 22.137. SA1 also studied 32 sensing related use cases in 3GPP TR 22.837 including smart home/city, UAS, traffic management, etc.

During the SA2#167 and subsequent SA#107, SA2 agreed to start the study on the function enhancement, end to end service operations and procedures to support Integrated Sensing and Communication with the considerations of a sub-set of use cases (e.g. commercial, automotive, public safety and emergency services) based on the SA1 requirements. The related objectives are listed in S2-2502780 (i.e. SP-250067).

As per 3GPP TS 22.137, information collected from non-3GPP sensors, e.g. radar camera or Wi-Fi sensing, can be used in 5G wireless sensing to achieve improved sensing result, or in any other way to enhance the sensing service. In SA6, the combination of 3GPP and non-3GPP sensing information (collected from the Application Layer) can be processed in the Enabler Layer to enrich sensing results for verticals. For example, non-3GPP sensing information (e.g., UAS color and model captured by a camera) can be combined with 3GPP sensing information (e.g., position) and then provided to the Application Layer.

Also as per 3GPP TS 22.137, the collection of non-3GPP sensing data and joint processing of 3GPP/non-3GPP sensing data need to be subject to user consent, regulation, and operator’s policy. SA6 can provide the application enabler capabilities addressing the user consent based on the exiting (or if necessary, enhanced) procedures specified in CAPIF.

In addition, it is also valuable to provide the sensing information together with other existing SEAL layer capabilities (e.g. Location, ADAE, ), and/or information from application enabler layer client or server to provide value added capabilities to verticals, e.g. Integrating positioning and sensing to enhance capabilities in UASs and V2X service.

It is also valuable to study the use of sensing information to optimize the existing application layer enablement behaviour, e.g. UAS and V2X etc.

In SA6#57, Study on Sensing application enabler for vertical applications (S6-233415) has already been endorsed to waiting the progress of sensing related work in SA2. Based on the recent progress of SA2 work, this SID has been updated from S6-233415 to better align with SA1 requirements and SA2 work plans.

Therefore, it is proposed to study the application enablement aspects to support Integrated Sensing and Communication in SA6.This study will run in parallel with SA2’s ISAC work, focusing on:

a)	user consent management of Sensing information exposure;

b)	the procedures related to non-3GPP sensing data (shall be based on the progress of SA2's SID);

c)	value-added services by combining of sensing information and existing SEAL layer capabilities; and

d)	usage of sensing information to optimize application layer enablement behaviour.

4	Objective

The objectives of the Rel-20 study item on Application enablement aspects to support Integrated Sensing and Communication include:

a)	study the application enabler layer architecture to provide sensing information and results to vertical applications;

b)	user consent management of sensing information exposure by using the existing or enhanced CAPIF procedures;

c)	feasibility study of non-3GPP sensing information/result provided by non-3GPP sensors, e.g., camera, LiDAR, from application layer, and the combination of the 3GPP/non-3GPP sensing information, including:

- i.	procedures to discover/select non-3GPP sensors (e.g., cameras, LiDAR) in application layer; and

- ii.	procedures to support the joint processing of the 3GPP/non-3GPP sensing information to enrich the sensing result and exposure of the enriched result in application enabler layer;

d)	feasibility study of combining the sensing information/results and SEAL capabilities (by enhancing existed SEAL functional entities or new SEAL functional entities when needed) as value added service to vertical applications; and

e)	feasibility study on usage of sensing information to optimize existing application layer enablement, e.g. UAS, V2X, etc, behaviour.

NOTE 1:	The study will first focus on existing SA6 use cases (e.g., UAS and V2X) and then analyze whether the common capabilities can be extended to other use cases. The normative work related to the enhancement of a specific enabler shall be addressed within that enabler.

NOTE 2:	The term 'non-3GPP sensors' in this SID refers to sensors that do not access the 5G Core Network, such as sensors on the Internet. The non-3GPP sensing information may be obtained by the Enabler from the application layer.

NOTE 3:	Objectives and solutions of this study should not establish new requirements for SA2.

NOTE 4:	For objective b), if any gaps in user consent management are identified in this SID, the following normative work should be addressed within the CAPIF normative work.

NOTE 5:	The progress of objectives c) is contingent upon the Rel-20 SA2 study’s agreement and scope. Their finalization requires SA2’s consolidated conclusion. If SA2 does not reach a clear decision on non-3GPP sensing, no normative work on objectives c) shall continue.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.700-abc | Study on Application enablement aspects to support Integrated Sensing and Communication | SA#xxx (TBC) | SA#xxx (TBC) |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

7	Work item leadership

SA6

8	Aspects that involve other WGs

SA2 for 3GPP systems aspect

SA3 for security and privacy aspects

SA5 for the potential charging and OAM aspect

9	Supporting Individual Members

Supporting IM name

China Mobile

Huawei

Hisilicon

CATT?

InterDigital?

ZTE?

China Telecom?

VIVO?

TD-tech?

China Unicom

Lenovo

Hytera?

Toyota?

Kyonggi University?

Convida Wireless LLC?