3GPP TSG-SA WG6 Meeting #66	S6-251178

Goteborg, Sweden, 7th – 11th April 2025	(revision of )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0281 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to access control policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF, TEI19 |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The AEF obtains the API access control policy from the CCF using the Obtain access control policy API. If there are multiple AEF instances for a service API, it is under-specified how the policies related to the service API (e.g., total number of API invocations) are handled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete in clause 8.12.2.1 that the operation includes in the request the AEF identifier. Complete in 8.12.3 how the access control policy is obtained and enforced. Complete in 8.17.3 and 8.18.3 the steps corresponding to the AEF obtaining and enforcing access control policies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification leading to misinterpretations and implementation mistakes |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.12.2.1, 8.12.3, 8.17.3, 8.18.3, Annex E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.12.2.1	Obtain access control policy request

Table 8.12.2.1-1 describes the information flow obtain access control policy request from the AEF to the CAPIF core function.

Table 8.12.2.1-1: Obtain access control policy request

| Information element | Status | Description |
| --- | --- | --- |
| AEF Identity information | M | Identity information of the entity requesting the access control policy. |
| Service API identification | M | The identification information of the service API for which the access control policy is being requested. |

* * * Second Change * * * *

### 8.12.3	Procedure

Figure 8.12.3-1 illustrates the procedure for obtaining policy to perform access control on the service API invocations.

Pre-conditions:

1.	The AEF is hosting the service API but the policy to perform access control is not available with AEF.

2.	The CAPIF core function is configured with the access control policies corresponding to one or more service APIs.

3.	Authorization details of the AEF are available with the CAPIF core function.

Figure 8.12.3-1: Procedure for the AEF obtaining service API access control policy

1.	The AEF sends an obtain access control policy request to the CAPIF core function for obtaining the policy to perform the access control on service API invocations by including the details of the hosted service API.

2.	The CAPIF core function checks whether the AEF is authorized to receive the access control policy corresponding to the service APIs requested.

3.	If authorization check is successful, the AEF is provided the access control policy for the service API via an obtain access control policy response. If authorization check is not successful, the AEF is provided with a failure indication via a obtain access control policy response. The AEF enforces the access control policy for the service API and API invoker.

NOTE 1:	To maintain synchronization between the AEF and the CAPIF core function for the policy cached at AEF, the AEF can subscribe to the policy update event at CAPIF core function according to the procedure in subclause 8.8.3 and receive notifications about any updated policy at CAPIF core function according to the procedure in subclause 8.8.4.

NOTE 2:	As specified in table E-1, the CAPIF core function is configured with access control policies per service API of an API provider domain. In a distributed deployment of CAPIF, where the functionality of the API exponsing function is distributed among multiple API exposing functions (e.g. with cascading API exposing functions, as described in clause 7.3), only one of the AEFs requests and enforces the access control policy for the service API, e.g. the AEF providing the service communication entry point to the service APIs.

* * * Third Change * * * *

### 8.17.3	Procedure

Figure 8.17.3-1 illustrates the procedure for service API access control.

Pre-conditions:

1.	The API invoker has performed the service API discovery and received the details of the service API which includes the information about the service communication entry point of the AEF in the CAPIF.

2.	The API invoker is authenticated and authorized to use the service API.

3.	The AEF in the CAPIF is configured with at least one access policy to be applied to the service API invocation corresponding to the API invoker and service API.

Figure 8.17.3-1: Procedure for service API access control

1.	The API invoker performs service API invocation according to the interface of the service API by sending a service API invocation request towards the AEF which exposes the service API towards the API invoker. The AEF acts as an access control entity.

2.	If the access control policy is not configured with AEF, then the AEF may obtain the access control policy configuration from the CAPIF core function as described in clause 8.12.3.

3.	Upon receiving the service API invocation request from the API invoker, the AEF checks for configuration for access control. As per the configuration for access control, the AEF performs access control on the service API invocation request as per the operator policy.

4.	The API invoker receives a service API invocation response for service API invocation from the AEF providing the service API.

* * * Fourth Change * * * *

### 8.18.3	Procedure

Figure 8.18.3-1 illustrates the procedure for CAPIF access control.

Pre-conditions:

1.	The API invoker has performed the service discovery and received the details of the service API which includes the information about the service communication entry point of the AEF-1 in the CAPIF.

2.	The API invoker is authenticated and authorized to use the service API.

3.	The AEF-1 in the CAPIF is configured with at least one access policy to be applied to the service API invocation corresponding to the API invoker and service API.

Figure 8.18.3-1: Procedure for CAPIF access control with cascaded AEFs

1.	The API invoker performs service API invocation according to the interface of the service API by sending a service API invocation request towards the AEF-1 which exposes the service API towards the API invoker, and acts as access control entity.

2.	Upon receiving the service API invocation request from the API invoker, the AEF-1 checks for configuration for access control as described in clause 8.12.3. As per the configuration for access control, the AEF-1 performs access control on the service API invocation as per the operator policy.

3.	The AEF-1 forwards the incoming service API invocation request to the service API provided by AEF-2.

4.	The AEF-1 receives a service API invocation response for service API invocation from AEF-2.

5.	The AEF-1 resolves the destination API invoker address and modifies the source address information of AEF-2 within the service API invocation response and forwards the service API invocation response to the API invoker.

* * * Fifth Change * * * *

Annex E (normative):
Configuration data for CAPIF

The configuration data is stored in the CAPIF core function and provided by the CAPIF administrator.

The configuration data for CAPIF is specified in table E-1.

Table E-1: Configuration data for CAPIF

| Reference | Parameter description |
| --- | --- |
| Subclause 4.2.2 | List of published service API discovery restrictions |
|  | > Service API identification |
|  | > API invoker identity information |
|  | > Service API category |
| Subclause 4.7.2 | List of service API log storage durations |
|  | > Service API identification |
|  | > Service API log storage duration (in hours) (see NOTE) |
| Subclause 4.7.4 | List of API invoker interactions log storage durations |
|  | > Service API identification |
|  | API invoker interactions log storage duration (in hours) (see NOTE) |
| Subclause 4.10 | List of access control policy for a service API (of an API provider domain) per API invoker and optionally per network slice |
|  | > Volume limit on service API invocations (total number of invocations allowed) |
|  | > Time limit on service API invocations (The time range of the day during which the service API invocations are allowed) |
|  | > Rate limit on service API invocations (allowed service API invocations per second) |
|  | > Service API identification |
|  | > API invoker identity information |
|  | > Network Slice Info |
| Subclause 8.34.3 | Group context information |
|  | > Group identifier |
|  | > List of UE IDs (group members) |
|  | > UE identifier of the Group Resource Owner |
|  | > Authorization information related to the list of supported applications |
|  | >> Application identifer |
|  | >> Purpose |
|  | >> Scope |
| NOTE: If no value is set for the duration, the duration is assumed to be unlimited. |  |

* * * End of Changes * * * *