3GPP TSG-SA WG6 Meeting #66	S6-251180

Goteborg, Sweden, 7th – 11th April 2025	(revision of )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0283 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Revocation on CAPIF interconnection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following ENs still remain in the specification. Editor's note: It is FFS whether additional IEs need to be included e.g. API invoker information based on SA3 progress on RNAA authorization token revocation. However, there is no dependency with SA3 progress: - SA3 does not specify authorization revocation procedures, but token revocation procedures. - Token revocation procedures do not depend on authorization revocation procedures. Mind that while a token may be valid (i.e. the permissions delegated to the application may still be valid), the API invoker operation on the service API might not be valid (e.g., the API invoker is surpassing the access control policy) - If an authorization token is revoked, the access to the service API may become not authorized, and in this case, the service API authorization revocation procedure may be triggered, which may include as cause authorization token revocation. Therefore, ENs in clauses 8.25.2.11 and 8.25.2.13 can be removed. In addition, to identify the API invoker involved in the Authorization revocation procedure, API invoker identification information needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The ENs in clauses 8.25.2.11 and 8.25.2.13 are removed - The API invoker identity is addedd in the flows for interconnect revoke API invoker authorization response and notify |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The API invoker cannot be identified in the revocation of API invoker authorization in interconnection scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.25.2.11, 8.25.2.13, 8.25.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.25.2.11	Interconnection revoke API invoker authorization request

Table 8.25.2.11-1 describes the information flow interconnection revoke API invoker authorization request from one CCF to another CCF.

Table 8.25.2.11-1: Interconnection revoke API invoker authorization request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the requesting CAPIF core function, may include identity, authentication and authorization information. |
| API invoker identity information | M | The information that determines the identity of the API invoker |
| Service API identification | M | The identification information of the service API for which the authorization is revoked. |
| Cause | M | The cause for revoking the API invoker authorization |

* * * Next Change * * * *

#### 8.25.2.13	Interconnection revoke API invoker authorization notify

Table 8.25.2.13-1 describes the information flow interconnection revoke API invoker authorization notify from one CCF to another CCF.

Table 8.25.2.13-1: Interconnection revoke API invoker authorization notify

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the notifying CAPIF core function, may include identity, authentication and authorization information. |
| API invoker identity information | M | The information that determines the identity of the API invoker |
| Service API identification | M | The identification information of the service API for which the authorization is revoked. |
| Cause | M | The cause for revoking the API invoker authorization |

* * * Next Change * * * *

#### 8.25.3.8	Procedure for CAPIF revoking API invoker authorization in CAPIF interconnection

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API. In RNAA scenarios, the API invoker obtained RO authorization to use the service API.

2.	The CCF-B may have available access control policies corresponding to the service API. The CCF-A requested to CCF-B the access control policies for the service API.

3.	The AEF in the CAPIF-A may be configured with the access policy to be applied to the service API invocation corresponding to the API invoker and the service API..

4.	The CCF-A is triggered to revoke API invoker authorization for service API access, e.g. AEF requested the revocation of API invoker authorization.

Figure 8.25.3.8-1: Procedure for revoking API invoker authorization in CAPIF interconnection

1.	If the CCF-B keeps service API authorization information for the API invoker, the CCF-A sends an interconnection revoke API invoker authorization request to the CCF-B with the details of the API invoker, the AEF and the service API.

2.	Upon receiving the information to revoke the API invoker's authorization for service API invocation, the CCF-B invalidates the API invoker authorization corresponding to the service API.

3.	The CCF-B sends an interconnection revoke API invoker authorization response to the CCF-A.

4.	Instead of step 1 to 3, if the CCF-A keeps service API authorization information for the API invoker, the CCF-A sends an interconnection revoke API invoker authorization notify to the CCF-B.

5.	The CCF-A invalidates the API invoker authorization corresponding to the service API.

6.	The CCF-B sends a revoke API invoker authorization notify to the API invoker whose authorization to access the service API has been revoked.

* * * END of Change * * * *