3GPP TSG-SA WG6 Meeting #66	S6-251183

Goteborg, Sweden, 7th – 11th April 2025	(revision of )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0286 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solving ENs on UE-accessing other UEs’ resources of a group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are ENs in clause 8.34 that need to be solved: - 8.34.3 Editor’s note: How the alternatives can be chosen by API invoker is FFS. The alternatives refer to the two possible alternative scenarios for the data controller of the list of UEs that belong to a Group and that delegate the authorization to a GRO. The list of UEs that belong to a group is application specific. E.g. I may allow access (as GRO) to my car connected devices (identified as a Group of UEs) to “MyCarBrand” application. The control of the members of the Group of UEs can be performed by the “MyCarBrand” application in the Application Service Provider or can be delegated to the operator (CAPIF provider). In addition, the set of parameters included when the Data Controller is the Application Service Provider needs to be corrected, since if the GRO Id is provided instead of the Group Identifier, no additional configuration is required in the CAPIF provider and the solution is simplified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarification of how the API invoker determines who is handling the Group is based on service agreements between the Application Service Provider and the CAPIF provider. - Clarification that the solution when the ASP is the data controller is the basically the same as the solution in 8.37. - The ENs indicating the dependencies for the security aspects with SA3 is transformed into a NOTE. - The set of parameters included in the API invoker request when the Data Controller is the API invoker is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear and uncomplete procedure for the UE access to resources of other UEs that belong to the same group. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.34.1, 8.34.2, 8.34.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.34.1	General

This clause specifies the procedures for API invoker on a UE to be able to access the services exposing resources of other UEs of a group. The UEs belong to the same group and the same VAL service provider (IoT case) or the same user and all of them belong to the same CAPIF provider domain.

There are two possible scenarios:

1.	The Data Controller of the authorization relationship between UE2 (the API invoker is in UE2) and UE1,given by the membership to the same group, is the Application Service Provider. The difference of this scenario compared with the procedure of an API invoker obtaining authorization from resource owner described in clause 8.31 is that:

a.	the API invoker is deployed in a UE different to the UE whose resources are being accessed,

b.	the API invoker needs to provide in the request, in addition to the identity of the UE whose resources are being accessed, the identity of the UE authorizing the access, i.e., the Group Resource Owner;.

2.	The Data Controller of the authorization relationship between UE2 (the API invoker is in UE2) and UE1, given by the membership to the same group,  is the CAPIF provider domain. The difference of this scenario compared with the procedure of an API invoker obtaining authorization from resource owner described in clause 8.31, is that:

a.	the API invoker is deployed in a UE different to the UE whose resources are being accessed,

b.	the API invoker needs to provide in the request, in addition to the identity of the UE whose resources are being accessed, the Group Identifier and the identity of the UE2;  and

c.	the group membership needs to be managed by the CAPIF provider as described in Annex E.

Whether the Data Controller of the Group is the Application Service Provider or the CAPIF provider is part of the service agreement between the Application Service Provider and the CAPIF provider, and hence known beforehand by the API invoker.

The solution for scenario 1 is developed in clause 8.37.3, with the difference that the UE authorizing the access may not be the UE whose resources are being accessed. The following subclauses develop scenario 2.

* * * Second Change * * * *

### 8.34.2	Information flows

NOTE:	The security aspects of this procedure are specified in 3GPP TS 33.122 [12].

* * * Third Change * * * *

### 8.34.3		Procedure

Figure 8.34.3-1 presents the procedure for enabling a UE-hosted API invoker accessing network-hosted resources owned by other UEs that belong to the same group for the scenario 2 described in clause 8.34.1.

Pre-condition:

1.	The network hosts individual resources for a group of UEs which can be accessed via AEF provided the necessary security conditions are met.

2.	UE1 and UE2 belong to the same group and the same VAL service provider (IoT case) or the same user.

3.	CCF is provisioned with the information of a Group Resource Owner (GRO) which indicates UE 1 as the authorized RO (e.g. by business relationship between the group of UEs) to provide resource owner authorization for the resources belonging to the UEs in this group.

Figure 8.34.3-1: UE-deployed API invoker accessing other UEs’ resources of a group

1.	The API invoker (e.g., in UE 2) sends an Obtain service API authorization request to the CCF for obtaining permission to access the service API for other UE's resources hosted in the network (e.g., location) by including the API invoker identity information and any information required for authentication of the API invoker. The request includes API invoker information, the group identifier, and the UE in a group whose resources are to be accessed, and theidentity of UE2. Additionally, the API invoker may include in the request information about the purpose and operations (scope) to be performed on the targeted resources.

2.	CCF performs authentication of the API invoker (using authentication information) as specified in 3GPP TS 33.122 [12].

3.	The CCF, based on the group identifer determines that RO authorization is provided by a GRO. The CCF shall additionally check whether UE2 belongs to the group.

4.	If step 3 is successful, the CCF performs the resource owner authorization check using the GRO as the RO for the requested resources of other UE(s) belonging to the group.

Editor’s note:	The authorization details for step 4 of how CCF interacts with GRO to obtain authorization which also includes the group identification is FFS and will be specified by SA3.

5.	Based on the group resource owner authorization received in step 4, the CCF sends an Obtain service API authorization response to the API invoker. The authorization response indicates that the access to UE (resource owner) network resource(s) is authorized for an indicated scope.

6.	The API invoker sends service API invocation request to the API exposing function with the RO authorization information received in step 5.

7.	The API invoker receives the service API invocation response resulting from the service API invocation once the API exposing function has checked whether the API invoker is authorized to invoke that service API based on the RO authorization information.

* * * END of Change * * * *