3GPP TSG-SA WG6 Meeting #66	S6-251184

Goteborg, Sweden, 7th – 11th April 2025	(revision of )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0287 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Obtain service API authorization for RNAA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 8.31.3, the following aspects need to be corrected/clarified: - In step 1, the obtain service API authorization request indicates that the API invoker may include, besides the API invoker identity, service information as application service provider and application identifier. Considering that the binding between the API invoker identity and the related application information is permanent and unique, it is not needed to include the application related information in every authorization request, but to keep it in the API invoker’s profile created during onboarding. - The obtain service API authorization request must include information identifying the service API, and this aspect is not clear in the current wording. Additionally, the request may include finer granularity information about the operation(s) and resource(s) for which authorization is requested, being this aspect also unclear. The obtain service API authorization does not include RO information, except the RO identity. - The CAPIF core function, to return authorization information to the API invoker, checks the restrictions to access to the requested service API, which combines API Invoker authorization information available in the CCF (as described in clause 8.11.3) and RO authorization information retrieved via ROF. This aspect is not clear either, and there is an ENs related to it. - The description of the parameters included in the request to the ROF to retrieve RO authorization though they are similar to the parameters included in the API invoker request, they are not exactly the same (e.g., the RO needs to know the App that is asking for permissions, but does not need to know the API invoker identity the CCF allocated to the application). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 8.31.3 is updated: - To remove API invoker’s application information from the Obtain Service API authorization request and clarify service API related parameters and RO related parameters. - To clarify the possibility to combine API invoker authorization information and RO authorization information to determine the authorization reply by the CCF to the API invoker. The EN is removed and the reference for completion to SA3 is added. - To clarify the set of parameters the RO may need to provide RO authorization information via the ROF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Obtain service API authorization procedure is incorrect. Setting unnecessary requirements may create misunderstandings and misuse of the specified functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.31.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.31.3	Procedure

Figure 8.31.3-1 illustrates the procedure for API invoker obtaining authorization from resource owner.

Pre-conditions:

1.	The resource owner function (ROF) can communicate with the CCF (Authorization function).

2.	Access to an API exposing function offered service API requires obtaining authorization from a resource owner (RO).

Figure 8.31.3-1: Procedure for API invoker obtaining authorization from resource owner

1.	The API invoker requests to obtain authorization information to invoke the service API exposed by the API exposing function (AEF) and to access information owned by the resource owner at the AEF through the invocation of an obtain service API authorization request to the CCF. The request contains API invoker information required for authorization, the information identifying the service API, the purpose for data processing and any information required for authentication of the API invoker. The request may include finer level service API access requirements (e.g., access per service API operation or access per API service resource) and resource owner-related information required to obtain resource owner authorization information.

2.	The CCF performs the authentication of the API invoker (using authentication information). Then the Authorization function determines the authorization by checking the authorization information available in the CCF as described in clause 8.11.3 and by checking the authorization information provided by the RO via the ROF. The request to the ROF contains application service information (e.g. the application service provider and application identifier), the purpose for data processing and the resource owner data information for which the API invoker requests access grants.

NOTE 1:	The detailed procedure to obtain the RO's authorization information is specified in TS 33.122 [12].

NOTE 2:	Step 1 can occur when the API invoker receives a failure in Service API invocation response indicating that authorization information from resource owner(s) is required.

3.	Based on the RO authorization information obtained via the ROF, the CCF sends an obtain service API authorization response to the API invoker.

NOTE 3:	How the finer service API authorization granularity is combined with the purpose and RO authorization granularity for data access and then returned by CCF to to the API invoker in the authorization reply is specified in 3GPP TS 33.122 [12].

4.	The API invoker sends service API invocation request to the API exposing function with the authorization information received from CCF (Authorization function) in step 3.

5.	The API invoker receives the service API invocation response resulting from the service API invocation once the API exposing function has checked whether the API invoker is authorized to invoke that service API based on the authorization information received from CCF (Authorization function).

* * * END of Change * * * *