3GPP TSG-SA WG6 Meeting #66	S6-251186

Goteborg, Sweden, 7th – 11th April 2025	(revision of )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0143 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to server initiated SEALD data transmission connectivity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | During SA6#63, a new service operation was introduced in CR 0102 to differentiate the regular transmission connection request sent from the server from the regualr transmission connection request sent from the client. However, the information flows were not updated accordingly. Clause 9.2.3.3 and 9.2.3.4 need to update the description of the operation and in addition, 9.2.3.4 needs to remove the information related to server side initiated request. Clause 9.2.3.18 needs to include the information related to server side initiated request. Clause 9.2.4.3 needs to refer to the flow information for server side initiated operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 9.2.3.3 and 9.2.3.4 is updated to correct table description.Clause 9.2.3.4 is updated to remove the “Capability for BAT and periodicity adaptation” and “Transmission assistance info” IEs. Clause 9.2.3.18 is updated to include the Transmission assistance infor IE and to remove the duplicated Capability for BAT and periodicity adaptation. Clause 9.2.4.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification leading to implementation mistakes |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.3, 9.2.3.4, 9.2.3.17, 9.2.3.18, 9.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************************* First Change *****************************/

#### 9.2.3.3	SEALDD regular transmission connection establishment request

Table 9.2.3.3-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the regular SEALDD connection establishment.

Table 9.2.3.3-1: SEALDD regular transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (SEALDD client). |
| SEALDD-UU flow ID | M (See NOTE 1) | Identity of the SEALDD-UU flow. |
| VAL server ID | O | Identity of the VAL server, applicable for SEALDD client side initiated request. |
| VAL service ID | O | Identity of the VAL service |
| Selected VAL server endpoint | M | Endpoint of the selected VAL server |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD client side used to establish SEALDD connection. |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID. |
| Capability for BAT and periodicity adaptation | O (See NOTE 2) | Indicates BAT and periodicity adaptation capability of the SEALDD client side (for client side initiated request) per SEALDD traffic descriptor. |
| Transmission assistance info | O (See NOTE 2, NOTE 4) | Indicates transmission assistance information for uplink SEALDD traffic of the SEALDD client side (for client side initiated request). It includes BAT, BAT window, periodicity, and periodicity range per SEALDD traffic descriptor. |
| L4S feedback capability | O (See NOTE 3) | Identifies the L4S feedback capability (i.e. ECN identification, L4S feedback) for client side initiated request |
| VAL UE client access capability | O | Indicates the access technologies supported by VAL UE client. E.g.,3GPP, Non-3GPP(WLAN) |
| XR Application device capability information | O (See NOTE 5) | Indicates XR Application device capability information (e.g., encoding capabilities) |
| NOTE 1: The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and data transmission session. NOTE 2: If provided, BAT window and periodicity range are mutually exclusive with capability for BAT and periodicity adaptation. NOTE 3: This IE is used for the SEALDD enabled congestion control for VAL applications, as specified in clause 9.8.2.2. NOTE 4: The periodicity range may only be present together with the periodicity when BAT and BAT window are present. The BAT window may only be present together with the BAT. NOTE 5: This IE is used for the SEALDD enabled XR traffic data delivery. |  |  |

# /******************************* Second Change *****************************/

#### 9.2.3.4	SEALDD regular transmission connection establishment response

Table 9.2.3.4-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the regular SEALDD connection establishment.

Table 9.2.3.4-1: SEALDD regular transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD server side (for client side initiated request). |
| Pending timer | O (See NOTE 1) | The pending timer to trigger the re-connection from SEALDD client when bandwidth limit check is failed. |
| Suggested traffic transmission bandwidth | O (See NOTE 1) | The suggested traffic transmission bandwidth used by SEALDD client or SEALDD server to perform bandwidth control for VAL users, including UL/DL. |
| Cause | O (See NOTE 2) | Indicates the reason for the failure, e.g. SEALDD policy mismatch. |
| NOTE 1: These IEs are used for the SEALDD enabled bandwidth control for different VAL users, applicable for client side initiated request. NOTE 2: This IE is only present if the Result is failure |  |  |

# /******************************* Third Change *****************************/

#### 9.2.3.17	SEALDD data transmission connection establishment initiated by SEALDD server request

Table 9.2.3.17-1 describes the information flow from the SEALDD server to the SEALDD client for requesting the SEALDD data transmission connection establishment.

Table 9.2.3.17-1: SEALDD data transmission connection establishment initiated by SEALDD server request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (SEALDD server). |
| SEALDD-UU flow ID | M (See NOTE 1) | Identity of the SEALDD-UU flow. |
| VAL service ID | O | Identity of the VAL service |
| Selected VAL server endpoint | M | Endpoint of the selected VAL server |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD server used to establish SEALDD connection. |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID. |
| SEALDD communication lifetime | O | Identifies the DD communication lifetime. |
| NOTE 1: The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and data transmission session. |  |  |

# /******************************* Fourth Change *****************************/

#### 9.2.3.18	SEALDD data transmission connection establishment initiated by SEALDD server response

Table 9.2.3.18-1 describes the information flow from the SEALDD client to the SEALDD server for responding to the SEALDD data transmission connection establishment.

Table 9.2.3.18-1: SEALDD data transmission connection establishment initiated by SEALDD server response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD client side used to establish SEALDD-UU data connection. |
| Suggested traffic transmission bandwidth | O (See NOTE 1) | The suggested traffic transmission bandwidth used by SEALDD client to perform bandwidth control for VAL users, including UL/DL. |
| Cause | O (See NOTE 2) | Indicates the reason for the failure, e.g. SEALDD policy mismatch. |
| Capability for BAT and periodicity adaptation | O (See NOTE 3) | Indicates BAT and periodicity adaptation capability for SEALDD client. |
| Transmission assistance info | O (See NOTE 3, NOTE 4) | Indicates transmission assistance information for uplink SEALDD traffic of the SEALDD client. It includes BAT, BAT window, periodicity, and periodicity range per SEALDD traffic descriptor. |
| NOTE 1: These IEs are used for the SEALDD enabled bandwidth control for different VAL users. NOTE 2: This IE is only present if the Result is failure NOTE 3: If provided, BAT window and periodicity range are mutually exclusive with capability for BAT and periodicity adaptation. NOTE 4: The periodicity range may only be present together with the periodicity when BAT and BAT window are present. The BAT window may only be present together with the BAT. |  |  |

# /******************************* Figth Change *****************************/

#### 9.2.4.3	Sdd_RegularTransmissionConnection_Establish operation

API operation name: Sdd_RegularTransmissionConnection_Establish

Description: The consumer requests for one time for SEALDD enabled regular data connection establishment.

Inputs: See clause 9.2.3.3 for SEALDD client side intiated operation and clause 9.2.3.17 for server side initiated operation.

Outputs: See clause 9.2.3.4 for SEALDD client side intiated operation and clause 9.2.3.18 for server side initiated operation.

See clause 9.2.2.1, 9.2.2.2 and 9.2.2.3 for details of usage of this operation.

# /******************************* End of Changes *****************************/