3GPP TSG-SA WG6 Meeting #66	S6-251188

Goteborg, Sweden, 7th – 11th April 2025	(revision of )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0382 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NRM enabled S&F transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified in TS 22.865, Annex A, the Store and Forward satellite operation mode “is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service. (…) (…)under “S&F Satellite operation” mode, the end-to-end exchange of signalling/data traffic is now handled as a combination of two steps not concurrent in time (Step A and B in Figure A-1). In Step A, signalling/data exchange between the UE and the satellite takes place, without the satellite being simultaneously connected to the ground network (i.e. the satellite is able to operate the service link without an active feeder link connection). In Step B, connectivity between the satellite and the ground network is established so that communication between the satellite and the ground network can take place. So, the satellite moves from being connected to the UE in step A to being connected to the ground network in step B”. However, in the currently specified solution it is not considered that the feeder and the service link are not concurrent in time, and that, as specified in TS 23.401, some network functionality is needed onboard the satellite to enable the forwarding of signalling between the NRM client and the NRM server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce a new NRM S&F function in the satellite. - Split the service procedures between the NRM client and the NRM S&F function in the satellite and the NRM S&F function in the satellite and the NRM server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VAL cannot use NRM to control data delivery when the UEs are in S&F Satellite operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.3.1, 21.3.2.1, 21.3.2.2, 21.3.2.3, 21.3.3.1, 21.3.3.2, 21.3.3.3, 21.3.3.4, 21.3.3.5, 21.3.3.6, 21.3.3.7, 21.3.3.8, 21.3.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************************* First Change *****************************/

### 21.3.1	General

This following sub-clauses shows how the application enablers provisioning the satellite S&F configuration to the VAL UE and expose the satellite S&F events information to the VAL server to help minimize the service interruption (e.g., adjust the DL frequency, etc.) when the VAL UE starts to perform the satellite S&F operation.

NOTE:	In S&F Satellite operation, the end-to-end exchange of traffic is handled in a sequence of steps reflecting the intermittent availability of the service link (when the satellite can exchange data with the NRM client) and the intermittent availability of the feeder (ground) link (when the satellite can exchange data with the ground network (NRM server)). To support S&F satellite operation, network functionality needs to be deployed on the satellite, as specified in 3GPP TS 23.401 [9].

# /******************************* Second Change *****************************/

#### 21.3.2.1	NRM Server exposing the S&F events to the VAL server procedure

Pre-condition:

-	The NRM client, the NRM server and the VAL server all are deployed on the ground.

-	The PGW-U in 3GPP CN is deployed on the satellite.

-	NRM S&F functionality is deployed on the satellite.

-	When the Service Link is available, the VAL UE registers to the 3GPP network via satellite access as defined in clause 5.3.2.1 of 3GPP TS 23.401 [9] and supports the S&F mode.

Figure 21.3.2.1-1: NRM Server exposing the satellite S&F events to the VAL server procedure

1.	The VAL server sends the satellite S&F events subscription request to the NRM server in which the subscribed events may include the UE status in S&F mode, the DL estimated delivery time, the trigger conditions, etc.

NOTE:	Step 1 is also applicable to the update procedure when the VAL server requires to update the satellite S&F events subscription request.

2.	The NRM server authorizes the subscription request from the VAL server.

3.	If the subscription request is authorized, the NRM server sends the satellite S&F events subscription request tothe NRM S&F functionality in the satellite and/or the 3GPP CN respectively.

3a.	When the Feeder Link is available, the NRM server may send (alternatively or in addition to step 3) the satellite S&F events subscription request to the NRM S&F functionality in the satellite.

3b.	When the Service Link is available, the NRM S&F function, if received from the NRM server a subscription request as described in step step 3a, sends the satellite S&F events subscription request to the NRM client.

4.	The 3GPP CN acknowledges the NRM server with the satellite S&F events subscription response respectively.

4b.	When the Service Link is available, the NRM client acknowledges the NRM S&F functionality in the satellite with the S&F events subscription response

4a.	When the Feeder Link is available, the NRM S&F functionality in the satellite acknowledges the NRM server with the satellite S&F events subscription response.

5.	The NRM server responds to the VAL server with the satellite S&F events subscription response.

6.	If the trigger condition in the subscription request is met, the 3GPP CN may report the requested satellite S&F events as specified in table 21.3.3.3 to the NRM server via the satellite S&F events subscription notification.

6b.	When the Service Link is available, if the trigger condition in the subscription request is met, the NRM client may report to the NRM S&F functionality in the satellite the requested S&F events as specified in table 21.3.3.3.

6a.	When the Feeder Link is available, the NRM S&F functionality in the satellite reports the requested satellite S&F events as specified in table 21.3.3.3 to the NRM server using the satellite S&F events subscription notification.

7.	The NRM server notifies the VAL server for the received satellite S&F events in step 6. The VAL server may take them into consideration and adjust the application behaviour (e.g. pending the DL data transition when the UE is in S&F Mode, re-transmit the pending DL data when the feeder link is available, adjust the DL traffic volume according to the DL estimated delivery time, etc.) to minimize the service interruption.

8.	The VAL server sends the satellite S&F events unsubscribe request to the NRM server to unsubscribe the subscription to monitor the satellite S&F events.

9.	The NRM server relays the satellite S&F events unsubscribe request to the 3GPP CN and receives the response from the 3GPP CN.

9a.	When the Feeder Link is available, the NRM server relays the satellite S&F events unsubscribe request to the NRM S&F functionality in the satellite.

9b.	When the Service Link is available, the NRM S&F functionallity in the satellite relays the satellite S&F events unsubscribe request to the NRM client

10.	The NRM server replies to the VAL server with the satellite S&F events unsubscribe response.

# /******************************* Third Change *****************************/

#### 21.3.2.2	VAL UE reports the UE satellite information procedure

The Figure 21.3.2.2-1 illustrates the procedure of the VAL UE/NRM client reports the UE satellite information to the NRM server.

This procedure also applies when the VAL UE/NRM client updating its UE satellite information to the NRM server.

Pre-condition:

-	When the Service Link is available, the VAL UE has registered to the 3GPP network via satellite access as defined in clause 5.3.2.1 of 3GPP TS 23.401 [9] and supports the S&F mode.

Figure 21.3.2.2-1: VAL UE reports the UE satellite information procedure

1b.		When the Service Link is available, the NRM client reports the UE satellite information to the NRM S&F functionallity in the satellite,

1a.	When the Feeder Link is available, the NRM S&F functionality in the satellite reports the received UE satellite information to the NRM server.

The request may contain the UE identity, the UE type (e.g. IoT) and the UE satellite related information, such as the serving satellite ID, the accessing satellite types (LEO, MEO and GEO), the S&F mode indicator, the maximum data storage quota which indicates the maximum data storage quota per application layer on the UE for all of services, etc.

2.	The NRM server checks the authorization for the request from the NRM client via the NRM S&F functionality in the satellite.

3a.	After successful authorization, the NRM server sends a Report UE satellite capability response to the NRM S&F functionality in the satellite.

3b.	The NRM S&F functionality in the satellite sends the Report UE satellite capability response to the NRM client and stores the UE identity and the satellite information.

# /******************************* Fifth Change *****************************/

#### 21.3.2.3	NRM server provisioning the S&F configuration procedure

Figure 21.3.2.3-1 shows how the NRM Server provides the S&F configuration to the NRM Client based on the service request from VAL server and the VAL UE reported satellite information.

This procedure also applies when the NRM server provides the updated S&F configuration to the NRM client.

Pre-condition:

-	The NRM client and the NRM server are deployed on the ground.

-	The PGW-U in 3GPP CN is deployed on the satellite.

-	NRM S&F functionality is deployed on the satellite.

-	The NRM Client has reported the UE satellite information to the NRM server via the NRM S&F functionality onboard the satellite as described in clause 21.3.2.2.

Figure 21.3.2.3-1: NRM Server providing the S&F configuration to the NRM Client procedure

1.	The NRM server takes advantage of the service request initiated by the VAL server, the reported UE satellite information (e.g. serving satellite ID, the maximum data storage quota per UE) as described in clause 21.3.2.2 and its pre-configuration, etc. to determine the on-board S&F parameters for the application layer (e.g., maximum S&F data storage quota per UE/service, maximum S&F data retention per UE/service, etc.).

2a.	After determined the satellite S&F parameters, when the Feeder Link is available, the NRM server provides such information to the NRM S&F functionality in the satellitefor the NRM client/VAL UE by sending a satellite S&F configuration request.

2b.	When the Service Link is available, the NRM S&F functionality in the satellite sends the satellite S&F configuration request to the NRM client/VAL UE.

3.	The NRM client/VAL UE stores the received satellite S&F parameters and may take them into account when the VAL UE starts to operate the S&F operations based on its satellite S&F configuration per service.

NOTE 2:	The VAL UE may consider the pre-configuration, the received parameters from the application enabled layer to determine/update its satellite S&F configuration per service.

4b.	When the Service Link is available, the NRM client sends the satellite S&F configuration response to the NRM S&F functionality in the satellite.

4a.	When the Feeder Link is available, the NRM S&F functionality in the satellite sends the satellite S&F configuration response to the NRM server.

# /******************************* Sixth Change *****************************/

#### 21.3.3.1	Satellite S&F events subscription request

Table 21.3.3.1-1 describes the information flow from VAL server to NRM server or from the NRM server to NRM client (via the NRM S&F function in the satellite) for satellite S&F events subscription request.

Table 21.3.3.1-1: Satellite S&F status events subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL users or VAL UEs or NRM server. |
| Satellite S&F events | O | Lists of the satellite S&F events that to be monitored for one or more UEs (e.g. UE status in S&F mode, the DL S&F estimated delivery time). |
| Triggering conditions | M | Identifies the conditions that triggered NRM client sending the notification report. |

# /******************************* Seventh Change *****************************/

#### 21.3.3.2	Satellite S&F events subscription response

Table 21.3.3.2-1 describes the information flow from NRM server to the VAL server or from NRM client to NRM server (via the NRM S&F function in the satellite) for response of satellite S&F events subscription request.

Table 21.3.3.2-1: Satellite S&F events subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL users or VAL UEs or NRM server. |
| Subscription status | M | It indicates the subscription result |

# /******************************* Eigth Change *****************************/

#### 21.3.3.3	Satellite S&F events notification

Table 21.3.3.3-1 describes the information flow from NRM server to VAL server or from the NRM client to NRM server (via the NRM S&F function in the satellite) to notify the subscribed satellite S&F events.

Table 21.3.3.3-1: Satellite S&F status events notification

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL users or VAL UEs or NRM server. |
| Satellite S&F events | O | List the notification for satellite S&F events that to be monitored for one or more UE(s). |
| > UE status in S&F mode | O (NOTE) | Indicates the UE is registered in S&F mode or moving from S&F mode to not registered in S&F mode. |
| > The DL S&F estimated delivery time | O (NOTE) | Indicates the estimated/expected time required to deliver the data to the UE from the time data has been received in the network. |
| Triggering events | M | Identifies the events that triggered NRM client sending the report. |
| Timestamp | O | Timestamp of the notification report. |
| NOTE: The definition refers to the clause 5.6.3 of 3GPP TS 23.682 [13]. |  |  |

# /******************************* Ninth Change *****************************/

#### 21.3.3.4	Satellite S&F events unsubscribe request

Table 21.3.3.4-1 describes the information flow from VAL server to NRM server or from the NRM server to NRM client (via the NRM S&F function in the satellite) for satellite S&F events unsubscribe request.

Table 21.3.3.4-1: Satellite S&F status events unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requesting ID for VAL server or NRM server. |

# /******************************* Tenth Change *****************************/

#### 21.3.3.5	Satellite S&F events unsubscribe response

Table 21.3.3.5-1 describes the information flow from NRM server to the VAL server or from NRM client to NRM server (via NRM S&F function in the satellite) for response of satellite S&F events unsubscribe request.

Table 21.3.3.5-1: Satellite S&F events unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Request status | M | It indicates the unsubscribe request result. |

# /******************************* Eleventh Change *****************************/

#### 21.3.3.6	Report UE satellite information request

Table 21.3.3.6-1 describes the information flow from the NRM client to the NRM server (via NRM S&F function in the satellite) for reporting the UE satellite related capabilities.

Table 21.3.3.6-1: Report UE satellite information request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or identity of the VAL UE. |
| UE type | O | Indicate the type of VAL UEs (e.g. IoT). |
| Satellite information | O | The satellite information of VAL UE when the VAL UE supports the satellite access. |
| > Satellite ID | O | Indicates the serving satellite ID for the VAL UE. |
| > Satellite type | O | Indicates the accessing satellite RAT types (LEO, MEO and GEO) for the VAL UE. |
| > S&F mode indicator | O | Indicates the UE support the S&F mode or not. |
| > Maximum data storage quota | O | Indicates the maximum data storage quota for the VAL UE for all of services on the application layer. |
| > Maximum data retention | O | Indicates the maximum data retention for the VAL UE per service on the application layer. |

# /******************************* Twelfth Change *****************************/

#### 21.3.3.7	Report UE satellite information response

Table 21.3.3.7-1 describes the information flow the NRM server to the NRM client (via the NRM S&F function in the satellite) for the response of reporting the UE satellite related information request.

Table 21.3.3.7-1: Report UE satellite information response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | It indicated the request result. |

# /******************************* Thirdteenth Change *****************************/

#### 21.3.3.8	Satellite S&F configuration request

Table 21.3.8-1 describes the information flow from the NRM server to the NRM client (via NRM S&F function in the satellite)  for satellite S&F configuration request.

Table 21.3.3.8-1: Satellite S&F configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL user or VAL UE. |
| VAL service ID | O | Identity of the VAL service that is requested. |
| Satellite S&F configuration information | O | Indicates the S&F configuration per UE per service when it is using the satellite S&F operation. |
| > Maximum S&F data storage quota | O | Indicates the maximum data storage quota per UE per service on the application layer when the UE is using the satellite S&F operation. |
| > Maximum S&F data retention | O | Indicates the maximum data retention per UE per service on the application layer when the UE is using the satellite S&F operation. |

# /******************************* Fourteenth Change *****************************/

#### 21.3.3.9	Satellite S&F configuration response

Table 21.3.3.9-1 describes the information flow from the NRM client to the NRM server (via NRM S&F function in the satellite) for the response of satellite S&F configuration request.

Table 21.3.3.9-1: Satellite S&F events subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL users or VAL UEs or NRM server. |
| Result | M | It indicates the request result |

# /******************************* End Change *****************************/