3GPP TSG-SA WG6 Meeting #66	S6-251189

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0463 | rev | - | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location information for emergency private and ad hoc call requests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For an initiated “MCPTT emergency group call request”, the location information of the calling user can optionally be forwarded to another user of the call. This information element is missing in the “emergency private call request” and “ad hoc group call request” (use in case of emergency or imminent peril). The information flows for both call requests included only the location information from MCPTT client to the MCPTT server, but not from the MCPTT server to the MCPTT client. This change could provide the option to optionally forward the location information to the calling user. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding optional “location information” in the “information flow” description of the “emergency private call request (MCPTT server – MCPTT client)” 10.7.2.1.5a and “ad hoc group call request (MCPTT server – MCPTT client)” 10.19.2.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not added, there is an inconsistency between the “emergency private call request” and the “Ad hoc emergency group call request” in relation to the “emergency group call request”. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.7.2.1.5a, 10.19.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 10.7.2.1.5a	MCPTT emergency private call request (MCPTT server to MCPTT client)

Table 10.7.2.1.5a-1 describes the information flow MCPTT emergency private call request from the MCPTT server to the MCPTT client.

Table 10.7.2.1.5a-1: MCPTT emergency private call request (MCPTT server to MCPTT client) information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ID | M | The MCPTT ID of the called party |
| Emergency indicator | M | Indicates that the private call request is an MCPTT emergency call |
| Alert indicator | M | Indicates whether an emergency alert is to be sent |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit floor request | O | An indication that the user is also requesting the floor. |
| SDP offer | M | Media parameters of MCPTT client. |
| Location information | O | Location of the calling party |

# * * * * Second change * * * *

#### 10.19.2.4	Ad hoc group call request (MCPTT server – MCPTT client)

Table 10.19.2.4-1 describes the information flow ad hoc group call request from the MCPTT server to the MCPTT client.

Table 10.19.2.4-1: Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| MCPTT ID | M | The identity of the MCPTT user towards which the request is sent |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID to be associated with the ad hoc group call |
| SDP offer | M | Offered Media parameters of MCPTT server |
| Broadcast indicator (see NOTE 1) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the ad hoc group call request is an MCPTT imminent peril ad hoc group call |
| Emergency Indicator (see NOTE 1) | O | Indicates that the ad hoc group call request is an MCPTT emergency ad hoc group call |
| Preconfigured MCPTT group ID | O | Group identity whose configuration is to be applied for this ad hoc group call. |
| Location information (see NOTE 2) | O | Location of the calling party |
| Call resulting criteria for determining the participants | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both that the MCPTT server used for determining the participants e.g., it can be a location based criteria to invite participants in a particular area |
| NOTE 1: If used, only one of these information elements is present. NOTE 2: This information element can be used if either the imminent peril or the emergency indicator is set. |  |  |

# * * * * End of changes * * * *