3GPP TSG-SA WG6 Meeting #66	S6-251207

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251xxx)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on NRM target use case

Spec:	3GPP TR 23.700-35 v0.3.0

Agenda item:	8.9

Document for:	Approval

Contact:	Yanmei Yang yangyanmei@huawei.com

- Introduction

This contribution is to propose the solutions for the Generic gap #4 on NRM service.

- Reason for Change

As generic gap#4 mentioned, it is important to provide the SEAL services description from benefits/advantages/use case perspectives.

The contribution S6-251206 provides summarized information of all the SEAL services. This contribution proposes to add more detail information about NRM service in the TR.

3. Proposal

It is proposed to agree the two alternatives to 3GPP TR 23.700-35 v0.3.0.

* * * First Change * * * *

8.2.5.1.x	NRM services to target application use case

NRM is defined to transform the following 4G/5G network layer APIs to SEAL layer service API, to ease the usage of 3GPP network layer capabilities for establishing different types of AC-AS connection to assure the QoE of different applications.

Figure 8.2.5.1.x-1 Capabilities of 5G system transformed by NRM

The NRM transforms 3GPP network capabilities and UE capabilities into the following types of service APIs, supporting network layer connection with different QoS:

1）APIs for supporting 4G/5G unicast network connection with different QoS

2） APIs for supporting 4G/5G multicast/broadcast network connection with QoS

3）APIs for supporting 5G connection for URLLC(TSC/TSN) data flows

4）APIs for supporting 5G connection for background data

Rel20 may introduce new APIs to further simplfy the use of 5G network capabilities to provide different type of network connection service for the different type of application layer communications between AC and AS.

Benefits to application providers:

-The APIs support better AC-AS connection performance compared to using the best-effort 4G/5G PDU connection service (bigger data rate, lower jitter, lower latency…).

Advantages of NRM compared to Network APIs:

-Simplify the implementenion of application

EN: How NRM simply the application implemention needs more contents.

* * * End of the Change * * * *