3GPP TSG-SA WG6 Meeting #66	S6-251209

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251xxx)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on SEALDD APIs based on Application layer requirement oriented

Spec:	3GPP TR 23.700-35 v0.3.0

Agenda item:	8.9

Document for:	Approval

Contact:	Yanmei Yang yangyanmei@huawei.com

- Introduction

This contribution is to propose the solutions for defining SEAL-DD APIs based on the principle of Application layer requirement oriented.

- Reason for Change

The potential use case for third party to using SEAL-DD including the following:

a) Application layer signalling between AC and AS for different applications with different data transmission requirements.

a.1) Best effort demanding applications

a.2) Low latency demanding applications like signalling for MCX call

- Transmitting the application layer data for different applications with different data transmission requirement:

b.1) Low latency and high reliable demanding application like remote robot/machine control.

b.2) High data rate with low latency demanding application like Realtime XR video applications, Machine vision application….

b.3) Low latency like video call/voice call.

b.4) Low cost demanding like background data

b.5) Low power demanding like IoT device applications.

b.6) Multiple flows synchronisation transmission service like multi-modal XR applications

Currently the availability of the APIs for the above requirement is as the following:

| Required services by Application requirements | API(s) available | NOTES |
| --- | --- | --- |
| a.1 | Solution 5 | —— |
| a.2 | Sdd_RegularTransmission in 3GPP TS 23.433 | APIs name may cause misleading |
| b.1 | Sdd_URLLCXX in 3GPP TS 23.433 | —— |
| b.2 | Not yet | —— |
| b.3 | Not yet | —— |
| b.4 | Sdd_BDT_X in 3GPP TS 23.433 | —— |
| b.5 | Not yet | SEALDD capability for Low power is not supported yet and should be added within Rel20 |
| b.6 | Sdd_XRTransmission | APIs name may cause misleading |

This contribution is to add the missing APIs to serve the above type of applications.

3. Proposal

It is proposed to agree the two alternatives to 3GPP TR 23.700-35 v0.3.0.

* * * First Change * * * *

          - SEALDD enabled High data rate & low latency (HRLC)

The SEALDD enabled regular data transmission defined in clause 9.2.2.2	of 3GPP TS 23.433[2] can be used with the following changes:

- The term Sdd_RegularTransmission is changed to Sdd_HRLCTransmission.

- The QoS information for the application traffic in step 1 of 9.2.3.1 and 9.3.3.3 should restricted to indicate required data rate & latency.

8.1.1.1.3.3	SEALDD enabled application data transmission for Low latency (LC)

The SEALDD enabled regular data transmission defined in clause 9.2.2.2	of 3GPP TS 23.433[2] can be used with the following changes:

- The term Sdd_RegularTransmission is changed to Sdd_LCTransmission.

- The QoS information for the application traffic in step 1 of 9.2.3.1 and 9.3.3.3 should restricted to indicate required latency.

* * * End of the Change * * * *