3GPP TSG-SA WG6 Meeting #66	S6-251227

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251xxx)

Source:	Nokia, Samsung

Title:	Discussion on handling of RNAA and user consent in 3GPP

Agenda Item:	11

Contact:	Niranth Amogh (niranth.amogh@nokia.com)
Sapan Shah (sapan.shah@nokia.com)
Basavaraj Pattan (basavarajjp@samsung.com)

## 1.	Introduction

In SA6#65, two study item proposals were discussed aiming to enhance CAPIF.

a)	S6-250538 – From Nokia [1] (S6-250252 [2] – From Samsung was merged into Nokia’s proposal)

b)	S6-250561 – From Ericsson [3]

Both a) and b) proposals were postponed from the last meeting.

The main aim of this discussion paper is to clarify the scope of work that should be undertaken in SA6 and proposes a way forward.

## 2.	RNAA and User Consent work in 3GPP

This section provides a summary of the work done till now in different SA working groups of 3GPP on RNAA and User Consent.

### 2.1	SA6

In 3GPP TS 23.222 [4], SA6 specified architecture, and methods to support Resource owner-aware Northbound API Access (RNAA). RNAA is an OAuth 2.0-based authorization mechanism for CAPIF and enables API invokers to have authorized access to resources of a resource owner provided by service APIs offered by the API exposing function. An Authorization function was designed to support obtaining the authorization for API invoker(s) to access resources of a resource owner exposed via service API(s). Currently, the entity Authorization function is enabled to be supported by CCF. The details of the security aspects for CAPIF supporting RNAA is specified in 3GPP TS 33.122 [5] by SA3.

Observation #1: The resource owner authorization work has impacts to both SA6 and SA3 working groups.

NOTE:	Based on LS S6-245326 [7] from SA3, the term “user consent” is solely used by SA3 and hence SA6 has decided to use the term “resource authorization” instead of “user consent”.

The LS S6-245326 [7] from SA3 also gave an action to SA6 to consult with SA3 on matters of user consent and usage of CAPIF RNAA.

Observation #2: SA6 shall not decide on matters related to user consent and usage of CAPIF RNAA, without consulting SA3.

### 2.2	SA3

#### 2.2.1	User consent

In 3GPP TS 33.501 [6], SA3 specified in Annex V, describing the generic security requirements and procedures to support user consent enforcement in 3GPP services. The LS S6-245326 [7], further clarifies about user consent as follows:

"The term “user consent” is solely used by SA3 to refer to the framework defined in the Annex V in TS 33.501. It is for providing technical means to manage subscriber permissions at the operator domain such that Network Functions of the 3GPP system can request or can be provisioned/configured with subscriber-related data in accordance with local laws and regulations"

Observation #3: User consent is applicable to manage subscriber permission at operator domain.

Further, The Annex V also provides following requirements (non exclusive):

The UDM shall support the following services related to the user consent. - Retrieval of user consent parameters. - Notification of user consent parameters change. - The user consent parameters shall be stored in the UDM/UDR as subscription data. - The user consent parameters shall be bound to a SUPI/GPSI.

Observation #4: The SA3 defined user consent involves parameters related to core network defined user identities (i.e. SUPI) and uses core network defined network function (i.e. UDM/UDR).

Observation #5: The user consent aspects as defined by SA3 involves entities defined by SA2.

#### 2.2.2	Resource Owner Authorization

In 3GPP TS 33.122 [5], SA3 specifies functional models to support security for Common API Framework (CAPIF) and Resource owner-aware Northbound API Access (RNAA). The specification provides methods to get authorization from resource owners to provide access to API invoker requesting access to the resources stored at (or available at) resource server and owned by resource owner. The LS S6-245326 [7], further clarifies about resource owner authorization as follows:

"RNAA addresses authorization by the resource owner, i.e. getting permission from the resource owner for exposing any type of information or allowing access to its resources by an API to execute update, create, delete resources operations. The resource owner can be an end user, subscription owner, enterprise, etc."

Observation #6: The resource owner authorization aspects as defined by SA3 involves CAPIF entities defined by SA6.

## 3.	Regarding Resource owner Function (RoF)

As specified in clause 6.3.8 of 3GPP TS 23.222 [4], the resource owner function is responsible for interactions with the resource owner in a similar way to the resource owner's user agent shown in clause 4.1 of IETF RFC 6749 [8].

"The resource owner function is responsible for interactions with the resource owner in a similar way to the resource owner's user agent shown in clause 4.1 of IETF RFC 6749."

For Authorization Code Flow, redirections is used via user agent only. So, RoF is same as user agent and as long as user agent is present in UE - it can be mapped to RoF of SA6.

## 4. 	Stage-3 excerpts to show the difference between RNAA authorization information and User Consent information

As per 3GPP TS 29.222 [9], the authorization information related to RNAA that is provided to API invoker by the Authorization function is an Authorization Code (which is in a form of String) and subsequently access token.

User consent information is specified in 3GPP TS 29.503 [10] as follows:

##### 6.1.6.2.85 Type: UcSubscriptionData Table 6.1.6.2.85-1: Definition of type UcSubscriptionData Attribute name; Data type; P; Cardinality; Description. userConsentPerPurposeList; map(UserConsent); O; 1..N; A map user consent (list of key-value pairs where UcPurpose serves as key; see clause 6.1.6.3.20).. ##### 6.1.6.3.20 Enumeration: UcPurpose Table 6.1.6.3.20-1: Enumeration UcPurpose Enumeration value; Description. "ANALYTICS"; User consent for analytics.. "MODEL_TRAINING"; User consent for model training.. "NW_CAP_EXPOSURE"; User consent for network capability exposure.. "EDGEAPP_UE_LOCATION"; User consent for the manipulation of UE information for the purpose of UE Location retrieval by the EDGEAPP EAS entity.. ##### 6.1.6.3.21 Enumeration: UserConsent Table 6.1.6.3.21-1: Enumeration UserConsent Enumeration value; Description. "CONSENT_NOT_GIVEN"; It shall indicate the user hasn't given his consent.. "CONSENT_GIVEN"; It shall indicate the user has given his consent..

Observation #7: The RNAA authorization information and User Consent information stage-3 formats are different.

In an example scenario, if the API invoker (Edge Application Server (EAS)) invokes Eees_UELocation API (clause 8.6.2.2 of 3GPP TS 23.558) on Edge Enabler Server, the EES (acting as an API exposing function for the API invoker and as consumer of the NWDAF services) may invoke NWDAF (Network Data Analytics Function) services as specified in clause 8.10.3 of 3GPP TS 23.558, to fetch UE expected behaviour analytics. In this scenario, the check for resource owner authorization for the consumption of Eees_UELocation API by EES is done as per CAPIF RNAA procedures specified in 3GPP TS 23.222, 3GPP TS 33.122. And, the check for user consent for the consumption of NWDAF services by EES, is checked by NWDAF, using the user consent stored in UDM. In such scenarios it is possible that the API invoker obtains the authorization successfully from resource owner (end user or subscriber) to invoke the northbound service APIs using CAPIF RNAA procedures, then invokes the northbound service APIs successfully. But, there is no user consent (with the required purpose of data processing) available at the UDM, for the related downstream service request to 5G core network NFs. In such scenarios, the API Invoker’s northbound service API request cannot be fulfilled, although the API invoker had obtained the authorization from the resource owner using CAPIF RNAA procedures. Such conflict of authorization at CAPIF and user consent at UDM could lead to service unavailability, non-fulfilment of the required services, potential security flaws due to inconsistent information.

Observation #8: Address relationship of resource owner authorization in CAPIF and user consent in UDM and resolve any conflict for RNAA when CAPIF RNAA is involved for resource owner authorization.

## 5.	Difference between SA6 SID proposals in FS_CAPIF_Ph4 and FS_COCOA

FS_CAPIF_Ph4 (S6-250538) proposes to undertake study on CAPIF enhancements including RNAA. The aim in FS_CAPIF_Ph4 is to analyse and address relationship of resource owner authorization in CAPIF and user consent in UDM and resolve any conflict for RNAA when CAPIF RNAA is involved for resource owner authorization.

FS_COCOA (S6-250561) proposes to study a standalone new user consent architecture which can enable a generic user consent management (e.g. user consent check, user consent revocation) by performing analysis of the user consent and privacy architectures specified by industry bodies like GSMA. However, it is not clear from the proposal how the backward compatibility is expected to be handled as User Consent and RNAA exists in 3GPP specifications since atleast Rel-18.

RNAA and User consent are identified to be different mechanisms as per 3GPP communication to GSMA (SP-241934).

Observation #9: RNAA and User consent are different mechanisms and RNAA aspects are proposed to be studied in FS_CAPIF_4 and User consent aspects are proposed to be studied in FS_COCOA.

Observation #10: Proposal in FS_COCOA is incomplete since it does not provide the proposal on how the backward compatibility is expected to be handled.

## 6.	Summary and Way forward proposal

It is clear from all observations (1 to 10) that the user consent related work is in the realm of SA3 that involves engaging multiple groups in 3GPP. Hence, it is proposed that:

Proposal #1: the study on requirements for user consent management and its architecture is best suitable for SA3 working group.

Proposal #2: SA6 should continue to focus on RNAA aspects of CAPIF in its R20 study proposal.

Proposal #3: SA6 should set up coordination with SA3 to assess which aspects of user consent can impact CAPIF based on SA3’s study on holistic user consent considerations for 3GPP services.

Proposal #4: The way forward consideration is that SA6 agrees on FS_CAPIF_Ph4 with RNAA scope and

Proposal #5: SA6 shall not decide on matters related to user consent until SA3 has made progress.

## 7.	References

[1]	S6-250538:	"New study item on CAPIF Phase 4".

[2]	S6-250252:	"Study on enhancement to CAPIF Phase 4".

[3]	S6-250561:	"New SID on Consolidated Architecture for User Consent Management for applications".

[4]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[5]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[6]	3GPP TS 33.501:	"Security architecture and procedures for 5G system".

[7]	S6-245326: "Reply LS to SA6 on obtaining user consent for the exposure of User Sensitive Information".

[8]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[9]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs".

[10]	3GPP TS 29.503:	"5G System; Unified Data Management Services; Stage 3".