3GPP TSG-SA WG6 Meeting #66	S6-251233

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0033 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial map participation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-App |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified in clause 9.3.1.2, unpon receiving the request to produce spatial map, if the SEAL SM server does not have sufficient information to produce spatial map then it determins to wait to get more sensor data. However, it may not be always prossible that the SM data source which can provide the processes sensor data is currently active and providing real time data to the SEAL SM server which can help the SEAL SM sever to produce the spatial map. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new procedure for SEAL SM server to request SM data source to participate in producing the spatial map |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SEAL SM server may never get requried infomration to compte producing spatial map. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.x (NEW), 9.5.x.1 (NEW), 9.5.x.2 (NEW), 9.5.x.3 (NEW), 9.5.x.3.1 (NEW), 9.5.x.3.2 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.5.x	SM data source participation in spatial map

#### 9.5.x.1	General

As specified in clause 9.3.1.2, unpon receiving the request to produce spatial map, if the SEAL SM server does not have sufficient information to produce spatial map then it determins to wait to get more sensor data. However, it may not be always prossible that the SM data source which can provide the processes sensor data is currently providing real time data to the SEAL SM server which can help the SEAL SM sever to produce the spatial map. The SM data source participate procedure allows the SEAL SM server to request SM data source to participate in creation of spatial map by providing processed sensor data.

#### 9.5.x.2	Procedure

Figure 9.5.x.2-1 depicts the procedure for SEAL SM server to request SEAL SM client to participate in creating the spatial map.

Pre-conditions:

1.	The SM data sources has registered with the SEAL SM server as specified in clause 9.5.2.2;

2.	The SM server has received request to produce spatial map of the area of interest for which it does not have sufficient information.

Figure 9.5.x.2-1: Procedure to request SM data source participation in spatial map

1.	The requestor (e.g, SEAL SM server) sends a Spatial Map participation request to the SEAL SM client. The request includes a Spatial Map description, SM specific participation criteria like required location to serve, availability duration, required battery capacity, requried compute capacity.

2.	Upon receiving the request, the SEAL SM client processes the participation criteria and performs the checks for each required criteria. If all required criteria are met, the SEAL SM client generates the success response. Otherwise, the SEAL SM client generates the failure response.

NOTE:	The SEAL SM client can interact with the user to obtain required permission by presenting the information related to spatial map.

#### 9.5.x.3	Information flows

##### 9.5.x.3.1	Spatial map participation request

Table 9.5.x.3.1-1 describes information elements in the Spatial map participation request from the SEAL SM server to the SEAL SM client.

Table 9.5.x.3.1-1: SM data source discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Spatial Map description | M | Provides details about the required spatial map |
| > Area of interest | M | Three-dimensional area of interest |
| SM specific participation criteria | O | Provides details on requirement for the participation |
| > required battery | O | Indicates required battery to participate |
| > required battery | O | Indicates required compute to participate |
| > availability duration | O | Indicates duration time required to participate |

##### 9.5.x.3.2	Spatial map participation response

Table 9.5.x.3.2-1 describes information elements in the Spatial map participation response from the SEAL SM client to the SEAL SM server.

Table 9.5.x.3.2-1: SM data source discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the discovery request was successful |
| Failure response | O (NOTE) | Indicates that the request failed |
| > Cause | O | Indicates the cause of the failure |
| NOTE: At least one of the information elements shall be provided. |  |  |

* * * End of Change * * * *