3GPP TSG-SA WG6 Meeting #66	S6-251234

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251xxx)

Source:	Nokia

Title:	Pseudo-CR on update to adoption solution#5: Target application use case of SEAL services

Spec:	3GPP TR 23.700-35

Agenda item:	9.3

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@nokia.com)

1. Introduction

This pCR enhances Adoption Solution#5: Target application use case of SEAL services.

2. Reason for Change

As specified in the Generic gap #4: “Linking SEAL services to target application use cases” – it is required to provide use cases for each SEAL service separately. The pCR proposes use cases for Spatial anchors, Spatial map and Digital asset services.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 (v0.3.0)

* * * First Change * * * *

### 8.2.5	Adoption Solution#5: Target application use case of SEAL services

#### 8.2.5.1	Description

This solution is to address the Generic gap #4.

#### 8.2.5.2	Target application use cases for SEAL services

##### 8.2.5.2.1	SEALDD service

This solution provide clarification on SEALDD service linked to target application use case, and provide analysis on SEALDD mechanism and CDN mechanism.

SEALDD provides some enhanced content delivery functions, which fully use of 5G network to perform content delivery, thus the SEALDD can provide content distribution with QoS guarantee which provide better service performance to third party.

In the real business deployment, CDN can be deployed by PLMN operators, or third party. Existing widely deployed CDN server use OTT mechanism. It cannot provide QoS guarantee benefitting from the capabilities of 5G/5GA enhancement for XR/Video/URLLC.

PLMN operators or third party CDN providers may support SEALDD functions in their system to offer CDN services for further enhancing the data delivering performance.

* * * Next Change * * * *

##### 8.2.5.2.x	Spatial anchors

A spatial anchor is an association between space and service information. The Spatial anchors SEAL service offers spatial anchors related capabilities (i.e. CRUDS) to one or more vertical applications. Spatial anchors enable the authorized users to place digital information at specific locations.

Spatial anchors can be used in many ways:

-	To show the path to the destination without obstructing the user's perception of their proximity (as described in clause 5.1 of 3GPP TR 22.856 [r22856])

-	To show the content that may be relevant to the user (i.e. potential shopper) (as described in clause 5.1 of 3GPP TR 22.856 [r22856])

-	To provide personalized message (like the food of a restaurant liked by the user) (as described in clause 5.1 of 3GPP TR 22.856 [r22856])

Figure 8.2.5.2.x -1: Localized Mobile Metaverse Services offering relevant information (from clause 5.1 of 3GPP TR 22.856 [r22856])

-	To display extensive information about the products of a shop (as described in clause 5.4 of 3GPP TR 22.856 [r22856])

Detailed use case it captured in clause 5.1 and clause 5.5 of 3GPP TR 22.856 [r22856].

##### 8.2.5.2.y	Spatial map

The Spatial map SEAL service offers spatial mapping and localization capabilities.

-	Spatial mapping is constructing or updating a map of an unknown location.

-	Localization is tracking an object to identify its location and orientation over time.

For spatial mapping service, sensing data gathered transparently is processed in order to identify the static and transient forms.

Spatial map service can be used in many ways (as described in clause 5.5 of 3GPP TR 22.856 [r22856]):

- 	A government conducting a digital city project can build a 3D spatial map of outdoor environment of an entire city or public spaces such as outdoor parks or indoor offices of their government building.

- 	A navigation service provider (or Spatial Localization Service) can build a 3D spatial map of outdoor environment of entire roads or public spaces. An operator’s partner or operator also can build a 3D spatial map of outdoor environment.

-	A customer wanting mapping of their indoor environment, e.g. the interior of a commercial space such as the cheese shop as described in 5.4.

Detailed use case it captured in clause 5.5 of 3GPP TR 22.856 [r22856].

##### 8.2.5.2.z	Digital asset

The SEAL digital assets service enables management and usage of digital assets in a secure and controllable way. Examples of digital assets include digital representation (avatar), software licenses, gift certificates, tokens and files (e.g., music files) that have been purchased.

Digital asset service can be used in many ways:

-	Avatar of a human can be used in collaborative and concurrent engineering in an XR meeting (clause 5.3 of 3GPP TR 22.856 [r22856])

-	To play immersive games and to attend digital live concert (clause 5.6 of 3GPP TR 22.856 [r22856])

-	To predict remote user’s action (clause 5.9 of 3GPP TR 22.856 [r22856])

-	To operate patient in critical heath care (clause 5.10 of 3GPP TR 22.856 [r22856])

-	To have immersive calling experience (clause 5.11 of 3GPP TR 22.856 [r22856])

* * * Next Change * * * *

<Proposed change in revision marks>