3GPP TSG-SA WG6 Meeting #66	S6-251243

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-250xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.958 | CR | 0004 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EDGEAPP TR EN resolution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_EXT |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.958 has the following EN: Editor's note: Alignment of EDGEAPP and GSMA OP for ECSP Management System is FFS and requires inputs from SA5. The CR proposes to address the above EN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Figures and text aligned to ECSP management from SA5 as per the LS response received from SA5 (S6-240014, S6-240036). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN will continue to remain unaddressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 6.2	Relationship between EDGEAPP architecture and GSMA OPG reference architecture

Figure 6.2-1 and Figure 6.2-2 illustrates the relationship between EDGEAPP architecture and GSMA OPG reference architecture [3].

Figure 6.2-1: Relationship between EDGEAPP architecture and GSMA OPG reference architecture without Federation

Figure 6.2-2: Relationship between EDGEAPP architecture and GSMA OPG reference architecture with Federation

EDGE-1 and EDGE-4 reference points can support similar function(s) as OP's User-Network interface (UNI), providing the Edge Enabler Client (corresponding to Edge/User Client in OP) with the information required to access the edge services. EDGE-1/EDGE-4 neither impact nor overlap with other existing 3GPP interfaces between the UE and the network, catering to the OP's requirements on UNI.

EDGE-2 and EDGE-8 reference points can support similar function(s) as OP's Southbound interface (SBI), through which the edge enabler layer (corresponding to the operator platform) access the 3GPP network capabilities and services (e.g. SCEF/NEF). Specifically, EDGE-2 and EDGE-8 cater to the requirements of the SBI-NetworkResource interface. ECSP management system utilizes ETSI NFV MANO as specified in 3GPP TS 28.538 [8] and caters to the requirements of OP's SBI-CloudResource interface over Os-Ma-nfvo reference point. The charging function is specified in 3GPP TS 32.257 [18].

EDGE-3 reference point can support similar function(s) as OP's Northbound interface (NBI), exposing the capabilities of Edge Enabler Server to the Edge Application Servers (EAS) hosted on the edge. OP's NBI also expands capabilities exposure to ASPs, for example to on-board applications to be deployed as EASs based on specific criteria.

EDGE-9 and EDGE-10 reference point can support similar function(s) as OP's East/Westbound interface (E/WBI), allowing the edge enabler layer to interact within and beyond its domains e.g., between operator platforms. OP's E/WBI focuses on use cases like user and application roaming or resource sharing across domains. The ECSP management system as specified in 3GPP TS 28.538 [8] caters to the requirements of OP's EWBI enabling the interactions between ECSP management systems of a Leading Operator (LO) and a Partner Operator (PO) to support use cases for Federation relationship management and Federation Management.

* * * Next Change * * * *

AnnexA:
Mapping roles between EDGEAPP architecture and GSMA OPG architecture

GSMA Operator Platform Group has published their edge requirements in Operator Platform Telco Edge Requirements [3].

Clause 3 of Operator Platform Telco Edge Requirements [3] provides the OP Roles and Interfaces Reference Architecture. Clause 3.2 of Operator Platform Telco Edge Requirements [3] defines roles along with their key functions.

The three main roles in the GSMA OP architecture and their mapping with EDGEAPP architecture entity is as follows:

1)	Capabilities Exposure Role: It enables an Application Provider to operate their applications. Table A-1shows the scenarios enabled by Capabilities exposure role as indicated in Operator Platform Telco Edge Requirements [3] and their mapping with 3GPP EDGEAPP architecture.

Table  A-1: Capabilities exposure role

| Interface | Scenario | Description | Mapping to 3GPP EDGEAPP |
| --- | --- | --- | --- |
| NBI | Edge Cloud Infrastructure Endpoint Exposure | The Application Provider uses an authenticated and authorized endpoint to carry out scenarios involving application instances on edge clouds; | ASP interacts with 3GPP Management system using Management Service for Edge Computing, as defined in Clause 8 of 3GPP TS 28.538, to carry out scenarios involving application instances on Edge Data Network. |
|  | Application Onboarding | The Application Provider uses the NBI to provide application images and metadata to the OP Federation Broker/Manager Role | Application instances are managed by 3GPP Management System as VNFs. VNF on boarding is defined in ETSI GS NFV IFA-013. This may require additional functionality to be defined between ASP and ECSP Management System. |
|  | Application Metadata/Manifest Submission | The Application Provider uses the NBI and the metadata model to submit application metadata to the OP and follows defined procedures to extend the metadata model specification |  |
|  | Application CI/CD Management DevOps | The Application Provider integrates the CI/CD framework used to create an application with the OP via NBI APIs (which implies an integration between a CI/CD framework and Application Onboarding and Lifecycle Management) |  |
|  | Application Lifecycle Management | The Application Provider observes and changes the operational state of application instances, including the geographical/network extent of the OP on which application instances may run; | Clause “7.1 Lifecycle management” of 3GPP TS 28.538. |
|  | Application Resource Consumption Monitoring | The Application Provider observes resource consumption of application instances, using the resource data model | Defined in clause “8.2.1 EAS performance assurance” of 3GPP TS 28.538. The virtual resource usage can be monitored by collecting the respective performance measurements as defined in clause 8.2.1.2 of 3GPP TS 28.538. |
|  | Edge Cloud Resource Catalogue exposure | The Application Provider inventories edge cloud resources nominally available to application instances |  |
|  | Network Capabilities exposure | The Application Provider inventories network capabilities, like Network Analytics, nominally available to application instances | EES (as per clause 8.7.3) |

2)	Service Resource Manager Role: The Service Resource Manager role in the OP is responsible for managing Cloud and Network resources from the Edge Cloud(s) via the SBI and UNI interfaces. Table A-2shows typical scenarios enabled by the Service Resource Manager role towards the different interfaces as indicated in Operator Platform Telco Edge Requirements [3] and their mapping with 3GPP EDGEAPP architecture.

Table  A-2: Service Resource Manager Role

| Interface | Scenario | Mapping to 3GPP EDGEAPP |
| --- | --- | --- |
| SBI | Inventory, Allocation and Monitoring of Compute resources from Edge Cloud Infrastructure via the Southbound Interface – Cloud Resources (SBI-CR); | Application instances are managed by 3GPP Management System as VNFs. VNF resource orchestration, including reservation, is defined in ETSI NFV IFA013 |
|  | Orchestration of Application instances on the Edge Cloud Infrastructure via the SBI-CR interface |  |
|  | Cloud resource reservation managed by the OP |  |
|  | Configuring UE traffic management policies to accomplish the application's requirements, or the UE's IP address shall be maintained |  |
|  | Exposure of usage and monitoring information to operator's charging engine via the Southbound Interface – Charging functions (SBI-CHF) to enable operators to charge for the OP's services. | The charging for edge is defined in 3GPP TS 32.257. |
| Southbound Interface – Network Resources (SBI-NR) | Fetch Cloudlet locations based on the mobile network data-plane breakout location |  |
|  | Subscribe and receive notifications on UE Mobility events from the network to assist applications | UE mobility events are exposed by NEF (TS 23.502) and EES and ECS subscribes to receive UE mobility notification to assist applications for ACR |
|  | Configure traffic steering in the Mobile Network towards Applications orchestrated in Edge Clouds | NEF exposes TrafficInfluence service (TS 23.502), EES uses the TrafficInfluence service to steer the traffic (related to traffic influence) |
|  | Receive statistics/analytics, e.g. to influence Application placement or mobility decisions |  |
|  | Receive information related to the network capabilities, such as QoS, policy, network information, etc | NEF exposes events related to network capabilities, EES and ECS subscribes to receive network capability related information. |
| UNI | Application Instantiation/Termination, e.g. based on triggers from the UNI |  |
|  | Application Endpoint exposure towards User Clients (UC) via the UNI | EES (clause 8.4.2 and clause 8.5) |
|  | Application Placement decisions, e.g. based on measurements/triggers from the UNI |  |
| SBI-OAM | Management of network slice lifecycle. |  |

3)	Federation Broker and Federation Manager Roles: The Federation Broker and Manager Roles in the OP are responsible for interfacing with other OPs via the East-West Bound Interface. Table A-3 shows typical scenarios enabled by the Federation Broker and Federation Manager Roles as indicated in Operator Platform Telco Edge Requirements [3] and their mapping with 3GPP EDGEAPP architecture.

Table  A-3: Federation Broker and Federation Manager Roles

| Interface | Scenario | Mapping to 3GPP EDGEAPP |
| --- | --- | --- |
| EWBI | Federation Interconnection Management | Clause “7.5.2 Edge Federation Establishment” of 3GPP TS 28.538. |
|  | Edge Cloud Resource Exposure and Monitoring towards partner OPs |  |
|  | Network and Analytics Capabilities Exposure towards partner OPs |  |
|  | Application Images and Application metadata transfer towards partner OPs | Application instances are managed by 3GPP Management System as VNFs. Application Image and metadata management is considered part of VNF onboarding. VNF onboarding is defined in ETSI NFV IFA013. |
|  | Application Instantiation/Termination towards partner OPs | Clause “7.5.3 Edge Federation Establishment” of 3GPP TS 28.538. |
|  | Application Monitoring towards partner OPs |  |
|  | Edge Cloud Resource Catalogue exposure | EES (Exposing EAS profile) (See NOTE 1) |
|  | Service Availability in visited networks | ECS (Table 8.3.3.3.3-2 and clause 8.17.2.4 of TS 23.558) (See NOTE 2) |
| NOTE 1: Edge Cloud Resource Catalogue may contain information other than EAS profile which is within scope of SA5. NOTE 2: EASIDs are considered as available services in partner ECSP and are exchanged between ECSs of home ECSP and partner ECSP. |  |  |

Based on the mapping tables in this clause, EES and ECS of 3GPP EDGEAPP architecture can be mapped to take responsibilities of the some roles as defined in GSMA OPG reference architecture.

* * * End of Changes * * * *