3GPP TSG-SA WG6 Meeting #66	S6-251245

Goteborg, Sweden, 07-11th April 2025	(revision of S6-250xxx)

Source:	Ericsson

Title:	Solution for Application traffic behaviour and SEALDD data delivery optimization Key Issue

Spec:	3GPP TR 23.700-52 v0.0.0

Agenda item:	9.7

Document for:	Approval

Contact:	Ashish S Sharma, ashish.sanjay.sharma@ericsson.com

1. Introduction

The pCR introduces a solution for Key issue based on the point 3 mentioned in the objectives of SID SP-250382

2. Reason for Change

Point 3: Data delivery for service/application with the following characteristic: the application layer interactions are non-continuous, each of the interactions is an independent application transaction.

The Key issue is proposed in the pCR S6-251078.

The application traffic behaves differently at various times. The SEALDD server is not aware of the application's traffic pattern. The proposed solution allows the SEALDD server to understand the SEALDD traffic patterns at different time intervals.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-52 v0.0.0

* * * First Change * * * *

## X.Y	Solution#Y: SEALDD enabled application traffic behavior characterization procedure

### X.Y.1	General

The SEALDD server handles the user plane delivery of the VAL traffic over SEALDD-UU. Due to the dynamic behavior of the application traffic and time-varying conditions in the network, the SEALDD may need to optimize its user plane transmission. The optimization could include establishing another connection, using different access networks, adjusting QoS parameters, modifying the SEALDD policy parameters, etc. This requires the SEALDD server to tune its transmission service at the SEALDD layer and also configure the underlying network accordingly. Therefore, the SEALDD server needs to be aware of the application traffic behavior and its correlation with underlying network conditions. To enable the SEALDD server to find out how the app traffic and network conditions are behaving at different times, an application traffic behavior characterization procedure is introduced.

The introduced ADAE traffic characterization service performs the application traffic characterization to build a traffic behavior model and provides an abstraction to the SEALDD server to query the traffic activity pattern and traffic activity period. This service provides the traffic activity period(s) and a traffic activity patterns(e.g., intervals of the peak traffic period, data volumes exchanged, latency etc). This enables the SEALDD server to understand the different traffic patterns at different times with the time-varying network conditions.

* * * Next Change * * * *

#### X.Y.2	SEALDD enabled application traffic behavior characterization procedure

Figure X.Y.2-1 illustrates the procedure SEALDD enabled application traffic behavior characterization procedure from the SEALDD server to the ADAE server.

Figure X.Y.2-1: SEALDD enabled application traffic behaviour characterization

- The SEALDD server sends the SEALDD enabled application traffic behavior characterization subscription request to ADAE server. The request contains the list of VAL service ID, VAL server ID, list of VAL UE identifiers, network slice identifier, Multi-modal Flow ID(s), SEALDD flow ID(s) and associated 5-tuple traffic descriptors of the SEALD-UU traffic.

The request also contains SEALDD traffic behavior query operations like traffic activity period, traffic activity criteria, and traffic activity pattern(e.g., peak activity traffic period, low activity traffic period, bursty trafficperiod).

2.	Upon receiving the request, the ADAE server performs an authorization check and responds with a SEALDD enabled application traffic behavior characterization subscription response.

3.	The ADAE server subscribes to NWDAF analytics services related to the user plane traffic like  DN performance, PFD determination, data volume, PDU session analytics mentioned in 3GPP TS 23.288 clause 6.14, clause 6.16, clause 6.18, clause 6.20, clause 6.23 of TS 23.288. The ADAE server uses the traffic descriptors and UE identifiers from the request message to fetch the NWDAF analytics.

The ADAE server also subscribes to the Network Performance analytics NWDAF service described in clause 6.6 TS 23.288 for the number of VAL UE(s) mentioned in the request message.

The ADAE also subscribes to the SEALDD server for data transmission quality measurement reports related to the requested SEALDD traffic as described in TS 23.433 clause 9.7.

The ADAE server obtains application performance analytics procedure for VAL server and VAL UE session performance analytics data as described in the TS 23.436 clause 8.2

Based on the processing of the NWDAF analytics and SEALDD server transmission quality measurement reports, the ADAE server performs the characterization of the SEALDD enabled application traffic behavior model. The ADAE server also correlates or links the traffic behavior model with the network performance analytics data. The ADAE server may use statistical or machine learning tools to build the traffic behavior model. It may also use the VAL service ID to classify the different traffic as per the VAL service.

NOTE: The ADAE server constantly updates the SEALDD-enabled application traffic behavior model with the analytics data.

4.	The ADAE server performs the requested traffic behavior operation on the SEALDD enabled application traffic behavior model to derive the traffic behavior insights and provide the notification response. The analytics server may use the traffic activity criteria associated with the traffic activity pattern in the request or use its local criteria if not provided. The notification response includes VAL Service ID, SEALDD flow ID, VAL UE ID, and SEALDD traffic behavior pattern result, data volumes exchanged, throughput, latency and jitter. The response provides current and predicted values with the confidence interval.

* * * End of Changes * * * *