3GPP TSG-SA WG6 Meeting #66	S6-251255

Gothenburg, Sweden 07th – 11th April 2025	(revision of S6-251057)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0380 | rev | 1 | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 7.4.2.6.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 19.03.2025 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 7.4.2.6.2 describes the procedure of ‘Group standalone short data service using media plane’. Therefore it uses Information Flow ‘MCData group session standalone data request’. NOTE 3 just mentions ‘MCData group standalone data request’ which is wrong. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of NOTE 3 in 7.4.2.6.2 Correction of NOTE 1 in 7.4.2.5.2 Allign IF 7.4.2.1.10 and 7.4.2.1.11 with those for signalling plane (7.4.2.1.8, 7.4.2.1.9) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Correction is required for clarification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2.1.10, 7.4.2.1.11, 7.4.2.5.2, 7.4.2.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 7.4.2.1.10	MCData group session standalone data request (MCData client – MCData server)

Table 7.4.2.1.10-1 describes the information flow for the MCData group session standalone data request (in subclause 7.4.2.6.2) sent from the MCData client to the MCData server.

Table 7.4.2.1.10-1: MCData group session standalone data request (MCData client – MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Transaction type | M | Standalone transaction |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| MCData ID list (see NOTE 4) | O | The specified MCData users who should send a disposition notification message. |
| Payload Destination Type | M | Indicates whether the payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE 3) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI, attached data hosts) |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| SDP offer | M | Media parameters offered |
| Requested priority | O | Application priority level requested for this communication session |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. NOTE 3: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption or IP data in IP connectivity sessions are for data host consumption. NOTE 4 If Disposition Type IE is not present, this IE shall not be present. If Disposition Type IE is present but this IE is not, which indicates that all receivers shall respond with disposition notification message. |  |  |

# * * * * Second change * * * *

##### 7.4.2.1.11	MCData group session standalone data request (MCData server – MCData client)

Table 7.4.2.1.11-1 describes the information flow for the MCData group session standalone data request (in subclause 7.4.2.6.2) sent from the MCData server to the MCData client.

Table 7.4.2.1.11-1: MCData group session standalone data request (MCData server – MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| MCData ID | M | The identity of the MCData user towards which the data is sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Transaction type | M | Standalone transaction |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| MCData ID list (see NOTE 4) | O | The specified MCData users who should send disposition notification message. |
| Payload Destination Type | M | Indicates whether the payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE 3) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI, attached data hosts) |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| SDP offer | M | Media parameters offered |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. NOTE 3: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption or IP data in IP connectivity sessions are for data host consumption. NOTE 4: If Disposition Type IE is not present, this IE shall not be present. If Disposition Type IE is present but this IE is not, which indicates that all receivers shall respond with disposition notification message. |  |  |

# * * * * Third change * * * *

##### 7.4.2.5.2	Procedure

The procedure in figure 7.4.2.5.2-1 describes the case where an MCData user is initiating group standalone MCData data communication with or without disposition request, to a group.

Pre-conditions:

1.	MCData users on MCData clients 1 to n belong to the same group and are already registered for receiving MCData service and affiliated.

2.	Optionally, the MCData client may have activated functional alias to be used.

3.	The MCData server may have subscribed to the MCData functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.4.2.5.2-1: Group standalone SDS using signalling control plane

1.	The user at MCData client 1 initiates an SDS data transfer to multiple MCData users selecting a pre-configured group (identified by MCData group ID) and optionally particular members from that group.

2.	MCData client 1 sends a MCData group standalone data request towards the MCData server. The MCData group data request contains MCData group ID as selected by the user at MCData client 1. The MCData group standalone data request contains conversation identifier for message thread indication. The MCData session data request may include additional implementation specific information in the application metadata container. The MCData group standalone data request may contain disposition request if indicated by the user at MCData client 1. MCData user at MCData client 1 may include a functional alias within the SDS data transfer.

If the MCData user at MCData client 1 initiates an MCData emergency short data service communication or the MCData emergency state is already set for the MCData client 1 (due to a previously triggered MCData emergency alert):

i)	the MCData group standalone data request shall contain an emergency indicator;

ii)	the MCData group standalone data request shall set an alert indicator if configured to send an MCData emergency alert while initiating an MCData standalone data request for the emergency short data service communication;

iii)	if the MCData emergency state is not set already, MCData client 1 sets its MCData emergency state. The MCData emergency state is retained until explicitly cancelled; and

iv)	once an MCData emergency communication has been initiated, the MCData group is considered to be in an in-progress emergency state until cancelled.

If the MCData user at MCData client 1 initiates an MCData imminent peril short data service communication:

i)	the MCData group standalone data request shall contain imminent peril indicator; and

ii)	once an MCData imminent peril communication has been initiated, the MCData group is considered to be in an in-progress imminent peril state until cancelled.

2a.	If either emergency indicator or imminent peril indicator is present in the received MCData group standalone data request, the MCData server implicitly affiliates MCData client 1 to the MCData group if the client is not already affiliated.

3.	MCData server checks whether the MCData user at MCData client 1 is authorized to send MCData group standalone data request. The MCData server resolves the MCData group ID to determine the members of that group and their affiliation status, based on the information from the group management server. The MCData server also checks whether any policy is to be asserted to limit certain types of message or content to certain members due, for example, to location or user privilege or affiliation. MCData server also verifies whether the provided functional alias, if present, can be used and has been activated for the user.

i)	If an emergency indicator is present in the received MCData group standalone data request and if the MCData group is not in the in-progress emergency state, the MCData group is considered to be in the in-progress emergency state until cancelled; and

ii)	If an imminent peril indicator is present in the received MCData group standalone data request and if the MCData group is not in the in-progress imminent peril state, the MCData group is considered to be in the in-progress imminent peril state until cancelled.

4.	MCData server initiates the MCData group standalone data request towards each MCData client determined in Step 3. The MCData ID list shall not be included in a unicast downlink delivery to an individual MCData client. The Disposition Type IE shall not be included in a unicast downlink delivery to MCData clients who are not in the MCData ID list in step 2. The MCData group standalone data request towards each MCData client contains:

i)	an emergency indicator, if it is present in the received MCData group standalone data request from the MCData client 1;

ii)	an imminent peril indicator, if it is present in the received MCData group standalone data request from the MCData client 1; and

iii)	an alert indicator, if requested to initiate an emergency alert in the received MCData group standalone data request from the MCData client 1.

5.	If the payload is for MCData user consumption (e.g. is not application data, is not command instructions, etc.) then the MCData user of MCData clients 2 to n may be notified. Otherwise if the payload is not for MCData user consumption, then the MCData user of MCData clients 2 to n shall not be notified. The action taken when the payload contains application data or command instructions are specific based on the contents of the payload. Payload content received by MCData client 2 which is addressed to a known local non-MCData application that is not yet running shall cause the MCData client 2 to start the local non-MCData application (i.e., remote start application) and shall pass the payload content to the just started application.

NOTE:	On receiving MCData group standalone data request over MBMS, the receiving MCData client(s) shall check if the MCData ID list IE is included. If so, the receiving MCData client(s) shall check if their own MCData ID is in the list. If not, step 6 and 7 are not required.

6.	If the MCData data disposition for delivery was requested by the user at MCData client 1, then the receiving MCData client(s) initiates a MCData data disposition notification for delivery report.

7.	If the MCData data disposition for read was requested by the user at MCData client 1, then once the receiving user reads the data, the receiving MCData client 2 initiates a MCData data disposition notification for read report.

8.	The MCData data disposition notification(s) from MCData client may be stored by the MCData server for disposition history interrogation from authorized MCData users. The MCData data disposition notification(s) from each MCData user may be aggregated.

9.	Aggregated or individual MCData data disposition notification(s) is sent to the disposition requesting user at MCData client 1.

# * * * * Forth change * * * *

##### 7.4.2.6.2	Procedure

The procedure in figure 7.4.2.6.2-1 describes the case where an MCData user is initiating group standalone MCData data communication with or without disposition request to a group.

Pre-conditions:

1.	MCData users on MCData client 1 to n belong to the same group and are already registered for receiving MCData service and affiliated.

2.	Optionally, the MCData client may have activated functional alias to be used.

3.	The MCData server may have subscribed to the MCData functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.4.2.6.2-1: Group standalone SDS using media plane

1.	User at MCData client 1 would like to initiate a SDS data transfer request to multiple MCData users selecting a pre-configured group (identified by MCData group ID) and optionally particular members from that group.

2.	MCData client 1 sends a MCData group session standalone data request towards the MCData server. The MCData group session standalone data request contains target recipient(s) as selected by the user at MCData client 1. The MCData session group standalone data request contains conversation identifier for message thread indication. The MCData session group standalone data request may include additional implementation specific information in the application metadata container. The MCData session group standalone data request may contain disposition request if indicated by the user at MCData client 1. MCData user at MCData client 1 may include a functional alias within the SDS data transfer.

If the MCData user at MCData client 1 initiates an MCData emergency short data service communication or the MCData emergency state is already set for MCData client 1 (due to a previously triggered MCData emergency alert):

i)	the MCData group session standalone data request shall contain an emergency indicator;

ii)	the MCData group session standalone data request shall set the alert indicator if configured to send an MCData emergency alert while initiating an MCData standalone data request for the emergency short data service communication;

iii)	if the MCData emergency state is not set already, MCData client 1 sets its MCData emergency state. The MCData emergency state is retained until explicitly cancelled; and

iv)	once an MCData emergency communication has been initiated, the MCData group is considered to be in an in-progress emergency state until cancelled.

If the MCData user at MCData client 1 initiates an MCData imminent peril short data service communication:

i)	the MCData group session standalone data request shall contain an imminent peril indicator; and

ii)	once an MCData imminent peril communication has been initiated, the MCData group is considered to be in an in-progress imminent peril state until cancelled.

2a.	If either an emergency indicator or an imminent peril indicator is present in received MCData group session standalone data request, the MCData server implicitly affiliates MCData client 1 to the MCData group if the client is not already affiliated.

3.	MCData server checks whether the MCData user at MCData client 1 is authorized to send MCData session group standalone data request. The MCData server resolves the MCData group ID to determine the members of that group and their affiliation status, based on the information from the group management server. The MCData server also checks whether any policy is to be asserted to limit certain types of message or content to certain members due, for example, to location or user privilege. MCData server also verifies whether the provided functional alias, if present, can be used and has been activated for the user.

i)	if an emergency indicator is present in the received MCData group session standalone data request and if the MCData group is not in the in-progress emergency state, the MCData group is considered to be in the in-progress emergency state until cancelled; and

ii)	if an imminent peril indicator is present in the received MCData group session standalone data request and if the MCData group is not in the in-progress imminent peril state, the MCData group is considered to be in the in-progress imminent peril state until cancelled.

3a.	The MCData server configures the priority of the underlying bearers for all participants in the MCData group.

4.	MCData server initiates the MCData group session standalone data request towards each MCData user determined in Step 3. The MCData ID list shall not be included in a unicast downlink delivery to an individual MCData client. The Disposition Type IE shall not be included in a unicast downlink delivery to MCData clients who are not in the MCData ID list in step 2. The MCData group session standalone data request towards each MCData client contains:

i)	an emergency indicator, if it is present in the received MCData group session standalone data request from the MCData client 1;

ii)	an imminent peril indicator, if it is present in the received MCData group session standalone data request from the MCData client 1; and

iii)	an alert indicator, if requested to initiate an emergency alert in the received MCData group session standalone data request from MCData client 1.

5.	The receiving MCData clients 2 to n automatically accepts the MCData group session standalone data request and responds with MCData group standalone data response towards MCData server.

6.	MCData server forwards the MCData clients 2 to n accepted response to the MCData user initiating the MCData group session standalone data request.

NOTE 1:	Step 6 can occur at any time following step 4, and prior to step 7 depending on the conditions to proceed with the data transmission.

7.	MCData client 1 and MCData server have successfully established media plane for data communication and the MCData client 1 transmits the SDS data.

8.	MCData server distributes the data received from MCData client 1 to MCData clients 2 to n over the established media plane. After completion of the MCData transfer from MCData client 1, media plane resources associated to the data communication are released.

NOTE 2:	MCData server is not required to wait for the complete reception of SDS data from MCData client 1 prior to initiating transmission to MCData client 2 to n.

9.	If the payload is for MCData user consumption (e.g. is not application data, is not command instructions, etc.) then the MCData user of MCData client 2 to n may be notified. Otherwise if the payload is not for MCData user consumption, then the MCData user of MCData client 2 to n shall not be notified. The action taken when the payload contains application data or command instructions are specific based on the contents of the payload. Payload content received by MCData client 2 which is addressed to a known local non-MCData application that is not yet running shall cause the MCData client 2 to start the local non-MCData application (i.e., remote start application) and shall pass the payload content to the just started application.

NOTE 3:	On receiving MCData group session standalone data request over MBMS, the receiving MCData client(s) shall check if the MCData ID list IE is included. If so, the receiving MCData client(s) shall check if their own MCData ID is in the list. If not, step 10 and 11 are not required.

10.	If the MCData data disposition for delivery was requested by the user at MCData client 1, then the receiving MCData client(s) initiates a MCData data disposition notification for delivery report.

11.	If the MCData data disposition for read was requested by the user at MCData client 1, then once the receiving user reads the data, the receiving MCData client 2 initiates a MCData data disposition notification for read report.

12.	The MCData data disposition notification(s) from MCData client may be stored by the MCData server for disposition history interrogation from authorized MCData users. The MCData data disposition notification(s) from each MCData user may be aggregated.

13.	Aggregated or individual MCData data disposition notification(s) is sent to the disposition requesting user at MCData client 1.