3GPP TSG-SA6 Meeting #66	S6-251261

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025              (revision of S6-251015)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0656 | rev | 1 | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add reference point between GMS and recording server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Airbus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2025-03-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A reference point between Recording server and GMS is missing. This is required to enable the Recording server to fetch (or receive) the Group Master Keys (GMK). The Recording server needs the GMKs for deciphering of group communications recordings before storage and/or to store the GMK with the recordings. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add REC-5 reference point between GMS and Recording server and a clause describing this reference point. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Essential reference point missing from the functional architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.2, 7.3.1.3, 7.5.2.37 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This reference point did exist until SA6#64, where it was removed (during the meeting) based on discussions in the MC breakout session. After re-evalution our conclusion is that such decision should be reverted. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.3.1.2	Functional model for an MC system

Figure 7.3.1.2-1 shows the functional model for the application plane for an MC system.

Figure 7.3.1.2-1: Functional model for application plane for an MC system

The common services core functions and reference points shown in figure 7.3.1.2-1 are shared across each MC service. The description of the functions and reference points specific to an MC service is contained in the corresponding MC service TS.

NOTE 1:	The ACM client and ACM server are not essential entities to enable MC services. The ACM client and ACM server are optional and only required if the administrative configuration management feature is deployed. In administrative configuration management case, only the UE of an authorized MC service user requires such an ACM client.

NOTE 2:	The recording server, recording admin client and replay client and mass storage(s) are optional entities. They are not essential to enable MC services. The recording admin client and replay client can also be implemented in devices that do not contain MCPTT / MCData / MCVideo service clients and that are connected to the CSC and recording server using other communication networks than EPS. See subclause 7.3.1.3.

NOTE 3:	The Le reference point is not an essential reference point to enable MC services. It is only required if location information of MC service UE(s) is obtained from the PLMN operator via the LCS.

Editor's note:	Aspects related to interconnection versus recording are FFS.

In the model shown in figure 7.3.1.2-1, the following apply:

-	A specific MC service server is an instantiation of a GCS AS in accordance with 3GPP TS 23.468 [18].

-	The functional alias management client is an integrated functional entity of the configuration management client. The functional alias management client is described in subclause 7.4.2.2.12.

-	The functional alias management server is an integrated functional entity of the configuration management server. The functional alias management server is described in subclause 7.4.2.2.13.

Figure 7.3.1.2-2 shows the functional model for the signalling control plane.

Figure 7.3.1.2-2: Functional model for signalling control plane

Figure 7.3.1.2-3 shows the relationships between the reference points of the application plane of an MC service server and the signalling plane.

Figure 7.3.1.2-3: Relationships between reference points of MC service application plane and signalling control planes

NOTE 4:	Application plane reference point CSC-7 makes use of SIP-2 reference point when the group management servers are connected by a single SIP core. Where they are joined by more than one SIP core, CSC-7 also makes use of the SIP-3 reference point.

NOTE 5:	For simplicity, the HTTP proxy, which provides the interconnection between HTTP-1, HTTP-2 and HTTP-3 reference points, is not shown in figure 7.3.1.2-3.

NOTE 6:	CSC-5, CSC-15, REC-3, REC-4 and REC-5 make use of SIP-1 and SIP-2 reference point. For simplicity, this mapping relationship is not shown in figure 7.3.1.2-3.

# * * * * Second change * * * *

#### 7.3.1.3	Functional model for Recording admin UE and Replay UE

The recording admin and replay functions can be implemented also in UEs that are not MC service UE’s i.e. do not have the MCPTT / MCData / MCVideo clients.

A minimum set of MC common functions are required in such UEs. Two examples are presented in this clause, a “Recording admin UE” and a “Replay UE”.

Figure 7.3.1.3-1: Functional model for application plane of a Recording admin UE

Figure 7.3.1.3-2: Functional model for application plane of a Replay UE

NOTE:	A recording admin client and a replay client can also be implemented in the same UE.

# * * * * Third change * * * *

#### 7.5.2.37	Reference point REC-5 (between recording server and group management server)

The REC-5 reference point, which exists between recording server and group management server, is used by the recording server to obtain information related to target groups for the recordings, including the group related key material.

The REC-5 reference point shall use HTTP-1 and HTTP-2 reference points for transport and routing of non-subscription/notification related signalling. The REC-5 reference point shall use SIP-2 reference point for transport and routing of subscription/notification related signalling.

# * * * * End of changes * * * *