3GPP TSG-SA6 Meeting #66	S6-251262

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025              (revision of S6-251016)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0456 | rev | 1 | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 7.4.2.3.2 MCPTT server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Airbus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2025-03-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 7.4.2.3.2 has incorrect information on the role of MCPTT participating server vs recording and lawful intercept. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the incorrect text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect stage 2 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 7.4.2.3.2	MCPTT server

The MCPTT server functional entity provides centralised support for MCPTT services.

All the MCPTT clients supporting users belonging to a single group are required to use the same MCPTT server for that group. An MCPTT client supporting a user involved in multiple groups can have relationships with multiple MCPTT servers.

NOTE 1:	Possible requirements for handling multiple distinct media on different MCPTT servers are not covered in this version of the document.

The MCPTT server functional entity represents a specific instantiation of the GCS AS described in 3GPP TS 23.468 [9] to control multicast and unicast operations for group communications.

The MCPTT server functional entity is supported by the SIP AS, HTTP client and HTTP server functional entities of the signalling control plane.

By assuming the role of a GCS AS, the MCPTT server functional entity is responsible for:

-	requesting the allocation of multicast resources utilizing the media distribution function;

-	announcing the association of multicast resources to calls to MCPTT UEs;

-	determining for each MCPTT UE involved in a given call whether to use unicast or multicast transport;

-	announcing the assignment of multicast transport for specific calls to MCPTT UEs; and

-	informing the media distribution function of the media streams requiring support for a given call.

The MCPTT server shall support the controlling role and the participating role. The MCPTT server may perform the controlling role for private calls and group calls. The MCPTT server performing the controlling role for a private call or group call may also perform a participating role for the same private call or group call. For each private call and group call, there shall be only one MCPTT server assuming the controlling role, while one or more MCPTT servers in participating role may be involved.

The MCPTT server performing the controlling roles is responsible for:

-	call control (e.g. policy enforcement for participation in the MCPTT group calls) towards all the MCPTT users of the group call and private call;

-	interfacing with the group management server for group policy and affiliation status information of this MCPTT server's served affiliated users;

-	enforcing functional alias priority handling;

-	managing floor control entity in a group call and private call;

-	managing media handling entity in call i.e. conferencing, transcoding; and

-	interfacing with the recording server for the management of the recording processes.

The MCPTT server performing the functional alias controlling role is responsible for:

-	interfacing with the functional alias management server for functional alias policy from the functional alias configuration;

-	functional alias activation, deactivation, take over and interrogation support for MCPTT user.

The controlling roles for group call, private call and functional alias are independent with each other.

The MCPTT server performing the participating roles is responsible for:

-	call control (e.g. authorization for participation in the MCPTT group calls) to its MCPTT users for group call and private call;

-	group affiliation support for MCPTT user, including enforcement of maximum N2 number of simultaneous group affiliations by a user;

-	enforcing functional alias priority handling;

-	relaying the call control and floor control messages between the MCPTT client and the MCPTT server performing the controlling role; and

-	media handling in call for its MCPTT users, i.e. transcoding, lawful interception for both unicast and multicast media.

NOTE 2:	The MCPTT server in the controlling role and the MCPTT server in the participating role can belong to the same MCPTT system or to different MCPTT systems.

For group regrouping involving multiple groups from primary and partner MCPTT systems,

-	the group host MCPTT server of the temporary group performs the controlling role and is responsible for the centralized floor control, and for arbitration according to the temporary group or user policies (e.g., priority);

-	the group host MCPTT server of the constituent MCPTT group is responsible for providing call invitations to their group members, and for filtering between constituent group members' floor control requests according to the constituent group or user policies (e.g., priority); and

-	the MCPTT server responsible for the constituent MCPTT group members performs the participating role.

# * * * * End of changes * * * *