3GPP TSG-SA6 Meeting #66	S6-251271

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025			   (revision of S6-251026)

Source:	Airbus

Title:	Pseudo-CR on References and Annex A for TR 23.700-39

Spec:	3GPP TR 23.700-39 v0.0.0

Agenda item:	9.2

Document for:	Approval

Contact:	Jukka Vialen (jukka.vialen@airbus.com)

1. Introduction

This contribution proposes content for TR 23.700-39 Annex A (List of stage 1 requirements).

NOTE: The title of this pCR mentions also ‘References’. In the pCR revisioning phase the reference section was removed from this pCR and added to S6-251270.

2. Reason for Change

Listing stage 1 requirements in the TR makes it easier for the reader to review this study document (no need to search those requirements from SA1 specifications).

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-39 v0.0.0.

* * * First Change * * * *

# Annex A (informative): List of stage 1 requirements

# A.1	Requirements for on-network recording and audit

Requirements for on-network recording and audit in 3GPP TS 22.280 [8] are listed in Table A.1-1. They are applied to all services MCPTT, MCVideo and MCData according to the reference tables in Annexes A, B and C of 3GPP TS 22.280 [8].

Table A.1-1: Requirements for on-network recording and audit

| Requirement | Description |
| --- | --- |
| [R-6.15.4-001] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log the metadata of the MCX Service Group Communications and MCX Service Private Communications under the organization's authority. |
| [R-6.15.4-002] | Metadata shall be logged for both the transmitting Participant and the receiving Participant(s). |
| [R-6.15.4-003] | The MCX Service shall provide a mechanism for a Mission Critical Organization to record the transmissions of the Group Communications and Private Communications under the organization's authority. |
| [R-6.15.4-004] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log at least the following metadata per communication: depending on service this may include; start time, date, MCX User ID, functional alias(es), MCX Group ID, Location information of the transmitting Participant, end time or duration, end reason, type of communication (e.g., MCX Service Emergency, regroup, private) and success/failure indication. |
| [R-6.15.4-005] | If an MCX Service Group Communication or MCX Service Private Communication uses end-to-end confidentiality, the MCX Service shall provide a mechanism for a Mission Critical Organization to maintain the end-to-end confidentiality when the MCX Service Group Communication or MCX Service Private Communication is logged. |
| [R-6.15.4-006] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log the metadata of non-communication related user activities under the agency's authority. |
| [R-6.15.4-007] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log at least the following non-communication activity types: MCX Service Emergency Alert, MCX Service Emergency Alert cancellation, In-progress Emergency cancellation, registration state change, overridden event, user remote logout, changing another user's affiliations, affiliation change, change of Selected MCX Service Group, (de)activation of functional alias(es) and User and Group Regroup operations. |
| [R-6.15.4-008] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log at least the following metadata per non-communication activity: time, date, MCX Service User identity, activity type and pertinent information about the activity (e.g. MCX User/Group ID(s) included in the temporary group created by the User/Group Regroup operation). The following metadata should be logged if applicable to the activity type: MCX Service Group ID, Location information of the MCX User, affiliation list, target MCX Service User ID and success/failure indication. |
| [R-6.15.4-009] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log metadata for all failed authorization attempts (e.g., invalid login password) by an MCX User. |
| [R-6.15.4-010] | The MCX Service shall provide a mechanism to collect metadata for network access events (e.g., pre-emption of the 3GPP system bearer services, loss of signal, failed registration attempts). |
| [R-6.15.4-011] | The MCX Service shall provide recording and audit functions for all types of MCX Service group communication (e.g., normal group, regroup group, Ad hoc group). |

# A.2	Requirements for off-network recording and audit

Requirements for off-network recording and audit in 3GPP TS 22.280 [8] are listed in Table A.2-1. They are applied to all services MCPTT, MCVideo and MCData according to the reference tables in Annexes A, B and C of 3GPP TS 22.280 [8].

Table A.2-1: Requirements for off-network recording and audit

| Requirement | Description |
| --- | --- |
| [R-7.14-001] | The Off-Network MCX Service shall provide a mechanism to collect metadata for MCX Service Group Communications and MCX Service Private Communications (e.g., initiating MCX Service User ID, MCX Service Group ID) and non-communication activities (e.g., changing group settings) from MCX UEs operating in off-network mode. Metadata shall be logged for both the transmitting Participant and the receiving Participant(s). |
| [R-7.14-001a] | The Off-Network MCX Service shall provide a mechanism for a Mission Critical Organization to record the media content of the transmitting Participant of Group Communications and Private Communications under the organization's authority from MCX UEs operating in off-network mode. NOTE: The handling of storage limitation of the UE is out of scope of 3GPP. |
| [R-7.14-002] | Upon return to on-network operation, the MCX Service shall provide a mechanism to retrieve communication and non-communication activity metadata from an MCX UE that has collected such metadata while operating in off-network mode. |
| [R-7.14-002a] | Upon return to on-network operation, the MCX Service shall provide a mechanism to retrieve the media content of the transmitting Participant of Group Communications and Private Communications from an MCX UE that has recorded such media content while operating in off-network mode. |

* * * End of changes * * * *