3GPP TSG-SA WG6 Meeting #66	S6-251282

Goteborg, Sweden, 7th – 11th April 2025	(revision of S6-251059)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.283 | CR | 0092 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interworking support for ad hoc group emergency alerts (LMR initiated) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph6-MC |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR adds a new procedure where an LMR user initiates an ad hoc group emergency alert involving both MCPTT users and LMR users. And if configured, the LMR client may setup a subsequent MCPTT ad hoc group emergency communication, using the ad hoc group ID from the ad hoc group emergency alert. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new interworking procedure for an ad hoc group emergency alert initiated by a LMR user. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interworking is not fully supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 10.17.3.2b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

##### 10.17.3.2b	Ad hoc group emergency alert initiated by an LMR user

Figure 10.10.3.2b-1 illustrates the procedure for an LMR user client initiating an ad hoc group emergency alert involving both MCPTT users and LMR users.

NOTE 1:	How the LMR users are determined and invited to the MCPTT ad hoc group emergency alert is outside the scope of the present document.

Pre-conditions:

1.	The LMR user or IWF is authorized to initiate an ad hoc group emergency alert.

2.	An LMR user wants to invite MCPTT users who are satisfying certain criteria for the ad hoc group emergency alert.

3.	The mapping relationship of group and user identities between the MCPTT system and the LMR system has been configured at the IWF.

NOTE 2:	For all the signalling messages passing through the IWF between the MCPTT system and the LMR system, the IWF performs the identity conversion and protocol translation.

Figure 10.10.3.2b-1: Ad hoc group emergency alert initiated by an LMR user

1.	The IWF initiates the ad hoc group emergency alert on behalf of the LMR system by sending an ad hoc group emergency alert request containing the details of the criteria to be applied by the MC service server for determining the participants list or the list of participants. The IWF may select a functional alias used for the MC service group emergency alert.

2.	The MC service server checks whether the user identified by the MCPTT ID supplied by the IWF is authorized for initiation of ad hoc group emergency alerts and whether the request is supported. The MC service server checks whether the provided functional alias, if present, can be used and has been activated.

3.	The MC service server sends the ad hoc group emergency alert request return to the IWF containing the result of whether the ad hoc group call is authorized or not. If the ad hoc group emergency alert request is not authorized, the MC service server and the IWF shall not proceed with the rest of the steps.

4.	The MC service server determines the list of participants to be invited for the ad hoc group emergency alert based on the information present in the information element Criteria for determining the participants. The MC service server stores the ad hoc group ID together with the list of participants for the duration of the ad hoc group emergency alert. The MC service server considers the alerted participants as implicitly affiliated to the ad hoc group. Depending on the criteria provided and based on local policy, the MC service server may modify the content of the criteria received in step 1 to determine the list of participants.

5.	The MC service server sends an ad hoc group emergency alert request including the ad hoc group ID, and the call resulting criteria towards the MC service clients 1 and 2.

6.	MC service users are notified of the ad hoc group emergency alert. The functional alias of the MC service user initiating the ad hoc group emergency alert and additional information related to the alert may be displayed.

7.	The receiving MC service clients send the ad hoc group emergency alert response to the MC service server to acknowledge the ad hoc group emergency alert.

8.	The MC service server sends the ad hoc group emergency alert response to the IWF to inform about successful alert establishment, containing the ad hoc group ID (generated by the MC service server in the case where the MC service ad hoc group ID created by the IWF is not acceptable or the case where the MC service ad hoc group ID was not provided by the IWF) and the call resulting criteria.

9.	If configured, the IWF may initiate on behalf of the LMR user in the LMR system a subsequent MCPTT ad hoc group emergency call, using the ad hoc group ID from the ad hoc group emergency alert.

* * * End of Change * * * *