3GPP TSG-SA WG6 Meeting #66	S6-251314

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251212)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on the gaps of GMS as trusted AF

Spec:	3GPP TR 23.700-35 V0.3.0

Agenda item:	8.9

Document for:	Approval

Contact:	Yang Yanmei Yangyanmei@huawei.com

1. Introduction

This contribution is to provide solution for the Gap#4-3-2-1of GMS services.

“Gap#4-3-2-1:	Identify the procedures from the SEAL specification to provide the direct interaction (not via NEF/SCEF) with CN NFs (e.g., PCF, SMF) and/or indirect interaction (via NEF/SCEF) per different deployments and requirements and make changes if required.”

2. Reason for Change

Currently only the solution for SEAL GMS invoking 5GC as non-trusted AF is specified. The option for GMS as trusted AF invoking 5GS for dynamiclly group creation is neither supported either in 3GPP TS 23.434, nor in 3GPP TS 23.501 and  23.502.

Even for non-trusted AF based mechnism, there still no decription about which GMS services and procedutes requires and how they use NEF group realted services in TS 23.434.

And according to 3GPP 23.501, the 5G VN group can be managed by two ways as the following:

5.29.2	5G VN group management

5G System supports management of 5G VN Group identification and membership (i.e. definition of 5G VN group identifiers and membership) and 5G VN Group data (i.e. definition of 5G VN group data). The 5G VN Group management can be configured by a network administrator or can be managed dynamically by AF.

…….

In order to support dynamic management of 5G VN Group identification and membership, the NEF exposes a set of services to manage (e.g. add/delete/modify) 5G VN groups and 5G VN members. The NEF also exposes services to dynamically manage 5G VN group data.

An AF can request provisioning of traffic characteristics, QoS parameters and monitoring of QoS parameters for a 5G VN group as described in clause 6.1.3.28 of TS 23.503 [45].

A 5G VN group is identified by the AF using External Group ID. The NEF provides the External Group ID to UDM. The UDM maps the External Group ID to Internal Group ID. For a newly created 5G VN Group, an Internal Group ID is determined by the UDM based on implementation specific means”

So how to enable GMS to use 5G VN group management services when deployed within trusted domain is a gap.

3. Proposal

It is proposed to agree the following Gaps to 3GPP TR 23.700-35 V0.3.0.

* * * First Change * * * *

4.3	Existing SEAL service: Direct and/or indirect interaction \with CN NFs per different deployments and requirements

4.3.1	Analysis

According to 3GPP TS 23.501 [10] and TS 23.502 [11], a MNO‘s AF(trusted AF) can invoke all the APIs exposed by NF(s) from SBA interface. Thus, SEAL server(s) deployed by an MNO within the MNO’s domain may act as the trusted AF and is able to invoke network service APIs exposed by 3GPP core network functions directly over the core network SBA interface.

There is description in clause 10.2 of TS 23.434 about using the NEF exposed VN group management service APIs. However, there is no any description about how to used NEF exposed VN group management service APIs in any procedures in clause 10.3.

The option of GMS as trusted AF also needs to be addressed, given network layer VN group management capabilities are needed for certain SEAL layer group service.

4.3.2	Gaps

Gap#4-3-2-1:	Identify the procedures from the SEAL specification to provide the direct interaction (not via NEF/SCEF) with CN NFs (e.g., PCF, SMF) and/or indirect interaction (via NEF/SCEF) per different deployments and requirements and make changes if required.

Gap#4-3-2-2: Study how to enable GMS to use 5G VN group management services when deployed within trusted domain.

Editor NOTE: The coordination with SA2 for the supporting of trusted AF managing dynamic VN group is required if the capabilities are needed for SEAL GMS.

* * * The end of  Change * * * *