3GPP TSG-SA WG6 Meeting #66	S6-251315

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251213)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on the evaluation of adoption solution1

Spec:	3GPP TR 23.700-35 V0.3.0

Agenda item:	8.9

Document for:	Approval

Contact:	Yang Yanmei Yangyanmei@huawei.com

1. Introduction

This contribution adds more information to justify the solution 1 and  proposes to add the evaluation of adoption solution1.

2. Reason for Change

Although some corrections for aligement are reuiqred for some pre-rel19 SEAL specifications, the support of application on UE as the API invokers has been accepted by SA6. The following are examples:

- TS 23.222 supports the API invoker on the UE already in clause7.5:

The API invoker may be deployed in the following ways:

a.	API invoker may be deployed as AF on the UE (i.e., 3rd party application).

b.	API invoker may be deployed as AF on the UE supporting several other 3rd party applications deployed on the UE.

c.	API invoker may be deployed on the network as AF.

The resource owner is connected via a UE and can use a Resource Owner Function on the UE to interact with the CCF acting as the Authorization Function to grant RO permission to the API invoker to access resource(s) of the resource owner provided by the service API. The CCF (Authorization Function) uses the RO authorization information, as part of API invoker authorization process.

NOTE:	The details of the protocol for CAPIF supporting RNAA is specified in 3GPP TS 33.122 [12].

When API invoker is deployed on a UE (cases a and b), with suitable authorization, the API invoker is allowed to access the resources (via service API) of the resource owner corresponding to the UE.

When API invoker is deployed on a UE (cases a and b), with suitable authorization, the API invoker is allowed to access the resources (via service API) of the resource owner of other UEs.

- TS 23.438 supports the API invoker on the UE already:

The following text is already defined in TS 23.438:

The VAL functions (VAL clients and servers) and the DA client (serving a VAL client) consume the services by interacting on the Sda interface of the DA server.

3. Proposal

It is proposed to agree the evauation of add the evaluation of adoption solution as the following P-CR.

.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications; Stage2".

[3]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Stage 2".

[4]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service; Stage 2".

[5]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service; Stage 2".

[6]	3GPP TS 23.437: "Service Enabler Architecture Layer for Verticals (SEAL); Spatial mapping and Spatial anchors management Stage 2".

[7]	3GPP TS 23.438: "Service Enabler Architecture Layer for Verticals (SEAL); Digital asset".

[8]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[9]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2"

[10]	3GPP TS 23.501: "System architecture for the 5G System (5GS)" stage 2"

[11]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[12]                      3GPP TS 28.533: " Management and orchestration; Architecture framework”

* * * Second Change * * * *

8.2.1	Adoption Solution#1: Service based Architecture of 3GPP system supporting SEAL services

8.2.1.1	Solution description

This solution is to solve GAP# 5-1-2-1.

Figure 8.1.1-1 Service based Architecture of 3GPP system supporting SEAL services

In the figure 8.1.1-1, SEAL services layer represents the SEAL server(s) supporting SEAL services defined in the SEAL specification 3GPP TS 23.434 [3].

SEAL services which are part of the 3GPP system can be deployed either by the MNO or third party.

SEAL server(s) can invoke service APIs exposed by 3GPP core network functions directly over the Core network SBA when deployed by MNO’s trusted domain. SEAL server(s) can invoke service APIs exposed by 3GPP core network functions via NEF as a non-trusted AF when deployed by the third party.

SEAL server(s) can invoke service APIs exposed by the OAM system over the OAM SBMA when deployed by MNO as specified in 3GPP TS28.533[12]. SEAL server(s) can invoke services APIs exposed by 3GPP core network functions via EGMF as non-trusted AF, when deployed by the third party.

SEAL services API can be exposed by MNOs or third party, to the Application server or the Application client(s) or the SEAL client(s).

NOTE 1:	In some serverless application software implementations, application server (VAL server) may not be implemented and deployed. In such deployments, Application client(s) can support all the application requirements by invoking SEAL service APIs.

NOTE 2:	3GPP TS 23.222 [9] supports where the API invokers can either be on the network side (Application server) or on the UE side (Application clients), to use services API.

8.2.1.2	Functional Elements

NOTE:	No new function elements are added to the existing 3GPP system.

8.2.1.3	Reference Points

NOTE:	No new reference points added to the existing 3GPP system.

8.2.1.2	Solution evaluation

This solution provides the holistic picture providing the role and responsibility of SEAL service layer within the complete 3GPP system perspective. It clarifies the SEAL services can be implemented as part of 3GPP system and can well avoid some misleading.

* * * The end of  Change * * * *