3GPP TSG-SA WG6 Meeting #66	S6-251316

Gothenburg, Sweden, 7th – 11th April 2025	(revision of S6-251225)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on new solution to KI on new NRM API of power saving

Spec:	3GPP TR 23.700-35 v0.3.0

Agenda item:	9.3

Document for:	Approval

Contact:	gecuili@huawei.com

- Introduction

This pCR is proposed to a new solution to the gaps identified in clause 4.2.

2. Reason for Change

As described in the corresponding discussion paper S6-251154.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR  23.700-35 v0.3.0.

# * * * First Change * * * *

4.4.	Existing SEAL service #3: NRM service to support power saving

4.4.1	Analysis

1.	The network API is not designed for dedicated application scenarios(e.g., power saving) in e.g., XR service, IoT service.

Several network APIs defined in 3GPP TS 29.122 [29.122] can be used by the application layer to achieve the power saving as listed below.  There are some issues for the ASP to enjoy the power saving features provided by the MNO, e.g., the event is not explictly linked to power saving as analyzed in the following table. This makes it difficult for the application developer or the service provider to derive the required set of input parameters to provide or the service. This analysis primarily focuses on how to design the APIs catering to the need of IoT applications.

| Network API in 3GPP TS 29.122 | Description | Gap |
| --- | --- | --- |
| MonitoringEvent API | allows the SCS/AS to subscribe to notifications about specific events in 3GPP networks when the corresponding event is detected. | - The “UE_REACHABILITY” mointoring event type is also used for powering saving case, however it is not explictly known by the consumer. |
| CpProvisioning API | allows the SCS/AS to add, change or delete the communication pattern parameter sets of the UE. | This is related with power saving for communication model with a regular interval. |
| NpConfiguration API | allows the SCS/AS to send suggested network parameters to influence certain aspects of UE/network behaviour such as the UE's PSM, extended idle mode DRX, and extended buffering configurations. | This is related power saving. |

In addition, the NEF AsSessionWithQoS API also covers the power saving aspects for the XR service as described in clause 4.4.9.2.10 of  3GPP TS 29.522 [29.522] and table 5.3-1.

2.	NRM is defined to simplify the utilization of network resource by 3rd party. However, there are some gaps support the power saving application requirements in a friendly way:

-	SS_NetworkResourceAdaptation covers as many as use cases e.g., unicast, multicast/broadcast, TSC. The supporting of IoT is defined as several API operations including BDT Configuration, Unified_Traffic_Pattern_and_Monitoring_Management operation, BDT_Configuration, Device_Triggering. Which operations should be used to achieve power saving is not clear to the ASP.

-	NRM Event monitoring API has the similar issue as the network MonitoringEvent API in 3GPP TS 29.122 [29.122].

-	The NRM service API granularity is not matched with the application scenarios(e.g.power saving). For a single use case, several NRM service APIs has to be invoked. For example, for power saving, the IoT appliction developer/service provider has to invoke the SS_NetworkResourceAdaptation with Unified_Traffic_Pattern_and_Monitoring_Management operation, EventMonitoring API with power saving related input parameters.

4.4.2	Identified gaps

-	How to support the application requirement on power saving in a simple and explicit way should be studied;

# * * * Next Change * * * *

8.b	Solution #x: New NRM service for power saving

8.b.1	Solution description

This solution proposes to address the gaps identified in clause 4.4 regarding the support power saving requirements at NRM. based on the following principles:

- The new NRM services are designed per application layer requirements as described in clause 8.4.A single service API serves a single service;

- The input parameters of each service are minimum and necessary to achieve the desired benefit corresponding to the application requirement(s);

- The name of the service, service APIs, input parameters are familiar to the applications.

The new NRM service are as follows:

-	Power saving service. This service enables the VAL server to provide the application traffic characteristics e.g., application traffic patterns, background data transfer information, time period between the start of the two data bursts in uplink and/or downlink direction, and the downlink protocol description related information to the network for power saving setting. At the same time, it allows the VAL server to subscribe the power saving related information e.g., UE idle status, maximum UE availability time from the network.

8.b.1.1	Procedures

Editor’s note:	The information flows to be added is FFS.

The figure 8.b.1.1-1 illustrates the high-level procedure for the application layer consumer (e.g., VAL server) to use the power saving service.

Figure 8.b.1.1-1: Power saving invocation procedure

1.	The application layer consumer (e.g., VAL server) invokes the power saving service. The input parameters e.g., application traffic patterns, background data transfer information, time period between the start of the two data bursts in uplink and/or downlink direction, and the downlink protocol description related information to the network for power saving setting are provided.

2. Upon receiving the service request, the NRM determines to invoke one or more network related APIs addressing the power saving aspect.

3. The NRM server may invoke one or more network related APIs (either via the NEF/SCEF or directly towards the NF(s) which provides the corresponding service) listed below:

3a.	MonitorEvent API;

3b.	CpProvisioning API;

3c.	NpProvisioning API;

3d.	AFSessionWithQoS API.

4.	The NRM server returns a power saving service response. This may be sent before step 3.

5.	Optionally, the NRM server further returns a power saving service notification.

8.b.1.3	NRM service for power saving

The following new NRM service listed in table 8.b.1.3 are introduced:

Table 8.b.1.3-1: List of new NRM services for network resource management

| Service Name | Service Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| Power saving service | _Create | Application layer entity | Request/response and subscribe/notify |
|  | _Update | Application layer entity | Request/response and subscribe/notify |
|  | _Delete | Application layer entity | Request/response and subscribe/notify |

8.b.2	Impacts on architecture, services, entities

This solution has no architecture impacts.

This solution creates new NRM service at NRM for power saving.

8.b.3	Solution evaluation