3GPP TSG-SA WG6 Meeting #66	S6-251364

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251216)

Source:	China Mobile

Title:	Pseudo-CR on Key Issue on enablement of standalone DC

Spec:	3GPP TR 23.700-57 v0.0.0

Agenda item:	9.6

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR proposes new Key Issue for TS23_700_57.

2. Reason for Change

Contribution with new Key Issue.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-57 v0.0.0.

* * * First Change * * * *

4.X	Key Issue #X: enablement of standalone DC

4.X.1	Description

Standalone IMS Data Channel Sessions is specified in Annex AC.10 of 3GPP TS 23.228 [23228], and the related IMS AS Services is specified in Annex AA.2.4 of 3GPP TS 23.228 [23228].

For the A2P use cases, the Standalone Data Channel provides application providers and vertical service providers with a method to establish Data Channels and interact with users through them, without requiring the setup of audio/video calls. For these providers, in most usage scenarios, the primary focus is on delivering application content (via the Data Channel) rather than managing underlying IMS session details. Therefore, it is valuable for the MMTel Enabler to enable applications to directly use the application interaction capabilities of Standalone DC, such as sending and receiving of a content like menu, without needing to know whether an IMS Session exists in the network.

4.X.2	Open Issue

This key issue aims to solve the following open issue in the MMTel Enabler layer:

1.	How to provide applications with direct access to Standalone Data Channel capabilities for application-layer interactions (e.g., menu content exchange), abstracting away the underlying IMS session awareness requirements.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[23228]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".