3GPP TSG-SA WG6 Meeting #66	S6-251365

Gothenburg, Sweden 7th – 11th April 2025	revision of S6-251198

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0146 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SEALDD enabled XR data transmission service for XR application |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - XR services include not only multi-model services but also single-model services. Moreover, multi-modal services are not limited to XR services. However, the statement in clause 9.12.2.1 regarding the establishment of XR data transmission connection is limited to multi-model services. This could lead to a misunderstanding of the wording related to the XR data transmission. - There are two wording of “request” that need to be corrected to “response” in clause 9.12.3.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that clause 9.12.2.1 is for the procedure for establishing multi-modal data transmission connection which is not limited to XR services. - The relevant information flow and API names have been modified in clause 9.12.3.1, 9.12.3.2, 9.12.3.3, 9.12.3.4, 9.12.4.1 and 9.12.4.2. - Corret the “request” to the “response” in clause 9.12.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SEALDD enabled multi-modal data transmission service for multi-modal application will remain misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12.2.1, 9.12.3.1, 9.12.3.2, 9.12.3.3, 9.12.3.4, 9.12.4.1, 9.12.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

### 9.12.2	Procedures

#### 9.12.2.1	SEALDD enabled multi-modal data transmission service for multi-modal application

##### 9.12.2.1.1	General

The following clauses specify procedures, information flows and APIs about SEALDD enabled data transmission for multi-modal application, including the SEALDD facilate PDU set handling.

##### 9.12.2.1.2	SEALDD enabled multi-modal data transmission establishment

Figure 9.12.2.1.2-1 illustrate the procedure for establishing multi-modal data transmission connection, and the SEALDD facilitates the multi-modal application to transmit its data between the VAL client and VAL server with RTP packetilization and PDU set inclusion.

Pre-condition:

-	The VAL server has discovered and selected the SEALDD server by CAPIF functions.

Figure 9.12.2.1.2-1: SEALDD enabled multi-modal data transmission connection establishment procedure

1.	The VAL server decides to use SEALDD service for multi-modal traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_multi-modalTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, list of requested flows, including SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements. As one of key parameters for multi-modal application, Crossflow measurement can be measured as procedure in clause 9.7.2.2 step 1.

2.	Same as step 2 of clause 9.2.2.2. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5 for each requested flow provided in step 1. The AF request may also include the Multi-modal Service ID, which could be determined by the SEALDD server according to VAL service ID and/or VAL server ID.

3-5.	Same as step 3-5 of clause 9.2.2.2.

6.	The SEALDD client establishes multi-modal transmission connection with the SEALDD server. The request includes the SEALDD client ID, VAL user/UE ID, VAL server ID, VAL service ID, SEALDD-UU flow IDs, and traffic descriptors for the multiple flows from the SEALDD client side. The SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-UU flow) to the SEALDD client, and may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the multi-modal transmission connection response. The response also includes a multi-modal SEALDD-UU flow ID which the SEALDD server allocates.

7-8.	Same as step 8-9 of clause 9.2.2.2.

NOTE :	If multiple requested flows are provided in step 1, the SEALDD client repeats steps 6-8 for each requested flow provided in step 1.

The multi-modal application traffic is exchanged between VAL client and VAL server via SEALDD layer as described in clause 9.2.2.2. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP).

# /**************************** Next changes **********************/

#### 9.12.3.1	SEALDD enabled multi-modal transmission request

Table 9.12.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting multi-modal transmission service.

Table 9.12.3.1-1: SEALDD enabled multi-modal transmission request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of specific UE or VAL user |
| List of requested flows | M | List of the requested multi-modal flows |
| >SEALDD-S Data transmission connection information for multi-modal XR | M | Address(s)/port(s) and/or URL(s) of the VAL server to receive the multi-flow packets from the SEALDD server (e.g. audio flow, tactile flow) |
| >QoS information | O | QoS information provided by VAL server |
| >Protocol description | O | The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP. |

# /**************************** Next changes **********************/

#### 9.12.3.2	SEALDD enabled multi-modal transmission response

Table 9.12.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to the multi-modal transmission response.

Table 9.12.3.2-1: SEALDD enabled multi-modal transmission response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-S information Data transmission connection information for multi-modal service | O | Address(s)/port(s) and/or URL(s) of the SEALDD server to receive the multi-flow packets from the VAL server for application traffic transfer (e.g. audio flow, tactile flow) |
| Cause | O (See NOTE) | Indicates the reason for the failure |
| NOTE: The IE is only present if the Result is failure. |  |  |

# /**************************** Next changes **********************/

#### 9.12.3.3	SEALDD multi-modal transmission connection establishment request

Table 9.12.3.3-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the multi-modal transmission connection establishment.

Table 9.12.3.3-1: SEALDD multi-modal transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD client ID | M | Identity of the SEALDD client. |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID |
| SEALDD-UU flow IDs | M | Identity of the SEALDD-UU flows. |
| VAL server ID | O | Identity of the VAL server. |
| VAL service ID | O | Identity of the VAL service. |
| SEALDD traffic descriptors | O | SEALDD traffic descriptors for multiple flows (e.g. address(s), port(s), transport layer protocol) of the SEALDD client side used to establish SEALDD multi-modal service connection. |

# /**************************** Next changes **********************/

9.12.3.4	SEALDD multi-modal transmission connection establishment response

Table 9.12.3.4-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the multi-modal transmission connection establishment.

Table 9.12.3.4-1: SEALDD multi-modal transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| Multi-modal SEALDD flow ID | M | Identity of the Multi-modal SEALDD-UU flow associated with the multi-modal service connection. |
| SEALDD traffic descriptors | O | SEALDD traffic descriptors for multiple flows (e.g. address(s), port(s), transport layer protocol) of the SEALDD server side used to establish SEALDD multi-modal connection. |
| Protocol description | O | The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP. |
| Cause | O (See NOTE) | Indicates the reason for the failure |
| NOTE: The IE is only present if the Result is failure. |  |  |

# /**************************** Next changes **********************/

### 9.12.4	APIs

#### 9.12.4.1	General

Table 9.12.4.1-1 illustrates the APIs exposed by SEALDD server for XR transmission.

Table 9.12.4.1-1: List of SEALDD server APIs for XR transmission

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_multi-modalTransmission | Request | Request/Response | VAL server |
| Sdd_XRTransmissionConnnection | Establish | Request/Response | SEALDD client |
|  | Trigger (NOTE) | Request/Response | SEALDD server |
|  | Inform | Request/Response | SEALDD client |
| Sdd_SEALDD-UUcConnection | Establish | Request/Response | SEALDD client |
| NOTE: The Sdd_XRTransmissionConnection_Trigger API operation is exposed by the SEALDD client. |  |  |  |

#### 9.12.4.2	Sdd_multi-modalTransmission Request operation

API operation name: Sdd_multi-modalTransmission Request

Description: The consumer requests for one time for multi-modal transmission service.

Inputs: See clause 9.12.3.1.

Outputs: See clause 9.12.3.2

See clause 9.12.2.1.2 and 9.12.2.2.2 for details of usage of this operation.