3GPP TSG-SA WG6 Meeting #66	S6-251366

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251138)

Source:	Telefonica, Apple, Fogus, UMA

Title:	Pseudo-CR on Candidate CAPIF_Ph4 Key Issue Certificate unavailability

Spec:	3GPP TR TBC

Agenda item:	10.5

Document for:	Endorsement

Contact:	Walter Featherstone, w_featherstone@apple.com

1. Introduction

The proposed CAPIF phase 4 SID includes an objective: Support for new CAPIF enhancements identified or originated from other WGs (e.g. SA5 for MSED support in CAPIF, CAPIF charging) or SDOs/industry forums (e.g. GSMA, ETSI OpenCAPIF).

2. Reason for Change

This discussion paper proposes a Key Issue based on findings from ETSI OpenCAPIF, specifically relating to certificate expiry/unavailability.

A call flow illustrating the certificate expiry issue in the case API Invoker offboarding and API Provider deregistration is illustrated below in Figure 1.

Figure 1: Flow showing API Invoker & API Provider with expired Certificate

The steps relating to Figure 1 are described below:

1. API Invoker onboards to the CCF

2. API Invoker registers AMF, APF and AEF into CCF.

3. API Invoker and API Provider entities certificates Expire

4. API Invoker tries to offboard from the CCF, but the operation fails as Certificate is not valid (expired)

5. API Provider tries to remove itself from CCF, but operation fails as Certificate is not valid (expired)

3. Conclusions

n/a.

4. Proposal

It is proposed to endorse the following changes in anticipation of a CAPIF Phase 4 study for which the 3GPP TR number is TBC.

* * * First Change * * * *

## X.x	Key issue #X: Certificate unavailability

### X.x.1	Description

As described in clause 6.1 of 3GPP TS 33.122 [x], a prerequisite to onboarding an API invoker to CAPIF, an API invoker is required to obtain onboarding enrolment information from the API provider domain (see step 1 of the referenced clause). This information is necessary to enable the API invoker to authenticate with the CAPIF core function (CCF) and establish secure communication over which to perform the onboarding process. The enrolment information must include details of the CCF (i.e., its address in the form of a URI and root certificate authority (CA) certificate) and includes an onboarding credential (i.e., an access token). With this information the API invoker can request to onboard to the CCF (clause 8.1 of 3GPP TS 23.222 [y]) and, if successful, receives its onboarding information in the response, which also includes the CCF assigned API invoker ID. A key attribute of the onboarding information response is the certificate assigned to the API invoker, which is signed by the CCF CA. API providers are also assigned signed certificates when registering to the CCF. The problem is that an entity (API Invoker or Provider) can no longer manage their resources at the CCF if they lose access to a valid certificate e.g., certificate expiry). This means for example:

- Since only the API Provider can request to remove API Provider entities (i.e., AMF, APF and AEFs), including  in the request a valid API Provider certificate, there is currently no way to delete or update those entities from CCF.

- Similarly, the same issue occurs with the API Invoker entity, there is no way to delete API invoker from CCF if a valid certificate is not provided when demonstrating that the API Invoker is authorized to perform the deletion or update.

The consequence is that an operator of CCF must interact with CCF database in order to remove obsolete/wrong information. This is not a good practice because:

- Changes on database create problems due to inconsistencies.

- Changes in most cases implies internal flows between internal CCF services and 3rd party services (e.g., sending EVENTS associated to registered API Invokers or Providers/APIs), and that logic will not be triggered if the CCF DB is changed directly.

### X.x.2	Open issues

This key issue will study approaches to managing entities (i.e., API Invoker or API Provider) without a valid CCF issued and signed certificate, specifically how such entities can be deleted from the CCF including removal of their security context, and how API provider published Service APIs can be deleted from the CCF.

* * * End of Changes * * * *