3GPP TSG-SA WG6 Meeting #66	S6-251367

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251139)

Source:	Telefonica, Apple, Fogus, UMA

Title:	Pseudo-CR on Candidate CAPIF_Ph4 Key Issue AEF operational status

Spec:	3GPP TR TBC

Agenda item:	10.5

Document for:	Endorsement

Contact:	Walter Featherstone, w_featherstone@apple.com

1. Introduction

The proposed CAPIF phase 4 SID includes an objective: Support for new CAPIF enhancements identified or originated from other WGs (e.g. SA5 for MSED support in CAPIF, CAPIF charging) or SDOs/industry forums (e.g. GSMA, ETSI OpenCAPIF).

2. Reason for Change

This discussion paper proposes a Key Issue based on findings from ETSI OpenCAPIF.

There are scenarios in which the CCF can be unaware of whether an AEF and the API(s) it offers are active and able to offer service.

Of note, registration of AEF does not include time-to-live (TTL) information as illustrated in the figure below based on 3GPP TS 29.222 [a], so an AEF can be registered indefinitely whether they are up or not.

POST https://example.com/api-provider-management/v1/registrations

{

"apiProvDomId": "string",

"regSec": "string",

"apiProvFuncs": [

{

"apiProvFuncId": "string",

"regInfo": {

"apiProvPubKey": "string",

"apiProvCert": "string"

},

"apiProvFuncRole": "AEF",

"apiProvFuncInfo": "string"

}

],

"apiProvDomInfo": "string",

"suppFeat": "string",

"failReason": "string",

"apiProvName": "string"

}

Figure: CAPIF API Provider Management API registration information highlighting that there is no TTL/Expiration Time in the schema (i.e., information elements)

The issue is illustrated in the figure below that shows a scenario in which an AEF does not respond to an API request.

Figure: AEF service API endpoint not responding to a request

The steps relating to the figure above are described below:

- API provider registers AMF, APF and AEF at the CCF.

- APF publishes the AEF Service API including the AEF ID as part of the AEF Profile

- API Invoker onboards to the CCF

- API Invoker discovers the AEF Service API including the AEF details in AEF Profile

- API Invoker requests a Security Token to consume AEF Service API

- API Invoker tries to consume AEF Service API from the AEF

In case the AEF does not answer the API service request, the CCF has provided details to an API Invoker on an AEF that is not offering service.

Of note, the study is anticipated to consider work beyond 3GPP, for instance ITU recommendation X.731    “Information technology - Open Systems Interconnection - Systems management: State management function”.

3. Conclusions

n/a.

4. Proposal

It is proposed to endorse the following changes in anticipation of a CAPIF Phase 4 study for which the 3GPP TR number is TBC.

* * * First Change * * * *

## X.x	Key Issue #X: AEF operational status

### X.x.1	Description

In clause 8.6.2.1 of 3GPP TS 23.222 [y] specifying the contents of the service API update request sent from the APF to CCF there is a note stating that “how to monitor service API status when the APF is unable to update service API status is not specified in this release”. The consequence is that there are scenarios in which the CCF can be unaware of whether an AEF and the API(s) it offers are active and able to offer service. Of note, registration of AEF does not include time-to-live (TTL) information, so an AEF can be registered indefinitely whether they are up or not. This can lead to a CCF providing erroneous information to API Invokers regarding AEFs that are no longer active. Examples of how this issue has been solved include implementation of a heartbeat mechanism between servers, as indicated for example through the EAS Availability Reporting Period information element as part of the EAS Profile in clause 8.2.4 of 3GPP TS 23.558 [z].

Of note is that certificates provided by the CCF to the API Provider functions do include an expiration time. However, CAPIF does not define any procedure or flow to enable renewing the certificates before expiring. Typically, Certificate Authorities expedite certificates with a duration of 1 year, which is not practical for heartbeating purposes. When an API Provider onboards a series of AEFs, those are registered in CAPIF forever, at least, until the AEF certificate expires. The consequence is that if one or some AEFs become non-functional (e.g., they stop answering API requests), the CCF could provide misleading information to API Invokers. This is because it will be returning AEF profile information associated to API Discovery requests from API Invokers where those AEF Profiles are associated with those AEFs might include non-responsive APIs.

### X.x.2	Open issues

This key issue will study approaches to address scenarios in which the API Provider Domain is unable to provide the status of its AEFs with associated Service APIs.

* * * End of Changes * * * *