3GPP TSG-SA WG6 Meeting #66	S6-251376

Gothenburg, Sweden, 7th – 11th April 2025	(revision of S6-251038)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.438 | CR | 0001 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fix for Digital asset media management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Confusion usage Digital asset profile identifier & Digital asset identifier |  |  |  |  |  |  |  |  |
| Summary of change: |  | Unfied usage Digital asset identifier |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent with Digital asset profile management |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.1, 8.4.3.1, 8.4.3.4, 8.4.3.5, 8.4.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

8.4.2.1	Upload digital asset media

Figure 8.4.2.1-1: Upload digital asset media

1.	A requestor (e.g. DA client) sends a upload digital asset media request to the DA server (at the digital asset media upload endpoint). The request includes information listed in Table 8.4.3.1-1. If the digital asset identifier corresponds to a digital asset collection, then a list of digital assets is included in the request.

2.	The DA server authenticates and authorizes the request and verifies the digital asset identifier. If authorized, the DA server receives the media object or media URI corresponding to the digital asset profile, stores the media or the information about the digital asset media.

3.	The DA server sends an upload digital asset media response to the requestor. The response includes information listed in Table 8.4.3.2-1.

# * * * * Second change * * * *

8.4.2.2	Download digital asset media

Figure 8.4.2.2-1: Download digital asset media

1.	A requestor (e.g. DA client) sends a download digital asset media request to the DA server. The request includes information listed in Table 8.4.3.3-1.

2.	The DA server authenticates and authorizes the request and verifies the digital asset identifier. If authorized, the DA server determines the digital asset media as per the information provided in step 1. If the digital asset identifier corresponds to a digital asset collection, multiple digital asset media objects are provided in the response.

3.	The DA server sends a download digital asset media response to the requestor. The response includes information listed in Table 8.4.3.4-1.

# * * * * Third change * * * *

8.4.3.1	Upload digital asset media request

Table 8.4.3.1-1 describes the information elements for a upload digital asset media request sent from a requestor (e.g. DA client) to a DA server.

Table 8.4.3.1-1: Upload digital asset media request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| List of digital assets | M | List of digital assets to be uploaded |
| > Digital asset identifier | M | Identifier of the digital asset profile |
| > Media URI (NOTE) | O | URI where the media is stored |
| > Media object (NOTE) | O | Media object (e.g. JPG file) |
| NOTE: One of the IE shall be included |  |  |

# * * * * Fourth change * * * *

8.4.3.4	Download digital asset media response

Table 8.4.3.4-1 describes the information elements for the download digital asset media response sent from a DA server to a requestor (e.g. DA client).

Table 8.4.3.4-1: Download digital asset media response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, fail. If the status is fail, a failure cause is provided. |
| List of digital assets | M | List of digital asset media objects included |
| > Digital asset identifier | M | An identifier for the digital asset. |
| > Media URI (NOTE) | O | URI where the media is stored |
| > Media object (NOTE) | O | Media object (e.g. JPG file) |
| NOTE: One of the IE shall be included |  |  |

# * * * * Fifth change * * * *

8.4.3.5	Update digital asset media request

Table 8.4.3.5-1 describes the information elements for an update digital asset media request sent from a requestor (e.g. DA client) to a DA server.

Table 8.4.3.5-1: Update digital asset media request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| List of digital assets | M | List of digital assets to be updated |
| > Digital asset identifier | M | Identifier of the digital asset profile |
| > Media URI (NOTE) | O | URI where the media is stored |
| > Media object (NOTE) | O | Media object (e.g. JPG file) |
| NOTE: One of the IE shall be included |  |  |

# * * * * Sixth change * * * *

8.4.3.7	Delete digital asset media request

Table 8.4.3.7-1 describes the information elements for a delete digital asset media request sent from a requestor (e.g. DA client) to a DA server.

Table 8.4.3.7-1: Delete digital asset media request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| List of digital assets | M | List of digital assets to be deleted |
| > Digital asset identifier | M | Identifier of the digital asset profile |

# * * * * End of changes * * * *