3GPP TSG-SA WG6 Meeting #66	S6-251xxx

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251151/ S6-251386)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on New key issue on SEALDD-S and SEALDD-UU user plane protocol

Spec:	3GPP TR 23.700-52 v0.0.0

Agenda item:	9.7

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This pCR is proposed to introduce a new Key issue.

2. Reason for Change

According to the objective the FS_SEALDD_Ph3 SID, the user plane aspect of SEALDD-S and SEALDD-U needs to be studied.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-52 v0.0.0.

* * * First Change * * * *

5.x	Key issue #x: SEALDD-S and SEALDD-UU user plane

5.x.1	Description

SEALDD services require both control signalling messages and user plane to support the delivery of application traffic. Currently, in 3GPP TS 23.433 [23.433] only the control signalling messages over SEALDD-UU and SEALDD-S are specified for SEALDD services. However, SEALDD user plane (i.e., SEALDD-UU, SEALDD-S) aspects are not fully and well specified. For example, the user plane protocol stack(s) is missing.

In addition, the user plane functionalities description is missing even though several user plane functionalities are already described in the specification, including but not limited to:

1)	The SEALDD flow ID is allocated to uniquely identify the application traffic.

2)	The SEALDD enabled data transmission quality measurement in clause 9.7.2of 3GPP TS 23.433 [23.433] requires the DL monitoring packet or the dummy DL SEALDD packet.

3)	SEALDD enabled E2E redundant transmission provides the replication and deduplication between the two SEALDD flows carrying application traffic.

In addition, different SEALDD services may use different SEALDD user plane protocols and functionalities to convey the application traffic, e.g., the user plane functionalities used by SEALDD E2E redundant transmission service could be different from the one used by SEALDD regular connection management service.

The overall SEALDD user plane protocol stack information is not specified in 3GPP TS 23.433 [23433], which can further assist stage 3 groups to make choice of suitable user plane protocols for supporting SEALDD services.

5.x.2	Open issues

1.	The user plane aspects (e.g., user plane protocol stack, user plane functionality description) of SEALDD-S and SEALDD-UU to be used for the SEAL data delivery services should be studied.