3GPP TSG-SA WG6 Meeting #66	S6-251387

Goteborg, Sweden, 07-11th April 2025	(revision of S6-251245)

Source:	Ericsson

Title:	Solution for Application traffic behaviour and SEALDD data delivery optimization Key Issue

Spec:	3GPP TR 23.700-52 v0.0.0

Agenda item:	9.7

Document for:	Approval

Contact:	Ashish S Sharma, ashish.sanjay.sharma@ericsson.com

1. Introduction

The pCR introduces a solution for Key issue based on the point 3 mentioned in the objectives of SID SP-250382

2. Reason for Change

Point 3: Data delivery for service/application with the following characteristic: the application layer interactions are non-continuous, each of the interactions is an independent application transaction.

The Key issue is proposed in the pCR S6-251078.

The application traffic behaves differently at various times. The SEALDD server is not aware of the application's traffic pattern. The proposed solution allows the SEALDD server to understand the SEALDD traffic patterns at different time intervals.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-52 v0.0.0

* * * First Change * * * *

## X.Y	Solution#Y: SEALDD enabled application traffic behavior characterization procedure

### X.Y.1	General

The SEALDD server handles the user plane delivery of the VAL traffic over SEALDD-UU. Due to the dynamic behavior of the application traffic and time-varying conditions in the network, the SEALDD may need to optimize its user plane transmission. The optimization could include establishing another connection, adjusting QoS parameters, modifying the SEALDD policy parameters, etc.. Therefore, the SEALDD server needs to be aware of the application traffic behavior and its correlation with underlying network conditions. To enable the SEALDD server to find out how the app traffic are behaving at different times, an application traffic behavior characterization procedure is introduced.

The ADAE server builds the traffic behaviour pattern based on the NWDAF analytics.  Since SEALDD traffic is flowing over 5GS, the NWDAF can get the analytics for the SEALDD traffic based on the traffic descriptors. It abstracts the traffic pattern characteristics and exposes a standard query operation to the SEALDD server to query the traffic pattern to get the understanding of the traffic behaviour.

The query operations include - traffic activity period and traffic activity pattern(e.g., peak activity traffic period, low activity traffic period, bursty trafficperiod).

For example,

the SEALDD server  can query traffic activity period– at time instant T1, what is the traffic pattern. The response from ADAE, at T1, the traffic behaviour is peak with details like data volumes exchanged, latency, throughput

Another example, the SEALDD can query traffic activity period – when is the peak activity traffic period. The response from ADAE is, at T1, T5, T6 intervals are of peak traffic with details like  data volumes exchanged, latency, throughput for each intervals.

This enables the SEALDD server to query the traffic behaviour from ADAE and derive the insights how the traffic is behaving at different time instances.

* * * Next Change * * * *

#### X.Y.2	SEALDD enabled application traffic behavior characterization procedure

Figure X.Y.2-1 illustrates the procedure SEALDD enabled application traffic behavior characterization procedure from the SEALDD server to the ADAE server.

Figure X.Y.2-1: SEALDD enabled application traffic behaviour characterization

- The SEALDD server sends the SEALDD enabled application traffic behavior characterization subscription request to ADAE server. The request contains the list of VAL service ID, VAL server ID, list of VAL UE identifiers, network slice identifier, Multi-modal Flow ID(s), SEALDD flow ID(s) and associated 5-tuple traffic descriptors of the SEALD-UU traffic.

The request also contains SEALDD traffic behavior query operations like traffic activity period, traffic activity criteria, and traffic activity pattern(e.g., peak activity traffic period, low activity traffic period, bursty traffic period).

2.	Upon receiving the request, the ADAE server performs an authorization check and responds with a SEALDD enabled application traffic behavior characterization subscription response.

3.	The ADAE server subscribes to NWDAF analytics services related to the user plane traffic like  DN performance, PFD determination, data volume, PDU session analytics mentioned in 3GPP TS 23.288 clause 6.14, clause 6.16, clause 6.18, clause 6.20, clause 6.23 of TS 23.288. The ADAE server uses the traffic descriptors and UE identifiers from the request message to fetch the NWDAF analytics.

The ADAE server also subscribes to the Network Performance analytics NWDAF service described in clause 6.6 TS 23.288 for the number of VAL UE(s) mentioned in the request message.

The ADAE server obtains application performance analytics procedure for VAL server and VAL UE session performance analytics data as described in the TS 23.436 clause 8.2

Based on the processing of the NWDAF analytics, the ADAE server performs the traffic characterization of the SEALDD enabled application traffic. The ADAE server may use statistical or machine learning tools to build the traffic behavior analytics. It may also use the VAL service ID, Flow IDs  to classify the different traffic as per the VAL service.

4.	The ADAE server performs the requested traffic behavior operation on the SEALDD enabled application traffic behavior pattern to derive the traffic behavior insights and provide the notification response. The notification response includes VAL Service ID, SEALDD flow ID, VAL UE ID, and SEALDD traffic behavior pattern result like data volumes exchanged, throughput, latency and jitter.

* * * End of Changes * * * *