3GPP TSG-SA6 Meeting #66	S6-251393

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025                       revision of S6-251112

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0734 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on ACR request for overload situation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To address the ACR due to overloaded situation, it was agreed that “For S-EAS detected case, the S-EAS includes list of UEs for which ACR is required in the ACR request to indicate S-EES to perform ACR for UEs in the group as the S-EAS is overloaded and may not meet to KPIs for common EAS” in cluase 8.8.2.5. However, the ACR request information element in the Table 8.8.4.4-1 has not been updated accordingly with still only a single UE ID included in the table. To correct this, this CR proposes to include a list of UE IDs in the corresponding table. R1: Also, per UE information elements can be also provided as in the following text “If the EEC or S-EAS detect the ACR event, the EEC or S-EAS may inform S-EES with ACID, and predicted/expected UE location or Expected AC Geographical Service Area in the ACR launching procedure.” Accordingly, further updates are introduced |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a list of UE ID in the IE table for ACR request |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned TS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****** Fist Change ******

8.8.4.4	ACR request

Table 8.8.4.4-1 describes information elements for the ACR request sent either from the EEC to the S-EES or T-EES, or by the S-EAS to the S-EES. In the EAS bundle case, the ACR request is sent by the main S-EES to its associated S-EES(s).

Table 8.8.4.4-1: ACR request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Identifier of the requestor (i.e. EECID or EASID). |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EASID | O | Identifier of the EAS |
| EAS bundle information | O | EAS bundle information. |
| > Bundle ID (NOTE 12) | O | Bundle ID as described in clause 7.2.10. |
| > List of EASIDs (NOTE 12) | O | A list of the EASIDs of the EASs in the bundle. |
| UE identifier(s) (NOTE X) | M | The identifier of the UE (i.e. GPSI), or a list of UE identifier (i.e., GPSI list) |
| > Predicted/Expected UE location or Expected AC Geographical Service Area (NOTE 8) | O | The predicted/expected location information of the UE. The UE location is described in clause 7.3.2 or the predicted/expected AC Geographical Service Area as described in clause 7.3.3.3 |
| ACID (NOTE 10) | O | The identifier of the AC. |
| > EASID | O | Identifier of the EAS |
| > EAS endpoint | M | Endpoint information of the T-EAS |
| ACR action (NOTE 3) | M | Indicates the ACR action (ACR initiation, ACR determination or ACR modification) |
| ACR initiation data (NOTE 2) | O | ACR initiation IEs to be included in an ACR request message when ACR action indicates it is ACR initiation request. |
| > T-EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS. In case of ACR to cloud, Endpoint information is of CAS. |
| > Previous T-EAS Endpoint (NOTE 7) | O | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS of the previous ACR. |
| > DNAI of the T-EAS | O | DNAI information associated with the T-EAS. |
| > N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to the T-EAS DNAI. |
| > Simultaneous EAS connectivity information | O | Indicates if simultaneous EAS connectivity is needed and the inactive time guidance for keeping connectivity towards the S-EAS. |
| > EAS notification indication | M | Indicates whether to notify the EAS about the need of ACR. |
| > Previous EAS notification indication (NOTE 7) | O | Indicates whether to notify the EAS about the cancellation of a previous ACR. |
| > S-EAS endpoint (NOTE 1) | O | Endpoint information of the S-EAS |
| > Bundled T-EAS endpoint list (NOTE 11) | O | A list of associated EAS endpoints in a EAS bundle. |
| > ACR parameters (NOTE 9) | O | Parameters of the ACR |
| >> Prediction expiration time | O | The estimated time the UE may reach the Predicted/Expected UE location or EAS service area at the latest |
| > EEC context relocation details | O | Information required for EEC context relocation using the EEC context push or EEC context pull mechanisms. |
| >> EEC Context ID (NOTE 5) | O | Identifier of the EEC Context |
| >> S-EES ID (NOTE 5) | O | Identifier of the EES that provided EEC context ID. |
| >> S-EES endpoint (NOTE 5) | O | The endpoint address (e.g. URI, IP address) of the EES that provided EEC context ID. |
| >> T-EES ID (NOTE 6) | O | Identifier of the T-EES. |
| >> T-EES endpoint (NOTE 6) | O | The endpoint address (e.g. URI, IP address) of the T-EES. |
| ACR determination data (NOTE 2) | O | ACR determination IEs to be included in an ACR request message when ACR action indicates it is ACR determination request. |
| > S-EAS endpoint | M | Endpoint information of the S-EAS |
| > Tunnel information | O | It includes service provider ID, the endpoint address (e.g. IP address) of the tunnel server. |
| ACR modification data (NOTE 2) | O | ACR modification IEs to be included in an ACR request message when ACR action indicates it is ACR modification request. |
| > S-EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the S-EAS. |
| > T-EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS. |
| > ACR parameters | M | ACR parameters |
| >> Prediction expiration time | O | The estimated time the UE may reach the Predicted/Expected UE location or EAS service area at the latest |
| NOTE 1: This IE shall be present if the EAS notification indication or previous EAS notification indication indicates that the EAS needs to be informed. NOTE 2: One of the ACR initiation, ACR determination or ACR modification shall be included corresponding to the ACR action. If more than one UE IDs are included in the ACR request, the list of this information elements per UE ID is also included, if available. NOTE 3: This IE shall indicate ACR determination if the request originates from the S-EAS. NOTE 4: Void. NOTE 5: This IE may be present only if the request originates from the EEC towards the T-EES. NOTE 6: This IE may be present only if the request originates from the EEC towards the S-EES. NOTE 7: These IEs shall be present when the EEC re-sends the ACR request as described in clause 8.8.1.3 to indicate a previous ACR is to be cancelled. NOTE 8: This IE may be present if the ACR procedure is for service continuity planning. If more than one UE IDs are included in the ACR request, the list of this information elements per UE ID is also included, if available. NOTE 9: This IE may be included when the ACR is decided and executed for service continuity planning for a predicted/expected UE location. NOTE 10: The IE shall be present when the action is ACR modification to identify the ACR to be modified. NOTE 11: The IE is present when the EEC informs S-EES(s) with applicable bundled T-EAS(s) information for the direct bundle EAS case or when the main S-EES informs its associated S-EES(s) with applicable bundle T-EAS(s) information for the direct bundle EAS case. NOTE 12: At least one of the IEs shall be present if EAS bundle information is provided. NOTE X: This IE shall include at least a single UE ID. The list of UE IDs may be included only if the request originates from S-EAS (see clause 8.8.2.5). If more than one UE IDs are included, the corresponding information elements (predicted/expected UE location or Expected AC Geographical Service Area) shall be included per UE ID, if available. |  |  |

****** End of Change ******